3GPP TSG RAN WG2 Meeting #131bis                                                       R2- 2507187

# Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:	10.3.2

Source:	Ofinno

Title:	Control Plane aspects for 6G

Document for:	Discussion and decision

# Introduction

This document discusses the control plan aspects and key features for 6G system based on the agreed 6G SID [1].

The gNB response decides whether to resume, reestablish, or reject. - Initial access [RAN1, RAN2, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

# Discussion

The foundational approach for 6G control plane design is driven by lessons learned from 5G, particularly the need to move beyond incremental solutions that resulted in heavy RRC design.

## RRC configuration structure

5G RRC specification, TS 38.331, uses ASN.1 PDUs with nested, optional fields and many blocks of configuration (e.g., radio bearer configurations, physical layer configurations, MCG/SCG Configurations, multiple PUCCH/PUSCH configurations, multi-connectivity related configuration etc.) resulting in a large static configuration model. It does provide for a flexible and scalable model where adding a feature, often including numerous options fields and backward-compatible branches, at the cost of large size and substantial signaling overhead. They require significant air interface resources increase the processing delay. It also requires that the signaling for every new feature is developed precisely to avoid configuration errors and interoperability issues.

An observation from the current 5G RRC configuration structure is that in several RRC messages, the information exchanged are quite elaborate and extensive. In many scenarios, the complete information exchanged in the RRC message may not be utilized, indicating that the radio resources used to provision the unused information in these RRC messages are actually wasted, leading to suboptimal usage of radio resources. The UE capability information message in 5G and RRC reconfiguration message (e.g., BWP configuration, CSI framework configuration etc.) are prime examples of this scenario. For example, a UE might report support of several UE capabilities that may not be utilized effectively in live network.

There is a need to simply the RRC configuration structure while still allowing easy addition of new features.

5G started big with supporting many features in the initial releases to support different devices ranging from a low-power IoT device to a high-speed eMBB device. While 5G was designed with a "lean design" in mind, many key features were introduced incrementally in later releases rather than being part of the foundational specification. This resulted in a bloated RRC specification and protocol design requiring a high memory footprint (e.g., to support/store a large set of RRC parameters) and long processing times (e.g., ASN.1 encode/decode of large messages). This approach also meant that not all devices and device types benefited from these features from day one, and it required backward compatibility constraints that added complexity.

An objective of 6G is to allow scalable and forward compatible design for even more diverse device types as that of 5G. RRC in 5G supports large feature sets and optional features resulting in: large PDU sizes, many messages, complex configuration etc. This hinders extremely constrained UEs (ambient IoT) and makes low latency control plane design harder in 6G. Learnings from 5G indicate that not all devices and deployments can benefit from all the features at once. It may be beneficial and sustainable to, instead of having all features at once, a lean RRC protocol is designed. This can help minimizing signaling overhead and latency in RRC procedures. in For example, the RRC protocol can be designed to provide UEs with minimal necessary signalling and allow UEs to receive only the configuration/ features that it requires for a target function/ operation (e.g., on demand). This facilitates low latency based on lean messages that are processed faster, and energy efficiency based on less radio resource and processing power spent of control signaling. This model should be targeted for easy scalability.

- RAN2 to study a lean RRC design/ signalling model based on a minimal necessary signalling and subsequent addition of on-demand configuration blocks.

5G RRC adopts a monolithic design where all control plane functions (e.g., connection management, mobility, bearer setup, measurements, paging, SI, security etc.) are bundled into a single protocol. This means that all device types (e.g., eMBB, mMTC, URLLC) need to implement all of these control plane functions even if only a subset of these functions is used. The lack of modularity requires that even for small updates, the full RRC message exchanges are required (e.g., RRC reconfiguration for minor mobility related reconfiguration). This leads to increased latency, higher UE energy consumption and radio resource wastage. This makes is difficult to adapt the protocol for different device types e.g., RedCap, IoT etc.

Figure 1 below represents an example of a modular RRC design. The RRC protocol/ control plane can be decomposed into separate RRC modules and each RRC module may be corresponding to a separate function. Each RRC module can be independently configured and updated. This modular RRC design can optimize support for different device types, and can be customized for specific use cases.

Figure: example of a modular RRC design

- RAN2 to study a modular RRC design/ architecture based on breaking down RRC protocol into independently configurable RRC modules.

## RRC states

5G introduced RRC_INACTIVE state as to bridge RRC_IDLE state (no RRC context in RAN) and RRC_CONNECTED state (full context in RAN) to address the inefficiencies of the IDLE-CONNECTED model for bursty and frequent traffic. A full transition from IDLE to CONNECTED for every small data burst incurred high latency and significant battery drain, as the UE repeatedly had to go through a long signaling sequence. By retaining the UE context in the RAN, 5G's RRC_INACTIVE state significantly reduces signaling overhead and latency, making it a powerful tool for accommodating new traffic profiles. This design choice represents a crucial lesson learned from prior generations, as it fundamentally re-architected the control plane to optimize for the IDLE-CONNECTED traffic profile.

However, there are some limitations here. RAN nodes must hold potentially large numbers of INACTIVE contexts resulting in a large memory footprint. Furthermore, RAN nodes may transition all device types into RRC_INACTIVE state based on a common setting based on a best guess value of inactivity timers, which may not be optimal for a wide range of use cases and unpredictable user behaviors. This static approach can still lead to inefficient state changes and suboptimal performance.

In 5G, while RRC_INACTIVE to RRC_CONNECTED transition is faster than RRC_IDLE to RRC_CONNECTED transition, it still contributes to initial access latency (e.g., random access and RRC message exchange/ reconfigurations). The UE context is maintained on one RAN node and RNA area are statically configuration according to the RAN node. This is sub-optimal for highly dynamic scenarios with varying traffic patterns and mobility profiles.

If network can identify the UE’s future data activity and mobility, the network can proactively move the UE to the suitable RRC state. For example, the traffic pattern of an XR service is largely different from that of an AI service or an IoT service. They may have different idle periods as well. A UE in high mobility may find a larger RNA area more efficient than a small RNA area that is suitable for UEs with low to medium mobility. 6G can reap the benefits of RRC_INACTIVE state if it is optimized/ tailored to specific device types, traffic patterns and mobility profiles.

- RAN2 to study optimizations for IDLE/INACTIVE states to address different device types, traffic patterns and mobility profiles efficiently.

## Initial Access

Initial access in 5G includes cell search, synchronization (PBCH/MIB), system information acquisition, and RACH procedure. It begins with the UE acquiring downlink synchronization by detecting the SSB, a set of signals transmitted by the gNB at fixed locations within a half-frame. The SSB contains the PSS and SSS, which allow the UE to acquire symbol and radio frame synchronization.

Once synchronized, the UE can decode the PBCH, which is also part of the SSB. The PBCH carries the MIB, which is essential for further system access. The MIB provides crucial parameters, such as the System Frame Number (SFN), the sub-carrier Spacing to be used for SIB1 reception, and configuration details for the PDCCH search space for SIB1 decoding. Following MIB decoding, the UE proceeds to acquire SIB1 (i.e., RMSI). SIB1 is mandatory for a UE to perform a random access and contains critical network access parameters, cell selection information, PLMN identity, and scheduling information for all other SIBs.

At mmWave/sub-6GHz with beamforming, initial acquisition includes beam sweeping (for both PBCH and RACH), increasing latency and complexity (e.g., the need to sweep many beams increases time-to-access) beyond what was seen in previous generations. The move to beamforming at higher frequencies is a direct physical consequence of the need to compensate for signal propagation losses. As a result, the UE can no longer simply listen for an omnidirectional signal. It must now actively search for and identify the best beam from a sweeping pattern.

The time to access, and the complexity of initial access in 5G is similar for all device types. Example, the procedure followed by an IoT device is similar to that of an eMBB device. 6G can benefit from: reducing the time taken to access a 6G cell, reducing complexity of the initial access procedure, device type specific initial access procedure/ requirements.

- RAN2 to study mechanisms to reduce time taken for initial access in 6G. Study how to optimize initial access for different device types.

## System Information

SIBs include cell selection info, access restrictions, RACH configuration, time/frequency resources etc. SIB scheduling and notification (SI-window, SI-messages, scheduling via MIB) control when UEs read/ acquire SIBs. Some SIBs are larger in size than others and some are more dynamic than others. System information is either broadcast or requested on-demand. The on-demand mechanism is a significant efficiency gain. It prevents the continuous broadcasting of SIBs that are rarely needed, saving system resources and energy for the network and UEs that don't need the information.

However, acquiring on-demand SI requires a UE to enter the connected state, which incurs RACH and signaling latency. Much of SIBs is static and is broadcast repeatedly, even if no UE requires it. Consequently, the network may opt to transition to on-demand mechanism of SIB transmission. It is also possible that in some use cases (e.g., IoT devices), a potential surge of simultaneous on-demand requests for the same SI (e.g., after a network outage) could cause RACH congestion. Furthermore, even when only minimal information in a SIB changes, the entire SIB needs to be transmitted by the RAN node and received by the UE. All the UEs, irrespective of the need, acquires the updated SIB.

An efficient SIB delivery would require a balance between the size of a SIB, number of SIBs and frequency of transmission/ repetitions of the SIBs. 6G can benefit from hybrid model as a baseline for SIB delivery. For example, keep critical/ RMSI as broadcast and deliver the other SI on-demand. By removing the pure broadcast mechanisms, the devices that do not requires the other SI does not need to acquire them. Allowing for delta updates for SIB modifications can improve energy efficiency and resource utilization. A context-aware or device type based SI updates and updated SI acquisition will help in the energy efficiency of the UEs that do not require the updated SI.

- Consider Broadcast of RMSI and on-demand delivery of other SI as baseline in 6G. RAN2 to study Delta update mechanism for SI updates and Context-aware SI acquisition.

## Paging

To reach a UE in RRC_IDLE, MF/SMF triggers paging which is delivered to NG-RAN and broadcast on paging occasions in the cell(s) of tracking area (TA) where the UE is known to be camping. For a UE in RRC_INACTIVE, the AMF, assuming that UE is connected to the network, sends a data to the RAN. The RAN node identifies the UE is in RRC_INACTIVE and broadcasts on paging occasions in the cell(s) of ran notification area (RNA).

Paging uses a periodic paging occasion determined by DRX cycles. UE monitors all of its paging occasions even if there is no paging during the paging occasion. This means the UE wakes up its radio to monitor the paging occasion which increases the energy consumption. The DRX cycle length, which is configured by the network, dictates the balance between power consumption and latency. Longer cycles save battery life but increase the time it takes for a UE to respond to a page, which is a sub-optimal solution for applications with varying latency requirements. To make the paging monitoring more energy efficient, an Early Paging Indication (EPI), a feature that informs a UE whether or not to monitor an upcoming Paging Occasion was introduced in Release 17.

The number of UEs that can be paged in a single paging occasion is limited by the PDCCH capacity. This becomes a limiting factor when a large number of devices to be paged. In release 19, a group paging mechanism is introduced  which allows network to page a group of devices simultaneously, rather than paging each device individually. This approach reduces signaling overhead and improves efficiency in scenarios involving a large number of devices, such as smart city applications and industrial IoT deployments.

Considering diverse device types and a variety traffic patterns, the number of devices in a cell is expected to be higher than that of 5G. Inherently, the number of UEs that can be paged using UE-specific paging will be limited. A group-specific paging may improve paging efficiency. However, this could lead to RACH congestion if all the devices that received the group-specific paging attempt RACH using the same RACH resource. Therefore, an optimal way to limit the number of UEs receiving the group-specific paging has to be determined in order to avoid RACH congestion. Another way to improve energy efficiency is by monitoring paging occasion based on UE’s context e.g., location, service type, mobility etc.

- Support UE-specific and group- specific paging mechanisms. RAN2 to study context-aware paging mechanisms.

## RRC reestablishment and RRC resume

RRC Reestablishment is invoked when link failure requires re-establishing an RRC connection (RRCReestablishmentRequest/Complete) using stored context at RAN and C-RNTI of the cell where link failed. This may involve additional recovery and security steps. RRC Resume is the intended fast path from RRC_INACTIVE to RRC_CONNECTED using stored context at RAN (RRCResume/RRCResumeComplete) and a resume ID. When the RAN node receiving the RRCResume fails to retrieve the UE context, the UE falls back to an RRC setup procedure.

The RRC Resume and RRC Reestablishment procedures are distinct but have overlapping functions. Except for the cause and triggering event, the signaling appears to be similar for these procedures. Both resume and reestablishment rely on stored UE context. The RAN node response to the request for RRC Resume/ RRC Reestablishment decides whether to resume, reestablish, setup or reject the RRC connection.

Implementing both RRC Resume and RRC Reestablishment separately in 6G results in some duplicated procedures and extra complexity (e.g., RAN node may keep multiple contexts for the same UE). Considering there is minimal difference between the 2 procedures, we think it is beneficial to have a single/ unified procedure to 1) recovery from link failure, and 2) transition from RRC_INACTIVE to RRC_CONNECTED state.

- Support a unified RRC recovery procedure for: reestablishing RRC connection/ link from link failure and resuming RRC connection/ transition from RRC_INACTIVE to RRC_CONNECTED.

# Conclusion

The contribution discussed aspects related to control plan design for 6G. The following observations and proposals are made:

- There is a need to simply the RRC configuration structure while still allowing easy addition of new features.

- RAN2 to study a lean RRC design/ signalling model based on a minimal necessary signalling and subsequent addition of on-demand configuration blocks.

- RAN2 to study a modular RRC design/ architecture based on breaking down RRC protocol into independently configurable RRC modules.

- Proposal 3.	RAN2 to study optimizations for IDLE/INACTIVE states to address different device types, traffic patterns and mobility profiles efficiently.

- RAN2 to study mechanisms to reduce time taken for initial access in 6G. Study how to optimize initial access for different device types.

- Consider Broadcast of RMSI and on-demand delivery of other SI as baseline in 6G. RAN2 to study Delta update mechanism for SI updates and Context-aware SI acquisition.

- Support UE-specific and group- specific paging mechanisms. RAN2 to study context-aware paging mechanisms.

- Support a unified RRC recovery procedure for: reestablishing RRC connection/ link from link failure and resuming RRC connection/ transition from RRC_INACTIVE to RRC_CONNECTED.

# Reference

- RP-252912, Study on 6G Radio, SID, September 2025.