3GPP TSG-RAN WG2 Meeting #131bis										R2-2507192

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	8.7.3

Source: 	Sharp

Title:  	Discussion on XR User Plane Open Issues

Document for: 	Discussion

# Introduction

This document discusses remaining open issues on XR User Plane.

# Discussion

## [PDCP-N001] UE implementation whether to re-send the gap report after HO

(RLC-12) No special handling is needed in R19 for PDCP SN gap report during UE mobility. It can be left to UE implementation whether to re-send the gap report after HO.

This issue is to decide whether to capture a NOTE for the agreement above. The agreement indicates that UE may re-send the gap report after HO by UE implementation. It means that it is allowed, and UE implementation can choose whether to re-send it. The RAN2 specification is written from the UE perspective. The UE implementation follows only what is written in the specification, whereas the network has more freedom in their implementation. In the layer 2 specifications, there are few parts which capture the case that UE implementation decides something. Without the NOTE or normative text, it is interpreted that UE is not allowed to re-send the gap report.

(PDCP-N001) Capture a NOTE to allow UE implementation to decide whether to re-send the gap report after handover.

## [RLC-X01] PDCP Awareness of RLC Packet Transmission Status

A open issue we put on the post email discussion: In last meeting, we have agreed: - [RLC-13] PDCP should indicate to RLC what needs to be included in the data volume calculation in RLC for each threshold - [RLC-13] We can remove definition of delay reporting and non-delay reporting data from RLC If all the RLC SDUs that RLC needs to calculated are indicated from PDCP, then RLC can indeed calculate the data based on PDCP’s indication. Then RLC calculation of the none delay reporting data can be removed. However, is that possible that RLC put some packets which are not know in PDCP before the data of one portion or is there possible that before DSR is triggered, there has already some data before one portion? Then in this way, since PDCP is not aware of data in RLC’s buffer, then I guess the part that RLC calculation of the none delay reporting data still can not be removed. And Rapp’s response is: “ My understanding is all SDU in RLC comes from PDCP, so PDCP is aware of all packets status (remaining time). RLC just needs to determine which one/segments should be calculated in the DSR, e.g. based on whether delivery to lower layer/retx. It seems no problem for this. We could think more if any issue, and further discuss it during maintenance phase. ” In last F2F meeting offline in India, after checking with LG, Sharp, we are not sure on this. We would like to hear more opinions from other companies.

A problematic case for this issue might be that (1) PDCP does not know the packet transmission status of SDUs in the RLC sublayer or (2) PDCP does not know the exact order of packet transmission in RLC or MAC. For the first case, all L2 UP sublayers are usually implemented as a single block, not multiple individual blocks based on four individual sublayers. Even if only some important cross-layer interactions are specified, UE implementation is able to access the UE internal information. We see that a sensible implementation would internally maintain the list of packet transmission status. The second case may be related to out-of-order transmission at RLC and MAC. However, from the UE perspective, out-of-order transmission within the logical channel does not have any necessity but requires implementation complexity. Therefore, we do not need to consider the case. In short, the current procedure based on PDCP indication works well.

(RLC-X01) RAN2 assumes that PDCP knows the transmission status of all packets (i.e., PDCP SDUs). No specification change is needed.

## [RLC-V01] Indication of Obsolete SDU to Receiving PDCP Entity

When t-RxDiscard expires, the RLC entity stops waiting to receive the obsolete RLC SDUs, and the corresponding RLC SDUs will be positively acknowledged in the STATUS report. However, the PDCP t-Reordering may be still running and waiting for receiving the outdated SDUs from the RLC layer, this can lead to unnecessary delays in packet delivery. Therefore, the receiving RLC entity should indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires.

t-RxDiscard expiry stops waiting to receive the obsolete RLC SDUs and moves the receiver window only in RLC. The issue is that PDCP window does not move regardless of RLC window status. It may increase PDCP transmission delay. However, the indication from RLC to PDCP is not always possible. Unless the receiving RLC entity decodes the received RLC SDU successfully, the RLC entity cannot know the associated PDCP SN for not-received RLC SDU. There are multiple cases that RLC does not know it, for example:

- Split bearer operation does not guarantee one-to-one correspondence between PDCP SN and RLC SN, as some PDCP PDUs may not be submitted to one RLC entity of the split bearer.

- PDCP re-establishment may trigger selective retransmissions. In this case, the receiving RLC entity cannot guess based on one-to-one correspondence.

- In downlink, the network may perform out-of-order transmission especially in CU-DU architecture.

- PDCP control PDU transmission may brake one-to-one correspondence between PDCP SN and RLC SN.

- PDCP SN Gap Report may break the one-to-one correspondence.

In short, the RLC indication to PDCP is not always possible. That is also why legacy NR specification assumes independent window operations in RLC and PDCP.

(RLC-V01) Indication of obsolete SDU to receiving PDCP entity is not supported.

## [RLC-N01] Meaning of RX_Next

The definition of RX_Next update should reflect t-RxDiscard expiry (to be aligned with the NOTE in 5.3.4) and text - which value of the SN is pointed - should be aligned with the procedural text in 5.2.3.2.x. Our suggestion is: a) RX_Next – Receive state variable This state variable holds the value of the SN following the last in-sequence completely received RLC SDU of the first RLC SDU not delivered to the upper layers, but still waited for, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires.

The issue is that, due to the introduction of t-RxDiscard, RX_Next may not hold the value of the SN following the last in-sequence completely received RLC SDU of the first RLC SDU not delivered to the upper layers but still waited for. Clause 7 in RLC specification does not need to be so precise, as the exact UE behaviours are captured in Clause 5 Procedures. A simple deletion of “following the last in-sequence completely received RLC SDU” to not restrict to the legacy operation would be enough.

(RLC-N01) “following the last in-sequence completely received RLC SDU” is deleted.

# Conclusions

Based on the discussion given above, RAN2 is requested to agree the following proposals:

- (PDCP-N001) Capture a NOTE to allow UE implementation to decide whether to re-send the gap report after handover.

(RLC-X01) RAN2 assumes that PDCP knows the transmission status of all packets (i.e., PDCP SDUs). No specification change is needed.

(RLC-V01) Indication of obsolete SDU to receiving PDCP entity is not supported.

(RLC-N01) “following the last in-sequence completely received RLC SDU” is deleted.