3GPP TSG-RAN WG2 Meeting #131bis										R2-2507193

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	8.20.1

Source: 	Sharp, CHTTL, SES

Title:  	UE Capability for Rel-19 Ku Band VSAT (R4 60-1/60-2)

Document for: 	Discussion

# Introduction

This document discusses a RAN4 LS on UE capability signaling for Rel-19 Ku band Very Small Aperture Terminal (VSAT) [1] and how the UE capability specification supports the feature.

# Discussion

In a RAN4-led WI on NTN Ku Band (WI code: NR_NTN_Ku_bands), RAN4 agreed to include two FR1-NTN bands (n248, n247) and two FR2-NTN bands (n509, n508) as the priority 1 Ku bands [2][3] to cover 10.7~14.5GHz frequency ranges, as follows:

### 5.2.2 Operating bands for FR1-NTN NTN satellite is designed to operate in the operating bands defined in Table 5.2.2-1. Table 5.2.2-1: NTN satellite bands in FR1-NTN NTN satellite operating band1; Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high; Duplex mode. n256; 1980 MHz – 2010 MHz; 2170 MHz – 2200 MHz; FDD. n255; 1626.5 MHz – 1660.5 MHz; 1525 MHz – 1559 MHz; FDD. n254; 1610 – 1626.5 MHz; 2483.5 – 2500 MHz; FDD. n252; 2000 – 2020 MHz; 2180 – 2200 MHz; FDD. n248; 14000 MHz - 14500 MHz; 10700 MHz – 127502,3 MHz; FDD. n247; 13750 MHz - 14000 MHz; 10700 MHz – 127502,3 MHz; FDD. NOTE 1: NTN satellite bands are numbered in descending order from n256. NOTE 2: For Region 2, the downlink is limited to 10700 – 12700 MHz. NOTE 3: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT.. ### 5.2.3 Operating bands for FR2-NTN NTN satellite is designed to operate in the operating bands defined in Table 5.2.3-1. Table 5.2.3-1: Satellite operating bands in FR2-NTN Satellite operating band; Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high; Duplex mode. n5121; 27500 MHz - 30000 MHz; 17300 MHz - 20200 MHz; FDD. n5112; 28350 MHz - 30000 MHz; 17300 MHz - 20200 MHz; FDD. n5103; 27500 MHz - 28350 MHz; 17300 MHz - 20200 MHz; FDD. n509; 14000 MHz - 14500 MHz; 10700 MHz – 127504,5 MHz; FDD. n508; 13750 MHz - 14000 MHz; 10700 MHz – 127504,5 MHz; FDD. NOTE 1: This band is applicable in the countries subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries subject to or referring to FCC 47 CFR part 25. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202). NOTE 4: For Region 2, the downlink is limited to 10700 – 12700 MHz. NOTE 5: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT..

Due to the addition of those bands, a VSAT UE operating in the newly added FR1-NTN bands is required to report two capabilities, i.e., VSAT UE type in NTN (fixed or mobile) and beam steering type (electronic or mechanical). Those capabilities are captured in the RAN4 UE feature list [4] as R4 60-1 and 60-2, respectively.

There are similar capabilities defined for Rel-18, i.e., ntn-VSAT-MobilityType-r18 (R4 40-1) and ntn-VSAT-AntennaType-r18 (R4 40-2). The only difference is that the Rel-18 capabilities are applicable only for FR2-NTN bands, and the Rel-19 capabilities are applicable for both FR1-NTN and FR2-NTN bands. In this context, the RAN4 UE feature list [4] indicates that R4 60-1 and 60-2 capabilities are extension of Rel-18 UE features 40-1 and 40-2, respectively, which are currently limited to FR2-NTN only.

The only difference is that the existing Rel-18 capabilities are applicable for only FR2-NTN bands, and the Rel-19 capabilities are applicable for both FR1-NTN and FR2-NTN bands.

The RAN4 LS [1] suggested extending the existing Rel-18 capability bits to be applicable for both FR1-NTN and FR2-NTN. RAN2 discussion should focus on detailed capability signaling for Rel-19 VSAT UE. We see that there are two options:

< Option 1: Direct extension of Rel-18 Capability to be applicable for both FR1-NTN and FR2-NTN >

If we strictly follow the suggestion in the RAN4 LS, the expected change should be to lift applicability only for FR2-NTN to make it applicable for both FR1 and FR2, as follows:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ntn-VSAT-AntennaType-r18 Indicates whether a VSAT UE uses electronic or mechanical steering antenna. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. | UE | No | No | FR2 only No |
| ntn-VSAT-MobilityType-r18 Indicates whether a VSAT UE is a mobile or fixed VSAT. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. | UE | No | No | FR2 only No |

In this option, the expected spec change can be minimized. However, this may or may not have a confusion on interpretation of the Rel-18 capability bits at the network. For example, when both Rel-18 UE and Rel-19 UEs report the same capability, the network may have a confusion whether those UEs support the capability in FR1-NTN or not.

< Option 2: Introduce new Rel-19 capability bits for both FR1-NTN and FR2-NTN >

We may consider introducing new Rel-19 capability bits applicable for both FR1-NTN and FR2-NTN. In this option, a Rel-19 VSAT UE can report only Rel-19 capability. This option is to have a clear separation of Rel-18 VSAT UEs and Rel-19 VSAT UEs. The expected change may be as follows:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ntn-VSAT-AntennaType-r18 Indicates whether a VSAT UE uses electronic or mechanical steering antenna in FR2-NTN. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. | UE | No | No | FR2 only |
| ntn-VSAT-MobilityType-r18 Indicates whether a VSAT UE is a mobile or fixed VSAT in FR2-NTN. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. | UE | No | No | FR2 only |
| ntn-VSAT-AntennaType-FR1-FR2-r19 Indicates whether a VSAT UE uses electronic or mechanical steering antenna in FR1-NTN and FR2-NTN as defined in TS 38.101-5 [34]. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. | UE | No | No | No |
| ntn-VSAT-MobilityType-FR1-FR2-r19 Indicates whether a VSAT UE is a mobile or fixed VSAT in FR1-NTN and FR2-NTN as defined in TS 38.101-5 [34]. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. | UE | No | No | No |

In our view, either option would work. RAN2 should choose one option to support the Rel-19 VSAT UE.

There can be two options to support Rel-19 VSAT UE capabilities:

- - Option 1: Direct extension of Rel-18 Capability to be applicable for both FR1-NTN and FR2-NTN

- - Option 2: Introduce new Rel-19 capability bits applicable for both FR1-NTN and FR2-NTN.

From the capability signalling perspective, RAN2 has tended to introduce a separate signalling among for a new release feature. In this context, we prefer Option 2, which can be considered as a clean solution without considering combination/confusion with legacy features/UEs.

Introduce new Rel-19 capability bits applicable for both FR1-NTN and FR2-NTN.

# Conclusions

Based on the discussion given above, we have the following observations on NTN Ku Band features:

- The only difference is that the existing Rel-18 capabilities are applicable for only FR2-NTN bands, and the Rel-19 capabilities are applicable for both FR1-NTN and FR2-NTN bands.

There can be two options to support Rel-19 VSAT UE capabilities:

- - Option 1: Direct extension of Rel-18 Capability to be applicable for both FR1-NTN and FR2-NTN

- - Option 2: Introduce new Rel-19 capability bits applicable for both FR1-NTN and FR2-NTN.

RAN2 is requested to agree the following proposal:

- Introduce new Rel-19 capability bits applicable for both FR1-NTN and FR2-NTN.

The corresponding draft CRs are R2-2507194 (38.306) and R2-2507195 (38.331).

# Reference

[1] R4-2512658	LS on UE Capability for Rel.19 Ku band VSAT	RAN4

[2] R4-2510831	Big CR for 38.101-5 for the addition of Ku bands	Eutelsat Group

[3] 3GPP TS 38.101-5	NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements, v19.1.1

[4] R4-2511883	Rel-19 RAN4 UE feature list for NR (version 2)	CMCC