3GPP TSG-RAN WG2 #131bis	R2-2507200

Prague, Czech Republic, 13th – 17th October 2025

Source:	NTT DOCOMO, INC.

Title:	On 6G User Plane

Document for:	Discussion

Agenda Item:	10.3.1

- Introduction

This contribution is to provide our general views on the first RAN2 discussion for 6G User Plane.

- Discussion

  - Considerations of NR layer 2 functions

When designing the 6GR User Plane from the ground up, considering that the NR User Plane has fundamentally worked, the initial step in the discussion should be to evaluate whether the functions supported by NR should be retained in 6GR.

Observation 1.	As a first step for discussing the 6GR User Plane architecture from scratch, it is appropriate to evaluate the necessity of the Layer 2 functions defined in the NR User Plane.

To facilitate this evaluation, the table below enumerates the functions supported by Layer 2 in the NR User Plane and summarizes our analysis regarding the necessity of each function in the context of 6GR.

| NR sublayer | Function | Analysis |
| --- | --- | --- |
| SDAP | Mapping between QoS Flow and DRB | It depends on SA2/RAN3 discussion on 6G QoS and slicing. |
| PDCP | Header compression | Necessary. Baseline is to support ROHC for 6G Voice and UDC for UL user data. EHC depends on whether Industrial IoT for 6GR is supported. |
|  | Security protection | Necessary for UP data and CP data (i.e., dedicated RRC messages and NAS messages). FFS for new plane (e.g., for AI/ML or sensing) if we have it. We need to coordinate with SA3. Note: Security protection for MAC CE and System information should not be discussed in this agenda. |
|  | In-sequence delivery | Necessary. CP data does not work without it. Regarding UP data, some services may not need re-ordering in AS. For example, each data burst may be small enough to deliver in a single PDU, or transport layer could be relied on for re-ordering. |
|  | Discarding of outdated data | Necessary. |
|  | PDCP retransmission | It depends on RAN2 discussion on mobility framework. If data forwarding during handover is required as in NR, it would be useful. |
|  | Split bearer and routing | It depends on whether 6G RAN supports Dual Connectivity, thus we should wait for RAN/RAN3 discussion on 6G architecture and migration. |
|  | Duplicate discarding | It depends on whether 6G RAN supports Dual Connectivity or PDCP retransmission. |
| RLC | ARQ | Necessary to ensure loss-less delivery in air interface. Note: We would like to selectively perform ARQ depending on upper layer configuration. Details are in a later section. |
|  | Segmentation | Necessary to fit the packet size when UL grant is smaller than data packet size. |
|  | Re-segmentation | Necessary because it is still reasonable to reduce the size of packet when it fails. |
| MAC | Multiplexing | Necessary. |
|  | HARQ | Necessary. Note: We would like to discuss modification of HARQ operation. Details are in a later section. |
|  | MAC CE | Necessary since NR legacy features (most of which are expected to be reused in 6G) rely on MAC CE. |

For each function identified as necessary in the above analysis, it is appropriate to use the implementation in NR as a baseline and to discuss potential optimizations or modifications for 6GR as needed.

Proposal 1.	6G Layer 2 supports the following functions, using NR as a baseline and applying modifications or optimizations as needed: header compression (ROHC and UDC), security protection for UP/CP data, in-sequence delivery, discarding of outdated data, ARQ, (re)segmentation, multiplexing, HARQ and MAC CE.

It is important to note that this analysis focuses solely on the functional requirements at Layer 2 and does not propose which sublayer should implement these functions. For example, identifying a function as necessary, such as one currently handled by the RLC sublayer, does not imply that 6GR must include an RLC sublayer or that the same function must be implemented in the same manner.

Proposal 2.	RAN2 studies which sublayer supports each legacy function and order of the functions to be processed in Layer 2.

  - SDAP layer

In NR, the SDAP layer provides the function of mapping between QoS Flows and DRBs.

However, based on our operational experience with NR, one of the challenges is that the presence of the SDAP layer increases network development costs.

For example, coordination between neighboring CUs is required to determine whether the SDAP layer should be supported, in order to ensure proper data forwarding during handover.

Furthermore, since the SDAP layer is positioned above the PDCP layer which is responsible for reordering user plane data, there is no specification-level guarantee that data will be delivered in its original correct order when QoS remapping occurs. Then this behavior is left to the implementation of the gNB, which is not ideal from our perspective.

Observation 2.	The SDAP layer in NR requires operational effort to ensure proper data forwarding during handover and does not guarantee in-order delivery when QoS flow remapping occurs.

Therefore, regarding the function of mapping between QoS Flows and DRBs currently supported by the SDAP layer in NR, it should be considered whether this functionality can be relocated to another layer, such as the PDCP layer, in the context of 6GR.

Proposal 3.	RAN2 studies whether SDAP sublayer can be merged to PDCP sublayer, i.e., PDCP can support mapping between QoS Flow and DRB, coordinating with other WGs.

  - Retransmission functions in Layer 2

In NR, error detection and retransmission are handled concurrently by HARQ at the MAC layer and ARQ at the RLC layer. These mechanisms have proven effective in the field, providing reliable communication with minimal errors.

However, the retransmission framework in NR contributes to increased transmission latency for user data. In particular, the delay associated with error detection and retransmission control at the RLC layer results in significant jitter.

Observation 3.	NR’s dual-layer retransmission, HARQ at MAC and ARQ at RLC, ensures reliability but introduces significant jitter of latency, primarily due to delays in ARQ retransmission at RLC.

For 6GR, it is worth considering the integration of ARQ and HARQ functions within the MAC layer, with the objective of minimizing overall retransmission-induced latency.

For example, latency could be reduced by enabling the receiving MAC entity to flexibly trigger ARQ retransmissions or re-segmentation.

Figure 1. ARQ and HARQ should be integrated to each other in MAC layer for 6GR.

Proposal 4.	RAN2 studies integration of ARQ and HARQ in MAC layer to minimize retransmission delay.

  - Upper layer-aware L2 functions

In both NR and pre-NR systems, the air interface has fundamentally been designed to deliver packets in a lossless manner by utilizing both ARQ and HARQ. One major reason for this design choice is that most user data relied on transport layers, where loss-based congestion control algorithms were predominantly used.

This type of algorithm interprets packet loss over the air interface as a sign of congestion caused by buffer overflow at network nodes and responds by reducing the congestion window. This behavior leads to a significant drop in throughput, negatively impacting user experience. This is one of the key reasons why the air interface up to NR has been designed to be lossless.

Figure 2. How network congestion is mistakenly detected even when packet loss happens in air interface.

Now we describe the congestion control algorithms at the transport layer in the table. (To be very correct, it may be noted that the L4S algorithm, introduced as a derivative of delay-based congestion control, is not limited to the transport layer — it can be implemented at various layers that terminate between nodes.)

| Congestion control algorithm | Examples | Description |
| --- | --- | --- |
| Loss-based congestion control | CUBIC, NewReno | This algorithm is at transport layer and determines that network congestion is worsening as the number of packets discarded during data transmission increases and reduces the congestion window size, i.e., lowers throughput. |
| Delay-based congestion control | BBR, TCP RACK | This algorithm is at transport layer and determines that network congestion is worsening when the amount of delay caused by packet transmission increases and reduces the congestion window size, i.e., lowers throughput. BBR is a delay-based algorithm that is already used in the latest popular applications, e.g., YouTube [1], and expected to be used for more applications. |
| Derivative of delay-based congestion control | L4S [2][3] | This algorithm estimates congestion between each node by calculating the proportion of ACKs received every RTT (Round-Trip Time) that are flagged with ECN (Explicit Congestion Notification). L4S is rapidly gaining adoption as a technology for enabling end-to-end ultra-low latency communication. |

As shown in the table, there is a growing trend among servers providing today’s major applications to adopt delay-based rather than loss-based congestion control algorithms. Specifically, examples include the BBR algorithm, invented in 2016 and currently used by heavy-traffic services like YouTube, and the L4S algorithm, which is rapidly gaining traction in mobile networks following the specification of its required technologies in 3GPP Release 18.

Observation 4.	Although NR was standardized during the dominance of loss-based congestion control, application servers are now increasingly adopting delay-based algorithms like BBR and L4S.

In delay-based congestion control algorithms, packet loss over the air interface does not lead to a reduction in throughput, so it may not have to ensure loss-less delivery by utilizing both ARQ and HARQ. Instead, congestion is detected based on transmission delays, thus it may be preferable to reduce delay jitters in the air interface.

Therefore, Layer 2 optimizations may be possible, such as disabling ARQ retransmission functions in the RLC layer, which could otherwise contribute to delay jitter as already mentioned in section 2.3 when using delay-based congestion control algorithms.

We believe that these optimized Layer 2 functions must be configurable by the base station. The reason is to ensure operational flexibility. Operators have the freedom to build a distributed network by deploying CDNs (Content Distribution Networks) near specific base stations. If any of the deployed CDNs adopt a delay-based congestion control algorithm, the corresponding base station should be able to configure its Layer 2 functions accordingly.

Observation 5.	Unlike loss-based algorithms, delay-based congestion control favors reducing delay jitters over ensuring lossless transmission on the air interface.

Proposal 5.	RAN2 studies introducing configuration by the base station of flexible Layer 2 functions to reduce delay jitters in AS interface considering growing share of delay-based congestion control algorithm in transport layer.

- Conclusions

2.1. Considerations of NR layer 2 functions

Observation 1.	As a first step for discussing the 6GR User Plane architecture from scratch, it is appropriate to evaluate the necessity of the Layer 2 functions defined in the NR User Plane.

Proposal 1.	6G Layer 2 supports the following functions, using NR as a baseline and applying modifications or optimizations as needed: header compression (ROHC and UDC), security protection for UP/CP data, in-sequence delivery, discarding of outdated data, ARQ, (re)segmentation, multiplexing, HARQ and MAC CE.

Proposal 2.	RAN2 studies which sublayer supports each legacy function and order of the functions to be processed in Layer 2.

2.2. SDAP layer

Observation 2.	The SDAP layer in NR requires operational effort to ensure proper data forwarding during handover and does not guarantee in-order delivery when QoS flow remapping occurs.

Proposal 3.	RAN2 studies whether SDAP sublayer can be merged to PDCP sublayer, i.e., PDCP can support mapping between QoS Flow and DRB, coordinating with other WGs.

2.3. Retransmission functions in Layer 2

Observation 3.	NR’s dual-layer retransmission, HARQ at MAC and ARQ at RLC, ensures reliability but introduces significant jitter of latency, primarily due to delays in ARQ retransmission at RLC.

Proposal 4.	RAN2 studies integration of ARQ and HARQ in MAC layer to minimize retransmission delay.

2.4. Upper layer-aware L2 functions

Observation 4.	Although NR was standardized during the dominance of loss-based congestion control, application servers are now increasingly adopting delay-based algorithms like BBR and L4S.

Observation 5.	Unlike loss-based algorithms, delay-based congestion control favors reducing delay jitters over ensuring lossless transmission on the air interface.

Proposal 5.	RAN2 studies introducing configuration by the base station of flexible Layer 2 functions to reduce delay jitters in AS interface considering growing share of delay-based congestion control algorithm in transport layer.

- References

[1]	“TCP BBR congestion control comes to GCP – your Internet just got faster,” Google Cloud Blog.

[2]	RFC 9330, “Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture.”

[3]	RFC 9331, “The Explicit Congestion Notification (ECN) Protocol for Low Latency, Low Loss, and Scalable Throughput (L4S).”