3GPP TSG-RAN WG2 Meeting #131bis	R2-2507203

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item:		10.3.2

Source:		Futurewei

Title:		Discussions on 6G Control Plane

Document for:		Discussion and Decision

# Introduction

According to the revised 6G SID [1], RAN2 is the lead WG responsible for the following CP aspects of 6G radio interface design:

(2) Physical Layer structure for 6GR,

  - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

  - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

  - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1]

  - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

  - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2]

  - MIMO operation [RAN1, RAN4]

  - Duplexing [RAN1, RAN4]

  - Initial access [RAN1, RAN2, RAN4]

    - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4]

    - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4]

  - 6GR spectrum utilization and aggregation.  [RAN1, RAN2, RAN4]

  - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4]

  - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1].

    - RAN4 can be involved, if necessary, based on the LS from RAN1

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

In this paper, we discuss some key control plane aspects of 6GR, while the signaling framework for UE capabilities aspect and the mobility aspects are discussed separately in other papers ([2] and [3]) that we submit to the same meeting.

# Discussions

## On RRC modelling

For obvious reasons, RRC_CONNECTED and RRC_IDLE states shall be supported by all 6G UEs.

RRC_INACTIVE state has long been touted as a useful power-saving mechanism for UEs with intermittent data usage, e.g., sensors, IoT devices, etc., due to enabling of rapid transitions to RRC_CONNECTED state when needed. However, in practice, commercial utilization of this feature has been disappointing in 5G NR, partly due to a lack of compelling use cases on the UE side that can significantly benefit from the low-latency advantage and partly due to the complexity would-be-imposed on the network side has been discouraging for the network operators.

In 6G era, UEs supporting new services, such as AI/ML (either as a data producer or a data consumer), ambient intelligence, and Digital Twin, may have a need for sending or receiving a large amount of bursty and aperiodic (e.g., event-driven) data, often with a stringent latency requirement. Therefore, RRC_INACTIVE state may remain important for these types of devices and applications and hence should be kept in 6GR, possibly with further enhancements. An open question is whether the support of RRC_INACTIVE state can be made optional at the UEs.

Proposal 1. RRC_CONNECTED, RRC_INACTIVE, and RRC_IDLE states are supported in 6GR. FFS whether the support of RRC_INACTIVE state can be optional at UEs.

## On RRC functions

For study on RRC functions in 6G, RAN2 can initially focus on the study of the following functions, until RAN Plenary decides whether dual connectivity (DC) and other migration option(s) are to be studied:

- System information provisioning/acquisition;

- Paging;

- Establishment, configuration, maintenance and release of RRC connection;

- UE capabilities signaling;

- Support of NAS signaling transfer;

- Radio bearer (SRB/DRB) management, including addition, modification and release of carrier aggregation;

- Support of AS security, including key management;

- Support of intra-RAT mobility;

- Detection of and recovery from RLF.

Proposal 2. For study on RRC functions in 6G, RAN2 initially focus on the following topics:

- System information provisioning/acquisition;

- Paging;

- Establishment, configuration, maintenance and release of RRC connection;

- UE capabilities signaling;

- Support of NAS signaling transfer;

- Radio bearer management (SRB and DRB), including addition, modification and release of carrier aggregation;

- Support of AS security, including key management;

- Support of intra-RAT mobility;

- Detection of and recovery from RLF.

## On initial access and system information acquisition

Frequent periodic transmissions of system-wide overhead signals, such as SSB, system information blocks, and CSI-RS, may reduce network energy efficiency. Therefore, in 6GR, we can consider replacing frequent periodic transmissions of these signals with infrequent periodic transmissions of them (or a lightweight version of them) plus on-demand or infrequent periodic transmissions of them. For example, frequent periodic transmissions of SSB (e.g., every 5 msec) and SIB1 (e.g., every 20 msec) can be replaced with infrequent periodic transmissions of SSB and SIB1 (e.g., every 160 msec) plus on-demand SSB and on-demand SIB1, where the transmission of on-demand SSB and on-demand SIB1 can be decided by the network by monitoring the traffic arrivals, UL wakeup signals (WUS), cell activation procedures, etc. For another example, CSI-RS transmissions with 256 ports and 40 msec periodicity may be replaced with CSI-RS transmissions with selected 16 ports and 40 msec periodicity plus CSI-RS transmissions with all 256 ports and 160 msec periodicity. Or, the CSI-RS with all 256 ports may be on-demand transmissions (potentially with repetitions and/or higher density) when high-speed data is to be sent. For yet another example, CSI-RS transmissions on a carrier of 400 MHz bandwidth and with 40 msec periodicity may be replaced with CSI-RS transmissions with 40 msec periodicity but each on a 40 MHz bandwidth hopping within the carrier.

When the above techniques and general principles for energy-efficient transmissions/receptions/monitoring are applied to a carrier in carrier aggregation cases, minimum configurations of overhead such as always-on transmissions and cell-level transmissions can be considered. For example, on a secondary component carrier (SCC), the always-on transmissions of SCell SSB may be replaced/reduced to one lightweight synchronization sequence without PBCH transmission, since all the configuration information for the SCC may be delivered to the UE on another carrier. This may further lead to a design on a SCC without SCell configuration at all, further reducing energy consumption.

Proposal 3: RAN2 study the following general energy saving techniques:

- Common Signaling and Channels Optimization

  - Reduce/minimize the energy consumption due to periodic transmission activities via prolonged periodicities, on-demand transmissions, lightweight transmissions, etc.

  - Reduce/minimize the energy consumption due to periodic/continuous reception/monitoring activities via prolonged periodicities, lower-power radio based reception/monitoring, sequence/preamble detection based reception/monitoring, etc.

  - Reduce/minimize the energy consumption on a carrier by reducing/minimizing always-on and cell-level transmissions and cell-level/UE-level configurations.

- Carrier design based solutions

  - Reduce carrier-level energy consumption and overhead via, e.g., removing SCell configurations on a secondary component carrier, on-demand bandwidth adaptation within a carrier.

## On lower layer control message protection

As MAC CEs have increasingly been used for conveying lower layer parameters or configuration indices, and activating or deactivating features, a faked MAC CE sent out by a malicious attacker, if undetected, can render the operation between the victim UE and victim gNB inoperable, or at least, degrade the system performance. In addition, certain MAC CEs, such as C-RNTI and Timing Advance Command MAC CEs, if uncyphered or not concealed, can also reveal the temporary identity and the location of the UE, which can then be used for tracking the UE. Therefore, MAC CEs should be a primary target of study for AS security enhancements in 6G.

Proposal 4. RAN2 study the issue of protecting MAC CEs, at lease for the critical ones, and if needed, work with SA3 to explore possible solutions.

## Potential inter-WG and/or inter-TSG dependencies/impacts

Dependencies/impacts on RAN3, SA2, and CT1 work are anticipated for NAS enhancements, enhancements to existing RAN-CN interface, new RAN-CN interfaces (SBA-based) and interactions, if any. Early alignments with these WGs in these design aspects are desirable.

# Conclusions

Based on the above discussions, we propose the following:

Proposal 1. RRC_CONNECTED, RRC_INACTIVE, and RRC_IDLE states are supported in 6GR. FFS whether the support of RRC_INACTIVE state can be optional at UEs.

Proposal 2. For study on RRC functions in 6G, RAN2 initially focus on the following topics:

- System information provisioning/acquisition;

- Paging;

- Establishment, configuration, maintenance and release of RRC connection;

- UE capabilities signaling;

- Support of NAS signaling transfer;

- Radio bearer management (SRB and DRB), including addition, modification and release of carrier aggregation;

- Support of AS security, including key management;

- Support of intra-RAT mobility;

- Detection of and recovery from RLF.

Proposal 3: RAN2 study the following general energy saving techniques:

- Common Signaling and Channels Optimization

  - Reduce/minimize the energy consumption due to periodic transmission activities via prolonged periodicities, on-demand transmissions, lightweight transmissions, etc.

  - Reduce/minimize the energy consumption due to periodic/continuous reception/monitoring activities via prolonged periodicities, lower-power radio based reception/monitoring, sequence/preamble detection based reception/monitoring, etc.

  - Reduce/minimize the energy consumption on a carrier by reducing/minimizing always-on and cell-level transmissions and cell-level/UE-level configurations.

- Carrier design based solutions

  - Reduce carrier-level energy consumption and overhead via, e.g., removing SCell configurations on a secondary component carrier, on-demand bandwidth adaptation within a carrier.

Proposal 4. RAN2 study the issue of protecting MAC CEs, at lease for the critical ones, and if needed, work with SA3 to explore possible solutions.

# References

RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, et al., RAN#109.

R2-2507201, Discussions on General Aspects of 6GR Layer 2, Futurewei, RAN2#131bis.

R2-2507206, Discussions on 6G Mobility, Futurewei, RAN2#131bis.