3GPP TSG-RAN WG2 Meeting #131bis	R2-2507208

Prague, Czech Republic, October 13rd – 17th, 2025

Agenda item:		9.7.2 Other

Title:		Discussion on voice support over NT-IoT-NTN

Source:		ETRI

Document for:		Discussion/Decision

# 1. Introduction

During the 109th meeting of RAN, a new WI on IoT-NTN Phase 4 was approved, including the following objectives [1]:

Objectives - Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required. - The objective can be updated with further details based on the outcome of the above down-selection. - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2] - Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4] - Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements - Note: Coordination with SA4 is expected

In this paper, we would like to provide our views on the highlighted aspects related to voice support over NB-IoT-NTN.

# 2. Discussion

2.1	Background

RAN approved the IoT_NTN_Ph4 WID in support of IMS voice over NB-IoT-NTN via GSO (GEO) targeting Rel-20. The WID calls for a down-selection between CP and UP solutions (RAN#110) and lists specific RAN enhancements (SPS for DL/UL, RRC connection setup modifications, emergency call support, UE Tx power study) as part of the core work.

SA has, in parallel, progressed enabling studies: SA4 SID on Ultra-Low Bit-Rate Speech (PHY 1–5 kbps; codec 0.8–4.8 kbps) and SA2 SID on IMS voice over GSO via NB-IoT-NTN, whose WT#1 includes the CP vs UP comparison and aims to conclude by TSG#110 (Dec-2025).

Performance requirements for satellite access captured by SA1/SA2 include IMS voice over GEO with [1–3] kbit/s experienced rate (UL/DL) and call setup time ≤ 30 s for handheld UEs. these KPIs may frame the RAN design space for NB-IoT-NTN.

NB-IoT-NTN over GEO must deliver conversational voice under:

- Long RTT (hundreds of milliseconds one-way), variable jitter, and intermittent channel conditions.

- Tight spectral budget (NB-IoT single PRB) and low device complexity/power.

- IMS interworking (SIP/SDP, RTP/RTCP) with QoS enforcement and emergency call support per SA requirements (≤30 s setup).

These constraints may require predictable latency, robustness to loss, and policy/QoS control end-to-end—features that naturally align with UP bearers and RAN QoS machinery.

2.1	ETRI’s views on CP and UP solutions

CP solution

The CP solution leverages the existing NAS signaling path to carry voice payloads without establishing a dedicated bearer in the user plane. Its strength lies in its simplicity: the reuse of existing signaling structures minimizes modifications for low-cost IoT devices, and initial call establishment procedures may appear shorter since no user plane bearer setup is required.

Nevertheless, this solution has intrinsic drawbacks. The control plane is not designed for sustained real-time conversational services, which leads to significant challenges in latency, jitter, and packet loss management. Since QoS differentiation is not available on the control plane, voice packets are treated similarly to control signaling, limiting the ability to ensure predictable service quality. These issues are particularly acute in the GEO NTN environment, where inherent propagation delays and variable link conditions exacerbate jitter and delay. Furthermore, emergency call handling, which requires explicit prioritization and policy-based routing, is poorly supported in the CP context.

Observation 1. The CP solution is simple but unsuitable for reliable GEO NTN voice due to its lack of QoS and latency control.

UP solution

The UP solution establishes a dedicated bearer specifically for voice traffic. This enables the application of standardized QoS frameworks, including guaranteed bit rates, latency and jitter management, and bearer-level prioritization for emergency services. Importantly, the UP solution aligns with the established VoNR architecture for terrestrial NR, thereby ensuring compatibility and consistency across terrestrial and non-terrestrial deployments.

Although bearer establishment introduces additional signaling steps and requires slightly higher implementation complexity, these trade-offs are outweighed by the benefits of assured quality, IMS architectural compatibility, and future-proof scalability. The UP approach allows for clear integration with IMS signaling, RTP/RTCP transport, and supports long-duration voice sessions in a way that CP fundamentally cannot.

Observation 2. The UP solution, though slightly more complex, is best suited for GEO NTN voice as it ensures QoS, IMS compatibility, and reliable long-duration service.

The following table provides a high-level comparison of CP and UP solutions in the GEO NB-IoT-NTN context:

Table 1. Main features of CP and UP Solutions

| Aspect | CP Solution | UP Solution |
| --- | --- | --- |
| Complexity | Simpler, reuses NAS signaling | Higher, requires dedicated bearer setup |
| Call Setup | Potentially faster initially (no bearer creation) | Slightly longer (bearer setup included) but within ≤30s KPI |
| QoS Support | None; best-effort only | Full QoS differentiation (GBR, latency, jitter control) |
| Codec fits | Header and NAS overhead may be proportionally high | RTP/UDP/IP overhead can be minimized; periodic frames map cleanly to SPS |
| Voice Quality | Unpredictable under GEO delay/jitter | Predictable, consistent conversational quality |
| Emergency Call | Limited prioritization | Explicit support with QoS and policy enforcement |
| Scalability | Poor for long-duration sessions | Robust, supports extended sessions and supplementary services |
| Standard Alignment | Diverges from VoNR IMS model | Fully aligned with IMS/VoNR architecture |

From this comparison, it is clear that the CP solution’s benefits are limited to simplicity and short-term implementation convenience, whereas the UP solution provides the necessary robustness, QoS management, and standard alignment to make voice over GEO NB-IoT-NTN viable and future-proof.

Proposal 1. RAN2 to down-select the User Plane (UP) solution for IMS voice over NB-IoT-NTN.

# 3. Conclusions

Observation 1. The CP solution is simple but unsuitable for reliable GEO NTN voice due to its lack of QoS and latency control.

Observation 2. The UP solution, though slightly more complex, is best suited for GEO NTN voice as it ensures QoS, IMS compatibility, and reliable long-duration service.

Proposal 1. RAN2 to down-select the User Plane (UP) solution for IMS voice over NB-IoT-NTN.

# 4. References

[1]	RP-252473, “Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4”.