3GPP TSG-RAN WG2 Meeting#131bis                               	R2-2507244

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item:	8.9.4

Source: 	Google

Title: 	Impact of the S&F mode transition time on AS

Document for:	Discussion and Decision

# Introduction

In Rel-19, the benefits of enabling the Store & Forward operation for IoT-NTN are identified and the following scopes are included in the revised WID of Rel-19 IoT-NTN [1].

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required.

RAN2 has agreed to introduce and signal an S&F mode transition time (i.e., t-ModeSwitching) in SIB31. This transition time indicates when the serving cell will switch between S&F and normal modes, depending on the presence or absence of sf-OperationMode in SIB1. The UE forwards this time to the upper layers upon acquisition from SIB31. This paper explores the implications and potential uses of t-ModeSwitching, with a specific focus on its application within the AS layer.

# Discussion

Since the satellite operation mode of a serving cell impacts both the UE AS (regarding cell access allowance) and UE NAS (regarding S&F capability indication when connecting to the cell), it is crucial for UEs to know the exact timing of operational mode changes. Given that the S&F mode indication (i.e., sf-OperationMode) and legacy barring indications can only change at SI modification period boundaries, the S&F mode transition time (i.e., t-ModeSwitching) signaled in SIB31 provides UEs with a more precise indication of when the serving cell changes its operational mode.

As the S&F mode indication (i.e., sf-OperationMode) and the legacy barring indications can only change at the boundaries of SI modification periods, the S&F mode transition time (i.e., t-ModeSwitching) signaled in SIB31 allows UEs to know more precisely when the serving cell changes its operational mode.

RAN2 agreed (in RAN2#129bis) that the transition time from the normal to the S&F mode is useful to the UE NAS. However, if the AS layer could effectively use this transition time, it would also provide an advantage to Rel-19 UEs that don't support S&F operation. According to TS 36.304 [2], if a Rel-19 UE not capable of the S&F operation considers a cell as barred due to the legacy barring indication (i.e., cellBarred-NTN = barred), the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

5.3.1 Cell status and cell reservations <irrelevant texts omitted> When cell status "barred" is indicated or to be treated as if the cell status is "barred", <irrelevant texts omitted> - else - If the field intraFreqReselection in field cellAccessRelatedInfo in SystemInformationBlockType1 (or SystemInformationBlockType1-BR message or SystemInformationBlockType1-NB) message is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled. - The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds. - If the field intraFreqReselection in field cellAccessRelatedInfo in SystemInformationBlockType1 (or SystemInformationBlockType1-BR message or SystemInformationBlockType1-NB) message is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell; - The UE shall exclude the barred cell and the cells on the same frequency as a candidate for cell selection/reselection for 300 seconds.

If the network decides to prohibit non-S&F-capable UEs from accessing a cell operating in the S&F mode, the network may want to lift this restriction immediately when the cell reverts to normal mode. However, the 300-second barring rule specified in TS 36.304 prevents non-S&F-capable UEs from immediately reconsidering these newly-unbarred cells for selection or reselection.

If a non-S&F-capable UE considers a cell as barred because it's currently operating in S&F mode, the UE shall exclude the cell as a candidate for cell selection/reselection for 300 seconds, even if the cell will revert to the normal mode before the 300-second timer expires.

Therefore, we think at least for the Rel-19 UE not capable of the S&F operation, it should be possible for the UE to exclude a cell operating in the S&F mode as the candidate for cell selection/reselection for less than 300 seconds. For instance, a Rel-19 UE not capable of the S&F operation can exclude a barred cell as the candidate for cell selection/reselection until t-ModeSwitching, if sf-OperationMode is also present and indicates ‘notBarred’.

A Rel-19 non-S&F-capable UE can exclude a barred cell as the candidate for cell selection/reselection until t-ModeSwitching, if sf-OperationMode is signaled by the cell with the value ‘notBarred’ (TP is attached in Annex A).

For UEs that don't support S&F, the goal is to keep them on normal-mode cells for as long as possible, even if they could camp on an S&F-mode cell. Therefore, when a non-S&F-capable UE is camping on a normal-mode cell and receives an indication that S&F mode is about to start, it should trigger intra-/inter-frequency measurements. Similar to the t-service mechanism in Rel-18, these measurements should be triggered regardless of other conditions, such as serving cell quality or distance to the reference location.

A Rel-19 non-S&F-capable UE can utilize the transition time from the normal mode to the S&F mode to determine when to trigger the measurement for cell reselection (TP is attached in Annex B).

# Conclusion

In this paper, we discuss several leftover issues on the S&F mode transition time. Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

Observation 1	As the S&F mode indication (i.e., sf-OperationMode) and the legacy barring indications can only change at the boundaries of SI modification periods, the S&F mode transition time (i.e., t-ModeSwitching) signaled in SIB31 allows UEs to know more precisely when the serving cell changes its operational mode.

Observation 2	If a non-S&F-capable UE considers a cell as barred because it's currently operating in S&F mode, the UE shall exclude the cell as a candidate for cell selection/reselection for 300 seconds, even if the cell will revert to the normal mode before the 300-second timer expires.

Proposal 1	A Rel-19 non-S&F-capable UE can exclude a barred cell as the candidate for cell selection/reselection until t-ModeSwitching, if sf-OperationMode is signaled by the cell with the value ‘notBarred’ (TP is attached in Annex A).

Proposal 2	A Rel-19 non-S&F-capable UE can utilize the transition time from the normal mode to the S&F mode to determine when to trigger the measurement for cell reselection (TP is attached in Annex B).

# Reference

RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.

3GPP TS 36.304 v18.4.0, Technical Specification Group Radio Access Network (E-UTRA); User Equipment (UE) procedures in idle mode.

# Annex A: TP for relaxing the access restriction

--------------------------------------------- TP on the running CR for TS 36.304 ---------------------------------------------------

5.3.1	Cell status and cell reservations

Cell status and cell reservations are indicated in the SystemInformationBlockType1 message (or SystemInformationBlockType1-BR message or SystemInformationBlockType1-NB message) TS 36.331 [3] by means of the following fields:

-	cellBarred (IE type: "barred" or "not barred") 
This field indicates if the cell is barred for connectivity to EPC.
This field is ignored by the UEs supporting crs-IntfMitig while crs-IntfMitigEnabled is included in SIB1. 
This field is ignored by the BL UEs or UEs in CE supporting ce-CRS-IntfMitig while crs-IntfMigitNumPRBs is included in SIB1-BR.
This field is ignored by UEs supporting NTN while cellBarred-NTN is included in SIB1-BR or SIB1-NB.
In case of multiple EPC PLMNs indicated in SIB1/SIB1-BR, this field is common for all EPC PLMNs

NOTE 1:	IAB-MT ignores the cellBarred, cellReservedForOperatorUse, intraFreqReselection and csg-Indication (i.e. treats intraFreqReselection as if it was set to allowed and the csg-Indication as if it was set to FALSE) as defined in TS 36.331 [3].

-	cellBarred-5GC (IE type: "barred" or "not barred")
This field indicates if the cell is barred for connectivity to 5GC.
This field is ignored if the UE does not support E-UTRA connected to 5GC or if the UE supports network-based CRS interference mitigation and nw-BasedCRS-InterferenceMitigation is included in SystemInformationBlockType1.
In case of multiple 5GC PLMNs indicated in SIB1, this field is common for all 5GC PLMNs.

-	cellReservedForOperatorUse (IE type: "reserved" or "not reserved")
This field indicates if the cell is reserved for operator use.
This field is ignored by the UEs supporting crs-IntfMitig while crs-IntfMitigEnabled is included in SIB1. 
This field is ignored by the BL UEs or UEs in CE supporting ce-CRS-IntfMitig while crs-IntfMigitNumPRBs is included in SIB1-BR. 
In case of multiple EPC or 5GC PLMNs indicated in SIB1/SIB1-BR, this field is specified per EPC or 5GC PLMN.

-	cellBarred-CRS (IE type: "barred" or "not barred")
This field indicates if the cell is barred for connectivity to EPC for UEs supporting network-based CRS interference mitigation.
barred means the cell is barred for UEs supporting crs-IntfMitig while crs-IntfMitigEnabled is included in SIB1. For BL UEs or UEs in CE capable of ce-CRS-IntfMitig, barred means the cell is barred while crs-IntfMitigNumPRBs is included in SIB1-BR.
This field is ignored by the UE if the UE does not support CRS interference mitigation or while crs-IntfMitigConfig is not included in SIB1 (SIB1-BR for BL UEs or UEs in CE).
In case of multiple PLMNs indicated in SIB1/SIB1-BR, this field is common for all PLMNs.

-	cellBarred-5GC-CRS (IE type: "barred" or "not barred")
This field indicates if the cell is barred for connectivity to 5GC for UEs supporting network-based CRS interference mitigation.
This field is ignored if the UE does not support E-UTRA connected to 5GC or network-based CRS interference mitigation.
In case of multiple 5GC PLMNs indicated in SIB1, this field is common for all 5GC PLMNs.

-	cellReservedForOperatorUse-CRS (IE type: "reserved" or "not reserved")
This field indicates if the cell is reserved for operator use for UEs supporting network-based CRS interference mitigation.
reserved means the cell is "reserved" for operator use for UEs supporting crs-IntfMitig while crs-IntfMitigEnabled is included in SIB1. 
For BL UEs or UEs in CE capable of ce-CRS-IntfMitig, reserved means the cell is "reserved" for operator use while crs-IntfMitigNumPRBs is included in SIB1-BR.
This field is ignored if the UE does not support CRS interference mitigation or while crs-IntfMitigConfig is not included in SIB1 (SIB1-BR for BL UEs or UEs in CE).
In case of multiple PLMNs indicated in SIB1/SIB1-BR, this field is specified per PLMN.

-	iab-Support (IE type: "true")
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is specified per PLMN. This field indicates if the cell is barred for IAB node or the cell does not support IAB node, or both. When this field is absent, the IAB node shall treat this cell as if cell status is barred.

-	cellBarred-NTN (IE type: "barred" or "not barred")
This field indicates if the cell is barred for connectivity to EPC via NTN.
This field is ignored if the UE does not support NTN connectivity.

The following description for handling of barred and reserved cells is per CN type. If the UE supports more than one CN type, the UE shall only exclude a cell as candidate for selection/reselection if it is excluded for both CN types.

NOTE 2:	Fields cellBarred-CRS and cellReservedForOperatorUse-CRS are not indicated in SystemInformationBlockType1-NB

When cell status is indicated as "not barred" and "not reserved" for operator use,

-	All UEs shall treat this cell as candidate during the cell selection and cell reselection procedures.

When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN,

-	UEs assigned to Access Class 11 or 15 (or corresponding Access Identity) operating in their HPLMN/EHPLMN shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for that PLMN set to "reserved".

-	UEs assigned to an Access Class in the range of 0 to 9 (or corresponding Access Identity 0), 12 to 14 (or corresponding Access Identity) or to Access Identity 1, 2 or 3 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN or the selected PLMN.

NOTE 3:	ACs 11, 15 (or corresponding Access Identity) are only valid for use in the HPLMN/ EHPLMN; ACs 12, 13, 14 (or corresponding Access Identity) are only valid for use in the home country TS 22.011 [4].

NOTE 4:	Access Identities 1, 2 are valid in the PLMNs as specified in TS 22.261 [41].

NOTE 5:	Access Identity 3 is only valid for PLMNs that indicate to potential Disaster Inbound Roamers that the UEs can access the PLMN as specified in TS 22.261 [41].

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls.

-	The UE shall consider other cells for cell selection/reselection according to the following rule:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MasterInformationBlock (or MasterInformationBlock-NB), the SystemInformationBlockType1 (or SystemInformationBlockType1-BR message or SystemInformationBlockType1-NB), the SystemInformationBlockType2 (or SystemInformationBlockType2-NB) or SystemInformationBlockType31 (or SystemInformationBlockType31-NB) if broadcasted for UEs supporting NTN:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	the UE may select the same cell in normal coverage if the UE was barred in the cell due to being unable to acquire MasterInformationBlock, SystemInformationBlockType1-BR, or SystemInformationBlockType2 in enhanced coverage, but was able to acquire MasterInformationBlock, SystemInformationBlockType1, and SystemInformationBlockType2 in normal coverage, if the selection criteria are fulfilled.

-	the UE may select the same cell in enhanced coverage if the UE was barred in the cell due to being unable to acquire MasterInformationBlock, SystemInformationBlockType1, or SystemInformationBlockType2 in normal coverage, but was able to acquire MasterInformationBlock, SystemInformationBlockType1-BR, and SystemInformationBlockType2, if the selection criteria are fulfilled.

-	else

-	If the cell is a CSG cell:

-	the UE may select another cell on the same frequency if the selection/reselection criteria are fulfilled.

-	else

-	If the field intraFreqReselection in field cellAccessRelatedInfo in SystemInformationBlockType1 (or SystemInformationBlockType1-BR message or SystemInformationBlockType1-NB) message is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled.

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds, or until t-ModeSwitching if t-ModeSwitching is present in SystemInformationBlockType31 (or SystemInformationBlockType31-NB) and sf-OperationMode is presented in SystemInformationBlockType1 (or SystemInformationBlockType1-NB) with the value ‘notBarred’.

-	If the field intraFreqReselection in field cellAccessRelatedInfo in SystemInformationBlockType1 (or SystemInformationBlockType1-BR message or SystemInformationBlockType1-NB) message is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell;

-	The UE shall exclude the barred cell and the cells on the same frequency as a candidate for cell selection/reselection for 300 seconds, or until t-ModeSwitching if t-ModeSwitching is present in SystemInformationBlockType31 (or SystemInformationBlockType31-NB) and sf-OperationMode is presented in SystemInformationBlockType1 (or SystemInformationBlockType1-NB) with the value ‘notBarred’.

The cell selection of another cell may also include a change of RAT or, if the previous and selected cell are both E-UTRA cells, a change of the CN type.

# Annex B: TP for triggering the measurement for cell reselection

--------------------------------------------- TP on the running CR for TS 36.304 ---------------------------------------------------

5.2.4.2	Measurement rules for cell re-selection

For NB-IoT measurement rules for cell re-selection is defined in clause 5.2.4.2.a.

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the measurements are performed using RSS as specified in [10] and the serving cell fulfils Srxlev > SIntraSearchP:

-	If distanceThresh and referenceLocation are broadcast in SystemInformationBlockType31, and if the UE has obtained its location information:

-	If referenceLocation is set to fixedReferenceLocation and if the UE supports location-based measurement initiation (quasi-)Earth for fixed cell, referenceLocation is used as serving cell reference location.

-	If the distance between the UE and the serving cell reference location is shorter than distanceThresh, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE shall perform intra-frequency measurements.

-	If referenceLocation is set to movingReferenceLocation and if the UE supports location-based measurement initiation for Earth moving cell, the UE derives the serving cell reference location based on ephemeris, epochTime and referenceLocation.

-	If the distance between the UE and the serving cell reference location is shorter than distanceThresh, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE shall perform intra-frequency measurements.

-	Else, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE may choose not to perform intra-frequency measurements.

-	Else if the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ:

-	If distanceThresh and referenceLocation are broadcast in SystemInformationBlockType31, and if the UE has obtained its location information:

-	If referenceLocation is set to fixedReferenceLocation and if the UE supports location-based measurement initiation for (quasi-)Earth fixed cell, the referenceLocation is used as serving cell reference location.

-	If the distance between the UE and the serving cell reference location, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE shall perform intra-frequency measurements.

-	If referenceLocation is set to movingReferenceLocation and if the UE supports location-based measurement initiation for Earth moving cell, the UE derives the serving cell reference location based on ephemeris, epochTime and referenceLocation.

-	If the distance between the UE and the serving cell reference location is shorter than distanceThresh, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE shall perform intra-frequency measurements.

-	Else, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to TS 36.133 [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If the measurements are performed using RSS as specified in [10] and the serving cell fulfils Srxlev > SnonIntraSearchP:

-	If distanceThresh and referenceLocation are broadcast in SystemInformationBlockType31, and if the UE has obtained its location:

-	If referenceLocation is set to fixedReferenceLocation and if the UE supports location-based measurement initiation for (quasi-)Earth fixed cell, the referenceLocation is used as serving cell reference location. The referenceLocation is used as serving cell reference location.

-	If the distance between the UE and serving cell reference location is shorter than distanceThresh the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	If referenceLocation is set to movingReferenceLocation and if the UE supports location-based measurement initiation for Earth moving cell, the UE derives the serving cell reference location based on ephemeris, epochTime and referenceLocation.

-	If the distance between the UE and serving cell reference location is shorter than distanceThresh the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	Else, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else if the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ:

-	If distanceThresh and referenceLocation are broadcast in SystemInformationBlockType31, and if the UE supports location-based measurement initiation and has obtained its location:

-	If referenceLocation is set to fixedReferenceLocation and UE supports location-based measurement initiation for (quasi-)Earth fixed cell, the referenceLocation is used as serving cell reference location.

-	If the distance between the UE and serving cell reference location is shorter than distanceThresh, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	If referenceLocation is set to movingReferenceLocation and UE supports location-based measurement initiation for Earth moving cell, the UE derives the serving cell reference location based on ephemeris, epochTime and referenceLocation.

-	If the distance between the UE and serving cell reference location is shorter than distanceThresh, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	Else, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Otherwise, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	If the UE supports relaxed monitoring and s-SearchDeltaP is present in SystemInformationBlockType3, the UE may further limit the needed measurements, as specified in clause 5.2.4.12.

If t-Service is present in SystemInformationBlockType3 of the serving cell, UE shall perform intra-frequency, inter-frequency or inter-RAT measurements, before the time t-Service regardless of the distance between the UE and serving cell reference location, and regardless whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ. The exact time to start measurements before t-Service is up to UE implementation and t-ServiceStartNeigh if present in SystemInformationBlockType33 may be used to decide on when to start measurements. UE shall perform measurements of higher priority inter-frequencies or inter-RAT frequencies regardless of the remaining service time of the serving cell.

If t-ModeSwitching is present in SystemInformationBlockType31 of the serving cell and sf-OperationMode is absent in SystemInformationBlockType1 of the serving cell, UE not supporting the satellite S&F operation shall perform intra-frequency, inter-frequency or inter-RAT measurements, before the time t-ModeSwitching regardless of the distance between the UE and serving cell reference location, and regardless whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ. The exact time to start measurements before t-ModeSwitching is up to UE implementation and t-ServiceStartNeigh if present in SystemInformationBlockType33 may be used to decide on when to start measurements.