3GPP TSG-RAN WG2 Meeting #131-bis	R2-2507247

Prague, Czech Republic, Oct. 13rd – 17th, 2025

# Agenda item:	10.4

# Source:	ETRI

# Title:	Discussion on 6G mobility

# Document for:	Discussion, Decision

- Introduction

Mobility management is one of the most critical features of cellular systems, enabling seamless service continuity while UEs move across cells and access technologies. In 5G, multiple handover (HO) schemes have been introduced – e.g., conventional L3 HO, Conditional HO (CHO), Dual Active Protocol Stack (DAPS), L1/L2-Triggered Mobility (LTM) and Conditional LTM(CLTM). Each scheme was designed for a specific purpose, but all of them together make the procedures complicated and not well integrated. It is expected that more scenarios such as NTN, multi-TRP and high-speed mobility will be integrated in 6G. If each scenario is handled separately, the number of mobility procedures will increase even further. Such diversity without harmonization results in higher design complexity, duplicated functionality, and inconsistent implementations across vendors and operators. Therefore, there is a strong need for a unified mobility framework that can serve as a baseline and integrate various mobility schemes into a single design principle. This includes unifying signaling procedures, simplifying RRC configuration management, and aligning measurement frameworks.

This paper discusses mobility-related issues that need to be studied during the 6G SI phase in order to design a unified mobility framework.

- Discussion

In 5G, several HO schemes such as L3 HO, CHO, DAPS, LTM, and CLTM were introduced independently, each to address a specific scenario. As these schemes accumulate, they generate overlapping functionalities, duplicated signaling, and inconsistent behaviors between vendors and operators. Looking toward 6G, the range of mobility scenarios is expected to expand further with NTN, multi-TRP, relay and extreme mobility use cases like high-speed trains and UAVs. If the current approach of defining separate HO procedures for each new scenario continues, the complexity of both UE and network implementations will grow exponentially, leading to unsustainable standardization and testing costs. Therefore, 6G requires a single unified mobility framework to serve as a common baseline. This framework should define common HO states, events, and signaling structures, while allowing extensions for special cases. Such an approach will simplify specifications, improve interoperability, and provide a scalable foundation for future innovations.

Observation 1: In 5G, multiple HO schemes without harmonization results in higher design complexity, duplicated functionality, and inconsistent implementations across vendors and operators. A single unified framework is necessary to integrate these schemes and provide a common baseline for 6G mobility

Proposal 1: Study a single mobility framework to avoid very complex design with various mobility schemes.

Handover signaling is another source of fragmentation. In 5G specification, the signaling procedures for conventional L3 HO/CHO and LTM are different, even though they share similar objectives: prepare a new cell, command the HO, and confirm completion. This lack of commonality complicates UE and network implementations, inflates standardization work, and reduces efficiency. A unified signaling framework, built on a common preparation/command/completion structure, can cover all scenarios. This would also allow reuse of existing messages and reduce unnecessary diversity.

Observation 2: Handover signaling in 5G specifications differs across scenarios, although the basic phases are the same. A unified HO signaling with common steps can simplify implementation and standardization.

An example of the unified HO signaling structure is illustrated in Figure 1 and is inspired by the signaling flows already introduced for CHO and LTM. The key feature of this design is the clear separation between the preparation and command phases. In legacy L3 HO, preparation and command are delivered together in a single RRCReconfiguration message. As a result, the network must consider preparation time when deciding the handover point, which may force earlier HO decisions than necessary. By contrast, in the unified structure, candidate cells can be prepared in advance during the preparation phase, and the HO can be executed later with only a simple HO command. This allows the HO decision to be made closer to the optimal timing and enables the UE to switch more quickly when the command is received.

This unified signaling approach therefore combines the strengths of CHO and LTM signaling structures, while also covering the conventional L3 HO case. When only one candidate cell is prepared, the flow is functionally equivalent to legacy L3 HO, but with more flexibility and efficiency due to the explicit phase separation. Most signaling messages can be reused from existing RRC procedures, while the HO command can be derived with minimal modifications from the LTM cell switch command MAC CE. This unified framework reduces signaling diversity, improves flexibility, and supports fast and reliable mobility across different HO scenarios.

Figure 1 An example of unified handover signaling

Proposal 2: Study a unified HO signaling (e.g., preparation /command/completion) for all handover scenarios.

Configuration management at the RRC level is another area where unnecessary complexity arises. Today, the same candidate cell may be configured multiple times depending on whether it is used for CHO, LTM-based HO, or conventional L3 HO. This redundancy increases signaling load, wastes UE resources, and may result in inconsistent configurations between procedures. For example, one UE may store multiple RRC configurations for the same target cell when it appears in different HO contexts. In 6G, a unified principle should be adopted: each candidate cell is associated with a single, scenario-independent RRC configuration. Different HO schemes that reference the same candidate cell should reuse that configuration. Specific HO features can be indicated via sub-IEs or parameters. This ensures consistency, reduces redundancy, and simplifies both UE and network operations.

Observation 3: The same candidate cell can be configured multiple times under different HO scenarios. A single common RRC configuration per candidate cell can avoid duplication and inconsistency.

Proposal 3: Study use of one common RRC configuration for the same candidate cell across all handover scenarios.

Mobility relies heavily on accurate and timely measurements. Current designs use two parallel frameworks:

- L3/RRM-oriented measurements (e.g., RSRP/RSRQ, Event A3/A5) are configured and reported via RRC signaling, following RRC-defined events and timers.

- L1/LTM-triggered measurements (e.g., beam failure detection, PHY-level link quality triggers) are primarily generated directly at lower layers and reported with configurations partially provided by RRC, but operate independently of RRC measurement events.

Because these two frameworks are configured, triggered, and reported separately, UEs may duplicate effort and the network must process two heterogeneous types of reports. This separation creates overhead and risks misalignment, such as inconsistent cell selection due to timing differences between L1 beam failure detection and L3 RRM event triggers, potentially increasing handover failure rates. In 6G, these two frameworks should be unified into a single measurement framework. Such a framework would align configuration, triggering, and reporting mechanisms, ensuring that UE reporting is efficient and that the network interprets measurements consistently. The result is reduced duplication, streamlined signaling, and more reliable mobility control.

Observation 4: Currently, L1 measurements and L3 RRM measurements are configured and reported through separate mechanisms. This independent handling results in duplication and possible inconsistencies between layers.

Proposal 4: Study an integration of L1/L3 measurements into a single measurement framework.

During 5G standardization, RACH-less HO was studied and shown to be very efficient in reducing HO latency. As a result, it has already been applied in some HO types, such as Conditional HO, LTM, and Conditional LTM. By removing the random access step, these methods can reduce the interruption time during handover. For the services that need ultra-low latency and minimal interruption, such as URLLC, XR, and high-speed mobility, RACH-less HO has been essential. In 6G, the use of RACH-less HO should be expanded further. Conventional L3 HO, which is still the baseline mobility procedure, should also support RACH-less operation. This would make RACH-less HO a common feature across all major HO types in 6G, ensuring consistently low latency and seamless service continuity from Day-1 deployment. Such an approach would unify latency enhancements across different HO mechanisms and guarantee smooth service for both conventional and advanced scenarios.

Observation 5: RACH-less HO was shown to be highly effective in reducing HO latency and has already been applied in CHO, LTM, and CLTM in 5G to reduce latency.

Proposal 5: Study RACH-less HO features as a technique to achieve low HO latency.

In addition to latency, robustness is another key performance requirement for 6G mobility. In 5G, Conditional HO and Conditional LTM were introduced to enhance robustness by preparing candidate cells in advance and enabling UE-triggered cell switch under predefined execution conditions. These mechanisms allowed UEs to switch immediately to a prepared cell and enabled robust mobility management, which proved effective in reducing HO failure rates and service interruption in dynamic environments. If CLTM can be extended to inter-CU in 6G, it would further strengthen this capability by extending conditional mobility across CU boundaries.

Observation 6: CHO and CLTM have already been applied in 5G to improve mobility robustness and have proven effective in reducing HO failure rates and service interruption.

To carry these benefits forward, 6G should support UE-triggered HO schemes such as CHO and CLTM from the very beginning as Day-1 features. Early adoption will ensure that robustness enhancements are available not only for conventional mobility scenarios but also for more demanding use cases such as NTN integration, ultra-dense deployments, and high-speed mobility.

Proposal 6: Study UE-triggered HO features (e.g., CHO and CLTM) as a technique to enhance HO robustness.

- Conclusions

In this paper, we discussed mobility-related issues that need to be studied during the 6G SI phase in order to design a unified mobility framework. Followings are observations and proposals made in this paper:

Observation 1: In 5G, multiple HO schemes without harmonization results in higher design complexity, duplicated functionality, and inconsistent implementations across vendors and operators. A single unified framework is necessary to integrate these schemes and provide a common baseline for 6G mobility

Observation 2: Handover signaling in 5G specifications differs across scenarios, although the basic phases are the same. A unified HO signaling with common steps can simplify implementation and standardization.

Observation 3: The same candidate cell can be configured multiple times under different HO scenarios. A single common RRC configuration per candidate cell can avoid duplication and inconsistency.

Observation 4: Currently, L1 measurements and L3 RRM measurements are configured and reported through separate mechanisms. This independent handling results in duplication and possible inconsistencies between layers.

Observation 5: RACH-less HO was shown to be highly effective in reducing HO latency and has already been applied in CHO, LTM, and CLTM in 5G to reduce latency.

Observation 6: CHO and CLTM have already been applied in 5G to improve mobility robustness and have proven effective in reducing HO failure rates and service interruption.

Proposal 1: Study a single mobility framework to avoid very complex design with various mobility schemes.

Proposal 2: Study a unified HO signaling (e.g., preparation /command/completion) for all handover scenarios.

Proposal 3: Study use of one common RRC configuration for the same candidate cell across all handover scenarios.

Proposal 4: Study an integration of L1/L3 measurements into a single measurement framework.

Proposal 5: Study RACH-less HO features as a technique to achieve low HO latency.

Proposal 6: Study UE-triggered HO features (e.g., CHO and CLTM) as a technique to enhance HO robustness.

- References

- RP-251395, Revised SID: Study on 6G Scenarios and requirements

- RP-251881, New SID: Study on 6G Radio

- TR 38.914, Study on 6G Scenarios and requirements