3GPP TSG-RAN WG2 Meeting #131bis	R2-2507251

# Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:		8.5.3 (Netw_Energy_NR_enh-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on NES user plane open issues

Document for:	Discussion and Decision

1.	Introduction

This document discusses open issues for NES user plane, which has been raised during the post email discussion ([POST131][111][NES] 38.321 CR). Two issues are identified as follows:

MAC Issue 1: RA-RNTI for PRACH adaptation (described in LS R1-2506587) MAC issue 2: whether "initialDownlinkBWP-RedCap" can be configured before SIB1 request procedure (e.g. acquired by UE earlier) and can be used for PDCCH monitoring. If so, can the network know that for a redcap UE performing an OD-SIB1 request.

In this contribution, we discuss mainly the first issue: RA-RNTI for PRACH adaptation.

2.	Discussion

In R2-131 meeting, the WI for network energy saving enhancement was completed from RAN2 point of view. However, the reply LS from RAN1 (R1-2506587) requires RAN2 to revisit the following RAN2 agreements about RA_RNTI calculation for additional ROs. According to the reply LS, there is no proper place in RAN1 specifications to capture the impact on f_id because f_id is not defined in RAN1 specification and the index nRA in RAN1 specification is an index for physical signal generation and cannot be directly used for determination of f_id.

R2-130 agreements on RA-RNTI collision issue No need to extend the RA-RNTI space for additional PRACH occasions, i.e. no RAN2 impact for RA-RNTI calculation. RAN2 assumes that f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs. RAN2 understand there is no MAC impact. Send an LS to RAN1 to inform RAN2 agreement.

According to RAN1 specification (subclause 6.3.3.2 in TS38.211), the index nRA numbers the PRACH frequency resource in increasing order, starting from the lowest frequency. Previously, we assumed that the index f_id in MAC specification is directly reused from the index nRA considering that f_id is the index of the PRACH occasion in the frequency domain in MAC specification.

Regarding reinterpretation of f_id for the additional ROs, we expected a change of description about nRA might be considered according to the above-mentioned RAN2 agreement, like the TP suggested in R1-2505844.

However, RAN1 decided to not capture a change related f_id for additional ROs agreed in RAN2. We think the main reason is that if a change is applied to description about nRA for additional ROs, the change would subsequently affect a number of places related to physical signal generation subsequently because nRA is used for physical signal generation as mentioned in R1-2505544.

Observation 1. RAN1 decided to not capture a change related f_id for additional ROs agreed in RAN2.

Respecting RAN1’s feedback, we consider a change for f_id for additional ROs in MAC specification, keeping the essential part of the agreement that there is no need to extend the RA-RNTI space for additional PRACH occasions. The f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs.

To make the change, we intend to consider following two aspects.

Firstly, we understand that the index of the PRACH occasion for additional ROs in the frequency domain is numbered separately from legacy RO. Please note that parameters such as msg1-FrequencyStart and msg1-FDM are separately configured for additional ROs.

Secondly, according to the RAN1 agreements informed by the reply LS from RAN1, UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs). When an additional RO overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid. It is captured in clause 8.1 of TS 38.213. Therefore, only the valid additional ROs are numbered for the index of the PRACH occasion in frequency domain. In other words, if invalid ROs are numbered for the index of the PRACH occasion in frequency domain, 0 ≤ f_id < 8 is not guaranteed. Please not the maximum number of FDM-ed valid ROs is 8. The invalid additional ROs are excluded from the index of the PRACH occasion in frequency domain.

To capture the reinterpretation of f_id for additional ROs with the above-mentioned two aspects in MAC, adding the following note below the paragraph describing RA-RNTI calculation in MAC specification may be considered as shown in the box below.

- NOTE: f_id for the PRACH occasion indicated by addlRACH-Config-Adapt is reinterpreted as the index of the PRACH occasion indicated by addlRACH-Config-Adapt in the frequency domain plus the number of the FDM-ed PRACH occasions which are not indicated by addlRACH-Config-Adapt, where the PRACH occasion indicated by addlRACH-Config-Adapt is valid PRACH occasion as described in clause 8.1 of TS 38.213 [6].

The grey highlighted text is inserted for the first aspect, and the green highlighted text is added for the second aspect.

### 5.1.3 Random Access Preamble transmission … The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as: RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier). NOTE: f_id for the PRACH occasion indicated by addlRACH-Config-Adapt is reinterpreted as the index of the PRACH occasion indicated by addlRACH-Config-Adapt in the frequency domain plus the number of the FDM-ed PRACH occasions which are not indicated by addlRACH-Config-Adapt, where the PRACH occasion indicated by addlRACH-Config-Adapt is valid PRACH occasion as described in clause 8.1 of TS 38.213 [6].

Proposal 1. Consider adding the following note to capture reinterpretation of f_id for additional ROs in MAC.

- NOTE: f_id for the PRACH occasion indicated by addlRACH-Config-Adapt is reinterpreted as the index of the PRACH occasion indicated by addlRACH-Config-Adapt in the frequency domain plus the number of the FDM-ed PRACH occasions which are not indicated by addlRACH-Config-Adapt, where the PRACH occasion indicated by addlRACH-Config-Adapt is valid PRACH occasion as described in clause 8.1 of TS 38.213 [6].

3.	Conclusion

This document discusses the open issue of RA-RNTI for PRACH adaptation.

Observation 1. RAN1 decided to not capture a change related f_id for additional ROs agreed in RAN2.

Proposal 1. Consider adding the following note to capture reinterpretation of f_id for additional ROs in MAC.

- NOTE: f_id for the PRACH occasion indicated by addlRACH-Config-Adapt is reinterpreted as the index of the PRACH occasion indicated by addlRACH-Config-Adapt in the frequency domain plus the number of the FDM-ed PRACH occasions which are not indicated by addlRACH-Config-Adapt, where the PRACH occasion indicated by addlRACH-Config-Adapt is valid PRACH occasion as described in clause 8.1 of TS 38.213 [6].