3GPP TSG-RAN2 Meeting #131bis	R2-2507264

Prague, Czech Republic, Oct. 13th - 17th, 2025

Agenda item:	8.11.2

Source:	Qualcomm Incorporated

Title:	Remaining issue of SBFD

WID/SID:	NR_duplex_evo-Core – Release 19

Document for:	Discussion and Decision

# Introduction

In this contribution, we will discuss some remaining issues of SBFD.

# Discussion

One remaining issue discussed in SBFD MAC is whether further clarification is needed for the parameter of ‘preambleReceivedTargetPower’ in the case that Msg3 is transmitted in non-SBFD symbols and SBFD RACH config option 2 is configured. The preambleReceivedTargetPower indicated to lower layer for Msg3 transmission power depending on the type of symbols (i.e., SBFD or non-SBFD) that the corresponding Msg3 is transmitted on, irrespective of the RO type used in preamble transmission.

In the email discussion, one way is to specify the indication as ‘the preambleReceivedTargetPower (not included in sbfd-RACH-DualConfig)’, and while another way is to indicate the preambleReceivedTargetPower (included in xxx).

In our view, it is good to have some clarifications to differentiate the parameters configured for different RO types to avoid misunderstanding. We believe the simplest way is to clarify the preambleReceivedTargetPower included in the RACH-ConfigCommon instead of using ‘not included in sbfd-RACH-DualConfig’.

Proposal 1: For the case that Msg3 is transmitted in non-SBFD symbols and SBFD RACH config option 2 is configured, it is to clarify the preambleReceivedTargetPower used is the one included in the RACH-ConfigCommon.

Another remaining issue discussed in SBFD RRC is whether can always configure rach-ConfigCommon and sbfd-RACH-DualConfig-r19 in one additionalRACH-Config. It should be noted that it has been agreed that only one RACH configuration option is supported/configured in a cell. To align with the RACH configuration option in the AdditionalRACH-Config-r17 and the configuration under the BWP-UplinkCommon, the conditional code (i.e., ‘NoSingleConfig’) should be used for the sbfd-RACH-DualConfig-r19 under the AdditionalRACH-Config-r17.

Thus, only when the sbfd-RACH-DualConfig-r19 is enabled for the dual RACH configurations and the random access parameters in SBFD symbols are configured by SBFD-RACH-DualConfig-r19 under BWP-UplinkCommon, the sbfd-RACH-DualConfig-r19 can be enabled together with rach-ConfigCommon under the same AdditionalRACH-Config-r17.

Proposal 2: Only when sbfd-RACH-DualConfig-r19 is enabled under BWP-UplinkCommon, the sbfd-RACH-DualConfig-r19 under AdditionalRACH-Config-r17 is configured/enabled.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: For the case that Msg3 is transmitted in non-SBFD symbols and SBFD RACH config option 2 is configured, it is to clarify the preambleReceivedTargetPower used is the one included in the RACH-ConfigCommon.

Proposal 2: Only when sbfd-RACH-DualConfig-r19 is enabled under BWP-UplinkCommon, the sbfd-RACH-DualConfig-r19 under AdditionalRACH-Config-r17 is configured/enabled.

# References