3GPP TSG-RAN2 Meeting #131bis	R2-2507279

# Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item:	8.7.3 (NR_XR_Ph3)

Source:	LG Electronics Inc.

Title: 	Remaining open issues for DSR

Document for:	Discussion and Decision

1.	Introduction

During the discussion on RLC and PDCP open issues [1-2], following open issue is left for DSR.

RLC Open issues Proposal 1: Following open issues are suggested to be discussed based on companies’ contributions: RLC-X01, RLC-V01, RLC-N01, RLC-N02. PDCP Open issues Proposal 2 (H001). Discuss whether to change the definition of “Non-delay-reporting PDCP SDU” so that each of a non-delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is exclusive.

In this document, remaining open issues for DSR are discussed.

2.	Discussion

[RLC-X01] non-delay-reporting RLC data identification in RLC layer

In RAN2#131 meeting [3], it was discussed whether the PDCP layer should also indicate the non-delay reporting PDCP SDU to the RLC layer and whether the RLC layer needs to identify the non-delay reporting RLC SDU. Based on the discussion, it was agreed that RLC layer calculates the delay-reporting data volume for each reporting threshold based on the DSR data indication provided from the PDCP layer, in order to simplify the RLC function and to avoid the miscalculation. Since the DSR data indication includes the delay-reporting PDCP SDU and non-delay-reporting PDCP SDU associated with i:th reporting threshold, the definition of delay-reporting RLC SDU and non-delay-reporting RLC SDU is removed as there is no need to identify the delay-reporting RLC SDU and non-delay-reporting RLC SDU in RLC layer.

[RLC-13] PDCP should indicate to RLC what needs to be included in the data volume calculation in RLC for each threshold [RLC-13] We can remove definition of delay reporting and non-delay reporting data from RLC

However, during the RLC open issue discussion [1], one company has raised an issue that there may be a case that PDCP layer cannot identify the order of buffered data if those data is submitted to the RLC layer, and the identification of non-delay-reporting RLC SDU may be needed in RLC layer as well.

| Company | Issue description |
| --- | --- |
| Xiaomi | A open issue we put on the post email discussion: In last meeting, we have agreed: - [RLC-13] PDCP should indicate to RLC what needs to be included in the data volume calculation in RLC for each threshold - [RLC-13] We can remove definition of delay reporting and non-delay reporting data from RLC If all the RLC SDUs that RLC needs to calculated are indicated from PDCP, then RLC can indeed calculate the data based on PDCP’s indication. Then RLC calculation of the none delay reporting data can be removed. However, is that possible that RLC put some packets which are not know in PDCP before the data of one portion or is there possible that before DSR is triggered, there has already some data before one portion? Then in this way, since PDCP is not aware of data in RLC’s buffer, then I guess the part that RLC calculation of the none delay reporting data still can not be removed. (…omitted) In last F2F meeting offline in India, after checking with LG, Sharp, we are not sure on this. We would like to hear more opinions from other companies. |

In general, the RLC layer would process the RLC SDUs in the submission order from the PDCP layer, so the PDCP layer would have knowledge of transmission order of RLC SDUs as well, i.e., same as the submission order from PDCP layer to RLC layer. In other words, it is not sure on which case the PDCP layer cannot know the buffering order in RLC layer, since the PDCP layer does not discard the PDCP SDUs after submitting to the RLC layer and keeps the order of PDCP SDUs, unless the PDCP SDUs are discarded, e.g., based on discard timer expiry.

On the other hand, if the RLC layer also identifies the delay-reporting RLC SDU based on the buffer status in RLC layer, the delay-reporting data volume calculation in RLC layer would be complicated, since some of non-delay-reporting RLC data is indicated from the PDCP layer in DSR data indication, while other non-delay-reporting RLC data needs to be further identified in the RLC layer.

Therefore, unless there is a real case when the PDCP layer cannot identify the delay-reporting PDCP SDU buffered in RLC layer, it would be simpler to keep the current agreement, i.e., to calculate delay-reporting data volume in RLC layer based on the DSR data indication from PDCP layer.

Proposal 1. [RLC-X01] There is no need to identify the non-delay-reporting RLC SDU in RLC layer, i.e., no spec change is needed.

[PDCP-H001] Definition of non-delay reporting PDCP SDU

In RAN2#131 meeting [3], it was discussed whether to move “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” to the definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU, in order to exclusively define the delay-reporting PDCP SDU and non-delay-reporting PDCP SDU, as PDCP open issue 1 [4].

PDCP-1 (Editorial) Whether the text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” needs to be moved to the definition section for both delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

Based on the discussion, it was agreed to keep the definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU, since it would be complicated to modify the definition with circular reference between the definition part and data volume calculation part and current data volume calculation is not broken, as it excludes the PDCP SDUs associated with lower reporting thresholds.

[PDCP-1]: Keep the current definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

However, during the e-mail discussion on PDCP open issues [2], it was proposed to update the definition of the non-delay-reporting PDCP SDU, in order to associate with a single reporting threshold for the same PDCP SDU.

Huawei’s proposed text Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the DSR-ReportingThreshold is a PDCP SDU that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold and that is not a delay-reporting or non-delay reporting PDCP SDU associated with any of the k:th DSR-ReportingThreshold where k<i. Fujitsu’s proposed text Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is a PDCP SDU that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold but after all delay-reporting PDCP SDUs associated with the i-1:th DSR-ReportingThreshold, and that is not a delay-reporting or non-delay reporting PDCP SDU associated with any of the k:th DSR-ReportingThreshold where k<=i.

However, it should be noted that even though a PDCP SDU is defined as a non-delay-reporting PDCP SDU associated with the single reporting threshold, delay-reporting data volume calculation cannot be updated since one PDCP SDU can be delay-reporting PDCP SDU associated with one reporting threshold and non-delay-reporting PDCP SDU associated with another reporting threshold. For example, assume that the buffer status is as depicted in Figure 1, and the pdu-SetDiscard is configured.

Figure 1.

Then, even though Huawei’s text or Fujitsu’s text is adopted, a PDCP SDU (e.g., SDU3) can be associated with multiple reporting threshold as follows:

- SDU3 is non-delay-reporting PDCP SDU associated with 1st reporting threshold, since SDU3 will be transmitted prior to SDU7, which is delay-reporting PDCP SDU associated with 1st reporting threshold.

- SDU3 is delay-reporting PDCP SDU associated with 2nd reporting threshold as well, based on the PDU set remaining time of SDU3.

Then, there is no benefits to define non-delay-reporting PDCP SDU in exclusive manner, since the PDCP data volume calculation should further exclude the PDCP SDUs which are not considered as delay-reporting PDCP data volume associated with lower reporting thresholds anyway.

Therefore, in order to associate with a single reporting threshold for the same PDCP SDU, and to remove the text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” from the data volume calculation, the definition of delay-reporting PDCP SDU should be further updated to exclude the PDCP SDU which is associated with non-delay-reporting PDCP SDU associated with the lower reporting threshold. An example TP to the correctly associate with a single reporting threshold for the same PDCP SDU is as follows:

Delay-reporting PDCP SDU: if pdu-SetDiscard is not configured, a delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is a PDCP SDU for which the remaining time till discardTimer expiry is less than the i:th DSR-ReportingThreshold and larger than or equal to the i-1:th DSR-ReportingThreshold (if i>1) or larger than zero (if i=1), and that is not a non-delay-reporting PDCP SDU associated with any of the k:th DSR-ReportingThreshold where k < i. If pdu-SetDiscard is configured, a delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is a PDCP SDU belonging to a PDU Set of which the PDU Set remaining time is less than the i:th DSR-ReportingThreshold and larger than or equal to the i-1:th DSR-ReportingThreshold (if i>1) or larger than zero (if i=1), and that is not a non-delay-reporting PDCP SDU associated with any of the k:th DSR-ReportingThreshold where k < i. Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is a PDCP SDU that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold but after all delay-reporting PDCP SDUs associated with the i-1:th DSR-ReportingThreshold (if i>1) and that is not a delay-reporting PDCP SDU associated with any of k:th DSR-ReportingThreshold where k ≤ i.

Then, there will be circular reference between the definition of delay-reporting PDCP SDU and the definition of non-delay PDCP SDU, which is not desirable. In detail, delay-reporting PDCP SDUs are defined based on the definition of non-delay-reporting PDCP SDU, while the non-delay-reporting PDCP SDUs are defined based on the definition of delay-reporting PDCP SDU.

Therefore, given that the XR WI is already completed and current data volume calculation is not broken, it would be safer to keep the current text in PDCP running CR, i.e., keep the current definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

Proposal 2. [PDCP-H001] Keep the current definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

3.	Conclusion

In this contribution, it is discussed for the open issues for DSR. This document includes following proposals.

Proposal 1. [RLC-X01] There is no need to identify the non-delay-reporting RLC SDU in RLC layer, i.e., no spec change is needed.

Proposal 2. [PDCP-H001] Keep the current definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

4.	Reference

[1] R2-2507017 List of RLC open issues for R19 XR

[2] R2-2507130 PDCP open issues for XR

[3] R2-2506205 Report from session on R19 XR and LTE Broadcast

[4] R2-2505439 Summary of [POST130][507][XR] PDCP running CR and open issues (LGE)