3GPP TSG-RAN WG2 Meeting #131bis                                 R2-2507282

Prague, Czech Republis, Oct. 13th –Oct. 17th, 2025

Agenda Item	: 7.0.2.16 (NR_XR_enh)

Source	: LG Electronics Inc.

Title	: Discussion on DSR triggering for RLC segment

Document for	: Discussion and Decision

Introduction

In RAN2#131 meeting, it was proposed to clarify that the RLC segments triggers DSR, but it was postponed due to different understandings on specification.

In this contribution, we show our views on the interpretation of current wording and whether the RLC segments triggers DSR or not.

Discussion

According to DSR procedure in TS 38.321 [1], DSR is triggered if the smallest PDCP discard timer value is less than RemainingTimeThreshold among PDCP SDUs that has not been transmitted in any MAC PDU.

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel.

In RAN2#131bis meeting, it was suggested to clarify that DSR should be triggered if there is any RLC segment of the PDCP SDU has not been transmitted in a MAC PDU, when the PDCP SDU has running PDCP discard timer value less than RemainingTimeThreshold [2]. However, it was postponed due to different interpretations on current text as follows:

- Interpretation 1: The current text “the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU” indicates the PDCP SDUs that have not been completely transmitted in MAC PDU. In other words, if only a subset of PDCP SDU has been transmitted in RLC segment, the remaining data of the PDCP SDU may trigger DSR based on the PDCP discard timer value.

- Interpretation 2: The current text “the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU” excludes the PDCP SDUs that have been partially transmitted in MAC PDU. In other words, if an RLC segment of the PDCP SDU has been transmitted in any MAC PDU, the PDCP SDU does not trigger DSR.

Meanwhile, if the data unit of the upper layer is mentioned in the other part of the MAC specification, it was clearly specified whether the SDU is whole SDU or partially transmitted SDU as follows:

The UE shall also follow the rules below during the scheduling procedures above: - the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity; - if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible; - the UE should maximise the transmission of data; - if the MAC entity is given a UL grant size that is equal to or larger than 8 bytes (when eLCID is not used) or 10 bytes (when eLCID is used) while having data available and allowed (according to clause 5.4.3.1) for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

However, in the current text on DSR procedure, there is no clear text on whether the whole PDCP SDU has been transmitted in MAC PDU or any segment of the PDCP SDU has been transmitted in MAC PDU. Thus, both interpretations are possible for the existing specification on DSR triggering procedure for now and further clarification is needed based on the reasonable behaviour of DSR procedure.

Observation 1. Both interpretations are possible based on the current text of DSR, since there is no clear text on whether the PDCP SDU is a whole SDU or not.

In our understanding, DSR is defined to report the amount of delay-critical data and to request an uplink grant for the delay-critical data. In this sense, even though only the subset of a PDCP SDU is already transmitted in RLC segment, (i.e., when the PDCP SDU is partially transmitted), it would be better to trigger the DSR if the remaining RLC segment becomes delay-critical data and has not been transmitted in MAC PDU. Then, based on the amount of delay-critical data including this remaining RLC segment, the network would be able allocate an uplink grant to transmit remaining data of the PDCP SDU.

Therefore, DSR triggering procedure should be clarified based on the Interpretation 1 above, i.e., if only a subset of PDCP SDU has been transmitted in RLC segment, the remaining data of the PDCP SDU may trigger DSR based on the PDCP discard timer value.

Proposal 1. RAN2 confirms that in Rel-18 remaining RLC segment of the PDCP SDU may trigger DSR, if the remaining PDCP discard timer value is less than RemainingTimeThreshold.

If proposal 1 is agreeable, it should be made clear in the specification.

In [1], Vivo suggested to change the text as “the PDCP SDUs buffered for the logical channel that have not been completely transmitted in any MAC PDU”. We agree with this direction, but the text may still be misleading due to the combination of “not”, “completely”, and “any”. Thus, we propose to change “in any” to “by” for further clarity.

- “the PDCP SDUs buffered for the logical channel that have not been completely transmitted by MAC PDU”

It should be noted that “PDCP SDUs that have not been transmitted in any MAC PDU” is also used in other part of DSR procedure as follows:

- In clause 5.4.9 of TS 38.321, for DSR cancellation, PDCP SDU associated with a DSR is determined among the PDCP SDUs which have not been transmitted in any MAC PDU.

- In the format of DSR MAC CE, the remaining time field indicates the shortest remaining time value among PDCP SDUs which have not been transmitted an any MAC PDU

Therefore, the same clarification should be applied to the other parts of DSR as well, in order to report the delay information of remaining RLC segment in DSR MAC CE if PDCP SDU has been partially transmitted by MAC PDU. In detail,

- if a PDCP SDU has not been completely transmitted by MAC PDU, DSR should not be cancelled and the remaining data of the PDCP SDU should be reported by DSR MAC CE.

- if a PDCP SDU has not been completely transmitted at the time of the DSR MAC CE generation, the shortest remaining time is determined by a discard timer value of the PDCP SDU in order to report the delay information of remaining RLC segment for the PDCP SDU.

Proposal 2. Clarify “have not been transmitted in any MAC PDU” in the MAC spec to “have not been completely transmitted by MAC PDU,” in order to report the delay information of RLC segment if a PDCP SDU is partially transmitted. Use TP in Annex A as a baseline.

Conclusion

In this contribution, we show our views on whether the RLC segments should be able to trigger DSR and whether any clarification in the current specification is needed, including following observation:

Observation 1. Both interpretations are possible based on the current text of DSR, since there is no clear text on whether the PDCP SDU is a whole SDU or not.

Based on the above observation, followings are proposed:

Proposal 1. RAN2 confirms that in Rel-18 remaining RLC segment of the PDCP SDU may trigger DSR, if the remaining PDCP discard timer value is less than RemainingTimeThreshold.

Proposal 2. Clarify “have not been transmitted in any MAC PDU” in the MAC spec to “have not been completely transmitted by MAC PDU,” in order to report the delay information of RLC segment if a PDCP SDU is partially transmitted. Use TP in Annex A as a baseline.

Reference

[1] TS 38.321 v.18.6.0

[2] R2-2505408 Correction on DSR triggering vivo

- Annex A. 38.321 TP for proposal 2

### 5.4.9 Delay status reporting The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of LCGs. This delay status for an LCG includes remaining time, which is the smallest remaining value of the running PDCP discardTimers among PDCP SDUs that are buffered for the LCG but have not been completely transmitted by MAC PDU as specified in clause 7.3 in TS 38.323 [4], and the total amount of delay-critical UL data for the LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively. RRC controls the DSR procedure by configuring the following parameter: - remainingTimeThreshold (per LCG): the threshold on remaining time for triggering a DSR for a logical channel within an LCG. If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been completely transmitted by MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel. If there is at least one DSR pending, the MAC entity shall: 1> if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the DSR MAC CE plus its subheader as a result of logical channel prioritization: 2> instruct the Multiplexing and Assembly procedure to generate the DSR MAC CE as specified in clause 6.1.3.72. 1> else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR: 2> trigger a Scheduling Request. NOTE 1: The availability of UL-SCH resources for the transmission of the DSR MAC CE follows the same critieria specified in clause 5.4.5. A PDCP SDU is considered to be associated with a DSR if it has not been completely transmitted by MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR. A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR. NOTE 2: It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader. (…omitted) 6.1.3.72 Delay Status Report MAC CE The Delay Status Report (DSR) MAC CE is identified by MAC subheader with an eLCID as specified in Table 6.2.1-2b. The fields in the DSR MAC CE are defined as follows: - LCGi: This field indicates the presence of delay information (i.e. the Remaining Time and Buffer Size fields) for the LCG i. The LCGi field set to 1 indicates that the delay information for the LCG i is reported. The LCGi field set to 0 indicates that the delay information for the LCG i is not reported; - Remaining Time: This field indicates the shortest remaining value of running PDCP discardTimer (described in clause 7.3 in TS 38.323 [4]) among all PDCP SDUs that are buffered for an LCG but have not been completely transmitted by MAC PDU, at the time of the first symbol of the first PUSCH transmission that includes this DSR MAC CE. The length of this field is 6 bits. This field is present only if the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the value r in this field indicates a remaining time within the range of (r, r + 1] msec; - BT: This field is present only if the corresponding LCG is configured with additionalBS-TableAllowed and the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the BT field set to 1 indicates that the buffer sizes specified in Table 6.1.3.1-3 are used to set the value of the Buffer Size field, while the BT field set to 0 indicates that the buffer sizes specified in Table 6.1.3.1-2 are used instead; - Buffer Size: The Buffer Size field indicates the total amount of delay-critical UL data for an LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively, after the MAC PDU has been built. If the corresponding LCG is configured with additionalBS-TableAllowed and the amount of delay-critical UL data for an LCG is within the buffer sizes specified in Table 6.1.3.1-3, the MAC entity shall use the buffer sizes specified in Table 6.1.3.1-3 to set the value of this field; otherwise, the MAC entity shall use Table 6.1.3.1-2 instead. This field is indicated in number of bytes. The length of this field is 8 bits; - R: Reserved bit, set to 0. The DSR MAC CE shall include delay information of all LCGs which have pending DSRs when the MAC PDU containing this DSR MAC CE is to be built. The Remaining Time, the BT, and the Buffer Size fields for an LCG shall be reported in two consecutive octets. These three fields for different LCGs shall be included in a DSR MAC CE in ascending order based on the LCGi. Figure 6.1.3.72-1: DSR MAC CE