3GPP TSG-RAN WG2 Meeting #131bis								R2-2507299

Prague, Czech Republic, Oct. 13th-17th

Agenda item:		8.7.3

Title: 	XR RLC option Issues

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

# Introduction

In the <R2-250xxxx_List of RLC open issues for R19 XR_v07_Rapp>, the following proposals has been made by rapporteur[1]:

Proposal 1: Following open issues are suggested to be discussed based on companies’ contributions: RLC-X01, RLC-V01, RLC-N01, RLC-N02.

In this contribution, we will discuss these open issues and give our proposals.

# Discussion

After RAN2#131 meeting, the following option issue is proposed[1]:

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| vivo | When t-RxDiscard expires, the RLC entity stops waiting to receive the obsolete RLC SDUs, and the corresponding RLC SDUs will be positively acknowledged in the STATUS report. However, the PDCP t-Reordering may be still running and waiting for receiving the outdated SDUs from the RLC layer, this can lead to unnecessary delays in packet delivery. Therefore, the receiving RLC entity should indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: RLC-V01 |

For this issue, we understand that if the receiving RLC entity indicates the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires, the SN should be the PDCP SN. But if the obsolete RLC SDUs or segment including the PDCP PDU header have not been received, the receiving RLC entity cannot known the PDCP SN of the obsolete RLC SDUs. Considering that the PDCP PDU header is not ciphered, the RLC entity can decode the PDCP header and obtain the PDCP SN when the RLC SDU segment including the PDCP header have been received.

Observation 1a:  The PDCP SN of obsolete RLC SDUs can be obtained in some cases.

If the RLC entity can obtain the PDCP SN of the obsolete RLC SDUs from the received RLC SDU segment including the PDCP header, indicating the PDCP SN of the discarded RLC SDUs to the receiving PDCP entity is beneficial for less latency, e.g. same effect as the PDCP SN gap report, but the transmitting PDCP entity cannot know the obsolete RLC SDUs info triggered by t-RxDiscard expiring, and cannot trigger PDCP SN gap report.

Observation 1b:  If available, indicating the PDCP SN of the discarded RLC SDUs triggered by t-RxDiscard expiring to the receiving PDCP entity is beneficial.

Proposal 1(RLC-V01): If available, the receiving RLC entity indicates the PDCP SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires.

After RAN2#131 meeting, the following option issue is proposed[1]:

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| Nokia | [N001] RX_Next in clause 7.1 The definition of RX_Next update should reflect t-RxDiscard expiry (to be aligned with the NOTE in 5.3.4) and text - which value of the SN is pointed - should be aligned with the procedural text in 5.2.3.2.x. Our suggestion is: a) RX_Next – Receive state variable This state variable holds the value of the SN following the last in-sequence completely received RLC SDU of the first RLC SDU not delivered to the upper layers, but still waited for, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires. [Rapp]: It is true, and we need to update it. Your poropsal is one way, which may miss some information for legacy case. Another way is to capture like: if t-RxDiscard is configured, xxx. Otherwise, xxxx. | Issue Type: Not essential but important How to address it: based on companies’ contribution Issue Number: RLC-N01 |

We understand this issue is important for RX window advance(e.g. to avoid the RX window stalling), and should be captured carefully. But for the case that t-RxDiscard is not configured or does not expire, the legacy procedure is accurate and should be kept.

When t-RxDiscard is configured or expires, the receiving RLC entity will not wait for the obsolete RLC SDUs, it has already captured by “the first RLC SDU not delivered to the upper layers, but still waited for”. The only thing need to do is to update the RX_Next updating condition that  “It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires”.

Proposal 2(RLC-N01): keep the description for RX_Next – Receive state variable, except to update the RX_Next updating condition that  “It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires”.

After RAN2#131 meeting, the following option issue is proposed[1]:

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| Nokia | [N002] Below NOTE in clause 5.3.3.3 NOTE: When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit. First, we should clarify that the NOTE is for the case where stopReTxDiscadedSDU is configured because stopping/resetting based on discard indication is limited to configuration of stopReTxDiscardedSDU. Second, it seems RAN2 has not carefully discussed ‘NACK’ part in the NOTE. To us, it seems not reasonable to stop polling even when there is an unACKed RLC SDU still under transmission, i.e., waiting for ACK/NACK after retransmission or before discard. Our suggestion is: NOTE: If stopReTxDiscardedSDU is configured, Wwhen all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit. [Rapp]: This issue was discussed in RAN2#130 based on Apple’s proposal. The agreement includes the ACK/NACK/Discard cases. I understand the motivation in Apple’s contribution is that in the case where a NACK has just been received, there is no need for polling since the status is already known, and the retransmission can be performed directly. While the current Note in the specification also states “may stop and reset,” so I think this should not be a major issue. But considering it has not been extensively discussed during RAN2#130, let’s discuss it. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: RLC-N02 |

For this issue, we have the same feeling as rapporteur that t-PollRetransmit should be stopped when all RLC SDUs are already positively or negatively acknowledged, because is used for triggering POLL and RLC STATUS report. When the transmitting RLC entity knows the status of all RLC SDUs,  POLL and RLC STATUS report need not be triggered.

But for “indicated as discarded from upper layer” case, there are the following sub-cases:

- sub-cases 1: when stopReTxDiscardedSDU is configured and indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU and if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers, the RLC STATUS report is still necessary for TX window advance(e.g. to avoid TX window stalling)

- sub-cases 1: when stopReTxDiscardedSDU is not configured and indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU and if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers, the RLC STATUS report is still necessary to trigger the RLC SDU or RLC SDU segment re-transmission or TX window advance.

Observation 2: Even if indicated from upper layer (e.g. PDCP) to discard a RLC SDU, POLL and RLC STATUS report is still necessary when the RLC SDU or the RLC SDU segment(s) has been submitted to lower layers.

Proposal 3(RLC-N02): Update the NOTE in clause 5.3.3.3 as follows:

NOTE:	When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP) and neither the RLC SDU nor a segment thereof has been submitted to the lower layers, the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

# Conclusion

The following observation and proposal are given based on the above analysis:

Observation 1a:  The PDCP SN of obsolete RLC SDUs can be obtained in some case.

Observation 1b:  If available, indicating the PDCP SN of the discarded RLC SDUs triggered by t-RxDiscard expiring to the receiving PDCP entity is beneficial.

Proposal 1(RLC-V01): If available, the receiving RLC entity indicates the PDCP SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires.

Proposal 2(RLC-N01): keep the description for RX_Next – Receive state variable, except to update the RX_Next updating condition that  “It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires”.

Observation 2: Even indicated from upper layer (e.g. PDCP) to discard a RLC SDU, POLL and RLC STATUS report is still necessary when the RLC SDU or the RLC SDU segment(s) has been submitted to lower layers.

Proposal 3(RLC-N02): Update the NOTE in clause 5.3.3.3 as follows:

NOTE:	When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP) and neither the RLC SDU nor a segment thereof has been submitted to the lower layers, the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

# References

- R2-250xxxx_List of RLC open issues for R19 XR_v07_Rapp