3GPP TSG-RAN WG2 Meeting #131bis									R2-2507300

Prague, Czech Republic, Oct. 13th-17th

Agenda item:		8.7.2

Title: 	XR RRC Corrections

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

# Introduction

In this contribution, we will discuss RRC open issues and give our proposals.

# Discussion

During the  XR RRC review stage, the following RIL is proposed[1]:

## N091 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. N091; XR; 1; UAI for measurement gap cancellation preference; R2-25xxxxx; Chunli WU (Nokia); V04; ToDo. [Description]: Curren conditions for UAI triggering allow the UE to send the UAI if it has not sent before or otherwise when the prohibit timer is not running. Considering now that we agreed single rohibit timer while the the UAI can include preference for mutliple measurement gaps, it makes sense to allow UAI to be sent for a newly configured measurement gao without delay even if UAI has been sent for other MG before. [Proposed Change]: Revise the first condition to allow such case or add the condition to allow UAI for a newly configured measuremeng gap with preference becomes available: 1> if configured to provide its preference for gap occasion cancellation ratio: 2> if the UE did not transmit a UEAssistanceInformation message with gapOccasionCancelRatio since it was configured to do so and if the UE has the preference for gap occasion cancellation ratio for at least one measurement gap configuration; or 2> if the preference for gap occasion cancellation ratio becomes available for at least one measurement gap configuration for which preference for gap occasion cancellation ratio has not been included in any UEAssistanceInformation message with gapOccasionCancelRatio previously transmitted by the UE; or 2> if the UE's preference for gap occasion cancellation ratio has changed for at least one measurement gap configuration since the last transmission of the UEAssistanceInformation message with gapOccasionCancelRatio and T346o is not running: 3> start the timer T346o with the timer's value set to gapOccasionCancelRatioProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UE's preference for gap occasion cancellation ratio.. [Comments]:[Rapp] Since this issue has not been discussed before, it could be discussed by the discussion paper submitted to the next R2 meeting.

For this issue, the following agreement has been agreed in RAN2#131meeting:

Agreements on measurement gap skipping and RRC open issues 2. The prohibit timer for the preference for gap occasion cancellation ratio is (14 out of 14) - started when UAI carrying the field gapOccasionCancelRatio is transmitted - stopped when releasing the GapOccasionPreferenceReportConfig when - connection reestablishment/resume procedure is initiated or cell reselection happens during reestablishment - GapOccasionPreferenceReportConfig is set to release ... - For UAI for reporting preference for gap cancellation ratio, when multiple gap configurations are provided, a single timer is maintained for all the gap configurations. (9 out of 14) ...

We think that “a single timer is maintained for all the gap configurations” means that when the prohibit timer is running, any new gap cancellation ratio preference cannot be reported. Otherwise, the timer is started by reporting gap cancellation ratio preference for measurement GAP configuration(e.g. measurement GAP configuration A), and will be restarted by reporting gap cancellation ratio preference for another measurement GAP configuration(e.g. measurement GAP configuration B). Upon restarting the prohibit timer, gap cancellation ratio preference for measurement GAP configuration A can be reported again, the prohibit timer will not take effect in multiple measurement GAP configuration cases.

Observation 1: If the prohibit timer can be restarted by reporting gap cancellation ratio preference for another measurement GAP configuration, the prohibit timer will not take effect in multiple measurement GAP configuration cases.

If we considering the reporting delay and wish that reporting the gap cancellation ratio preference for one measurement GAP configuration does not affect reporting the gap cancellation ratio preference for another measurement GAP configuration, RAN2 should revert the previous agreement and the prohibit timer should be started for each measurement GAP configuration.

Considering that UE will not report the gap cancellation ratio preference, e.g. there is window for the  gap cancellation ratio, we prefer that any gap cancellation ratio preference should not be reported if the prohibit timer is running.

Proposal 1: RAN2 follow the previous agreement of a single timer is maintained for all the gap configurations. And any gap cancellation ratio preference should not be reported if the prohibit timer is running.

# Conclusion

The following observation and proposal are given based on the above analysis:

Observation 1: If the prohibit timer can be restarted by reporting gap cancellation ratio preference for another measurement GAP configuration, the prohibit timer will not take effect in multiple measurement GAP configuration cases.

Proposal 1: RAN2 follow the previous agreement of a single timer is maintained for all the gap configurations. And any gap cancellation ratio preference should not be reported if the prohibit timer is running.

# References

- XR RRC comment file_v09_Rapp