3GPP TSG-RAN WG2 Meeting #131bis					                      R2-2507301

Prague, Czech Republic, Oct. 13th-17th

Agenda item:		8.7.3

Title: 	XR Scheduling option Issues

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

# Introduction

In the <R2-250xxxx_List of RLC open issues for R19 XR_v07_Rapp>, the following proposals has been made by rapporteur[1]:

Proposal 1: Following open issues are suggested to be discussed based on companies’ contributions: RLC-X01, RLC-V01, RLC-N01, RLC-N02.

In this contribution, we will discuss these open issues and give our proposals.

# Discussion

After RAN2#131 meeting, the following option issue is proposed[1]:

| Huawei, HiSilicon | D.2 | The following paragraph might be a bit misleading For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize the HARQ Process ID with the highest priority, where the priority of HARQ process is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. In this selection, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the highest priority that can be applied or has been applied for it in the LCP procedure for the MAC PDU (see clause 5.4.3.1.3). ……Omitted…… it seems to imply that additionalPriority is always considered as long as a logical channel is configured with priorityAdjustmentThreshold, which is not correct as the additionalPriority is only applied if the logical channel has priority adjustable PDCP SDU that can be multiplexed or has been multiplexed in the LCP procedure for the MAC PDU. We think the following change could be made ……Omitted…… In this determination, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the additionalpriority if there is a priority adjustable PDCP SDU that can be multiplexed or has been multiplexed from it in the LCP procedure for the MAC PDU (see clause 5.4.3.1.3). ……Omitted…… [Rapp] It is not clear how you conclude from the current text that “additionalPriority is always considered”. The “highest priority” in the current text simply means that “it is the default priority if additional priority is not used; otherwise, it is the additional priority.” [Nokia] Current text looks ok which says the highest applied or to be applied priority which could be the legacy one or additional one depending on whether the additional one is used in any of the LCP steps. [HW]Just by the wording itself “the highest priority that can be applied or has been applied for it in the LCP procedure for the MAC PDU”, if additionalPriority is configured, then the highest priority that can be applied is always the additionalPriority. That is the not the intended behavior, right? [Rapp] Even if additionalPriority is configured, it can’t be applied if the priority adjustment criteria are not met. I tend to think this is how most people would interpret it. But if some companies still have concerns or think it can lead to a different interpretation, I am open to rewording it. Additional comments are welcome. |
| --- | --- | --- |

And the following proposal has been made by rapporteur:

Proposal 3.  Discuss more on Issue D.2.

For this issue, the key ambiguity is “that can be applied”. e.g. if additionalPriority is configured, additionalPriority will be used for the LCH in the case that at least one priority adjustable PDCP SDU is buffered for a logical channel and additionalPriority will not be used for the LCH in other cases. Per our understanding, it can say that additionalPriority can be applied for the LCH, but we think this is not the intention in the current text. So,  “that can be applied” in the current text may lead some misunderstanding.

Observation 1: “that can be applied” in the current text may lead some misunderstanding.

Considering that the following text is used to capture the condition to use additionalPriority, similar text can be used  for other case to capture the condition to use additionalPriority.

2> if at least one priority adjustable PDCP SDU is buffered for a logical channel: 3> apply additionalPriority of this logical channel;

For example, “In this selection, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the highest priority that can be applied or has been applied for it in the LCP procedure for the MAC PDU (see clause 5.4.3.1.3). ” can be changed to “In this selection, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the additionalpriority if there is at least one priority adjustable PDCP SDU that will be multiplexed or has been multiplexed from it in the LCP procedure for the MAC PDU (see clause 5.4.3.1.3). ”

Proposal 1 Reword the text for Issue D.2. as follows:

In this selection, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the additionalpriority if there is at least one priority adjustable PDCP SDU that will be multiplexed or has been multiplexed from it in the LCP procedure for the MAC PDU (see clause 5.4.3.1.3).

# Conclusion

The following observation and proposal are given based on the above analysis:

Observation 1: “that can be applied” in the current text may lead some misunderstanding.

Proposal 1 Reword the text for Issue D.2. as follows:

In this selection, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the additionalpriority if there is at least one priority adjustable PDCP SDU that will be multiplexed or has been multiplexed from it in the LCP procedure for the MAC PDU (see clause 5.4.3.1.3).

# References

- R2-250xxxx_List of RLC open issues for R19 XR_v07_Rapp