3GPP TSG RAN WG2 Meeting #131bis	 R2-2507307

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda Item:	10.2

Source: 	ITL

Title: 	General consideration on 6GR

Document for:	Discussion and decision

Introduction

In RAN#108, a new Study Item (SI) on 6G Radio (6GR) was approved [1]. This contribution provides a general perspective on the key aspects of 6GR design and outlines areas for RAN2 studies.

Discussion

The 6GR design should evolve beyond incremental improvements of 5G, targeting support for new services, fulfillment of IMT-2030 requirements, deployment practicality, and adaptability to diverse device types. The following aspects are identified as essential directions for RAN2 studies.

For new services, 6GR is expected to natively support service that expand well beyond the current eMBB and URLLC. AI is recognized in ITU-R as a foundational pillar for 6G. RAN must provide the capabilities needed by AI applications (e.g., robots, vehicles, drones), ensuring ultra-low latency and reliable inference. Sensing is one of the six IMT-2030 scenarios, supporting use cases such as low-altitude economy and traffic monitoring. 6GR should ensure efficient sensing operation, with minimal overhead and interference mitigation, including study of dedicated sensing signals. Extending eMBB, immersive services aim for uninterrupted interactive experiences across physical and virtual spaces. This requires higher throughput, lower latency, and improved mobility robustness.

For deployment requirements, commercial viability depends on cost-effective and resilient deployment, 6GR should support converged deployments where one base station can handle multiple device classes (e.g., LPWA), simplifying operation and upgrades. Persistent problem cases such as high-speed mobility and dense urban deployments must be addressed with dedicated enhancements.

Proposal 1: RAN2 is invited to study techniques for 6GR with consideration of the following aspects:

- New service (e.g., AI, sensing, immersive communication)

- Deployment requirements

New services

As part of new services considerations, the SID defines the following objectives:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

As stated in the SID, the scope of this SI is limited to the agreed existing and new services. However, no requirements for such services have been agreed or captured in the TR 38.914 yet. At the current stage, companies are expected to provide their views on existing and new services. These views include considerations for low-cost devices (e.g., IoT, LPWA, RedCap), XR, voice, FWA, positioning, broadcast/multicast, sidelink, unlicensed spectrum, HRLLC, AI, sensing, and others.

Sensing

- Digital Twin Services

Future 6G systems are expected to provide real-time digital replicas of the physical world. By combining RAN sensing with AI processing, local digital twins (e.g., of roads, buildings, and traffic participants) can be generated and then aggregated into global digital twins. This capability is essential for applications such as smart cities, intelligent transportation, and smart homes.

Compared with traditional sensing methods, RAN-based sensing enables collaboration between network and devices, eliminating blind spots and improving robustness. To realize this vision, the 6G RAN should support:

- Localization accuracy from sub-meter to meter-level.

- Spatial resolution at sub-meter level.

- Detection probability of 95–99% with false alarm probability of 1–5%.

- Capability to identify both static and dynamic targets.

- Mechanisms for managing sensing measurement data.

- Road Digitalization

Digitalization of roads requires accurate detection and tracking of vehicles and pedestrians in real time. For autonomous driving and traffic management, 6G RAN must support:

- Localization accuracy of ~0.5 m for vehicles and ~1 m for pedestrians.

- Detection probability exceeding 95–99% with false alarm probability below 2–5%.

- Support for a new metric, sensing target density, with requirements up to 750 vehicles per 1000 m × 24 m road segment.

AI

According to ITU-R recommendations and 3GPP TR 22.870, AI is recognized as a foundational pillar for 6G systems, expected both to enhance network performance (AI for 6G) and to enable AI-based services (6G for AI). While AI for 6G focuses on operational improvements of the network, supporting AI services in 6G provides new opportunities for operators and application providers.

In TR 22.870, new use cases illustrate the requirements for AI-enabled 6G:

- Collaborative AI Agents (Sec. 6.6–6.8): AI entities can represent devices, persons, drones, or vehicles, and may reside in either the UE or the network. Large-scale hosting of AI agents and support for agent-to-agent communication are foreseen.

- Collaborative Robots with Digital Twin (Sec. 7.13): Robots act as UEs and interact with DT applications over the 6G system. This requires tight coupling of communication and sensing.

These use cases highlight the need for 6G RAN to process communication, sensing, and AI functions simultaneously, enabling deep integration across these domains.

Immersive communication

6G is expected to enable highly immersive services that integrate augmented, virtual, and mixed reality environments, as well as AI-driven applications and automated driving. Unlike traditional eMBB use cases, these services combine diverse traffic types (video, audio, haptic, control, and application data), each imposing distinct QoS requirements. The 3GPP TR 22.870 and related studies provide initial service categories and technical targets.

- Immersive Communication:

- Services such as holographic communication and XR gaming require extreme downlink capacity.

- XR headsets with wide field-of-view demand ultra-high resolution (up to 16K) to avoid visual artifacts (screen-door effect).

- This translates to DL rates up to 500 Mbps with ~10 ms latency and 99% reliability.

- Network-assisted AI Inference for Embodied AI:

- Robots, autonomous vehicles, and drones may offload real-time video streams to the network for AI model inference.

- Requirements scale with video resolution and number of cameras, from modest 20 Mbps up to 100 Mbps uplink, with ~15 ms latency and 99% reliability.

- Operator-assisted Automated Driving:

- Robo-taxis and driverless fleets require continuous video uploading for operator or AI monitoring.

- Depending on camera configurations, 20–100 Mbps uplink, 15 ms latency, and 99% reliability are needed.

Implications for 6GR new services

- High Capacity: Both UL and DL capacity targets must exceed current NR capabilities, requiring advanced spectrum usage, MIMO, and coding.

- Latency and Reliability: Tight targets (10–15 ms and 99% reliability) will demand RAN scheduling innovations and resilient link design.

- Service Diversity: The requirements vary significantly across services; a flexible and scalable framework is essential to support such heterogeneity.

- Joint Optimization: UL-heavy and DL-heavy use cases must be jointly supported in a unified RAN design.

Unified Data Management

In 5G systems today, different categories of data—such as RAN measurements, SON/MDT information, AI-related data, and sensing results—are collected and processed through independent mechanisms. This fragmented handling often leads to duplication, inconsistency, and inefficiency, making it challenging to deliver integrated data services to applications.

With the introduction of new 6G services (e.g., AI Agents, Digital Twin, XR, advanced sensing), the volume and diversity of data within the RAN will significantly increase. To support these services effectively, it becomes essential to design RAN-level procedures and protocols that allow integrated collection, management, and exposure of diverse data types.

From the RAN2 perspective, this implies the need to study:

- New signaling mechanisms to support unified reporting of measurement, AI-related, and sensing data.

- Protocol enhancements for joint collection, coordination, and distribution of multi-domain data.

Observation 1: As new 6G services emerge, the amount and diversity of data in the system will increase substantially. Without unified handling, this may lead to inefficiency and limit the ability of the RAN to support advanced services

Proposal 2: Considering the introduction of new services in 6GR, RAN2 is invited to study

- New signaling mechanisms to support unified reporting of measurement, AI-related, and sensing data

- Protocol enhancements for joint collection, coordination, and distribution of multi-domain data

Deployment scenarios

For 6GR studies, it is essential to establish a common understanding of deployment scenarios to study RAN performance requirements and features. In NR, deployment scenarios (e.g., indoor hotspot, urban macro, dense urban, rural) were first captured at a high level and later refined in dedicated TRs.

TR 38.914 already contains descriptions of candidate deployment scenarios for 6G, including indoor hotspot, dense urban, urban macro, sub-urban macro, rural, extreme long-distance coverage, air-to-ground, and non-terrestrial networks. These scenarios include attributes such as carrier frequency, aggregated system bandwidth, inter-site distance, BS/UE antenna elements, user distribution and mobility, and service profiles.

- Indoor Hotspot (open office floor, ~120 m × 50 m; ISD example 20 m @ ~30 GHz)

- Dense Urban (macro with/without micro; continuous hex grid for macro, random micro drops)

- Rural (wide-area coverage; ISD examples 1.7/5/7.5 km)

- Urban Macro (ubiquitous coverage; macro only or macro+micro)

- Sub-Urban Macro (continuous coverage; energy saving & positioning in scope)

- High-Speed

- Extreme Long-Distance / Low-Density (very large macro cells, basic data/voice)

- Urban Coverage for Massive Connection

- Air-to-Ground (A2G) (upward macro towards aircraft; relay on aircraft)

- Non-Terrestrial Network (NTN) (orbit/payload types to be captured)

- Urban Grid

- Highway Scenario

Observation 2: TR 38.914 provides a useful baseline set of scenarios covering both terrestrial and non-terrestrial deployments

Observation 3: A subset of carrier frequencies and bandwidth configurations will be needed for IMT-2030 self-evaluation and 3GPP internal studies, while other configurations may be optionally used for specific feature evaluations

Proposal 3: RAN2 is invited to start initial discussion on 6GR deployment scenarios based on TR 38.914, with focus on:

- Confirming the baseline set of scenarios (indoor hotspot, dense urban, urban macro/sub-urban macro, rural, extreme long-distance, A2G, NTN, etc.)

- Identifying attributes relevant for RAN2 procedure studies (e.g., signaling overhead, mobility, latency constraints)

Conclusion

In this contribution, we discussed general aspects for 6GR. According to discussion in section 2, we have the following observations and proposals:

Proposal 1: RAN2 is invited to study techniques for 6GR with consideration of the following aspects:

- New service (e.g., AI, sensing, immersive communication)

- Deployment requirements

Observation 1: As new 6G services emerge, the amount and diversity of data in the system will increase substantially. Without unified handling, this may lead to inefficiency and limit the ability of the RAN to support advanced services

Proposal 2: Considering the introduction of new services in 6GR, RAN2 is invited to study

- New signaling mechanisms to support unified reporting of measurement, AI-related, and sensing data

- Protocol enhancements for joint collection, coordination, and distribution of multi-domain data

Observation 2: TR 38.914 provides a useful baseline set of scenarios covering both terrestrial and non-terrestrial deployments

Observation 3: A subset of carrier frequencies and bandwidth configurations will be needed for IMT-2030 self-evaluation and 3GPP internal studies, while other configurations may be optionally used for specific feature evaluations

Proposal 3: RAN2 is invited to start initial discussion on 6GR deployment scenarios based on TR 38.914, with focus on:

- Confirming the baseline set of scenarios (indoor hotspot, dense urban, urban macro/sub-urban macro, rural, extreme long-distance, A2G, NTN, etc.)

- Identifying attributes relevant for RAN2 procedure studies (e.g., signaling overhead, mobility, latency constraints)

Reference

RP-252912 revised SID: Study on 6G radio