3GPP TSG-RAN WG2 Meeting #131-Bis	R2-2507309

Prague, Czech, 13 – 17 October, 2025

Agenda item:	8.7.3

Source:	InterDigital

Title:	Remaining MAC open issues

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This TDoc discusses the remaining MAC open issues of XR.

# 2	Discussion

In the RAN2#131 post-meeting email discussion on XR MAC open issues, a question was raised if the following highlighted text part added for logical channels configured with priorityAdjustmentThreshold is somewhat misleading in terms of which priority the UE would apply for the logical channel:

### 5.4.1 UL Grant reception (…) For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize the HARQ Process ID with the highest priority, where the priority of HARQ process is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. In this selection, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the highest priority that can be applied or has been applied for it in the LCP procedure for the MAC PDU (see clause 5.4.3.1.3). If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection among initial transmission and retransmission with equal priority, the UE shall prioritize retransmissions before initial transmissions. The priority of a HARQ Process for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than the priority of a HARQ Process for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU. If the MAC entity is not configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize retransmissions before initial transmissions. The UE shall toggle the NDI in the CG-UCI for new transmissions and not toggle the NDI in the CG-UCI in retransmissions. (…)

While we think this can be interpreted in the correct way; ie., the UE applies either the priority or the additionalPriority for the logical channel depending on whether the logical channel has a priority adjustable PDCP SDU to be multiplexed (new transmission) or whether it multiplexed a priority adjustable PDCP SDU into a MAC PDU already in the HARQ buffer (re-transmission); we think that this can also be interpreted in a way that UE always applies the additionalPriority for the LCH in this case. Generally, the specification language shall be unambiguously interpretable and, hence, we think some improvement for the wording could be justified.

Proposal: Agree the following TP into TS 38.321:

### 5.4.1 UL Grant reception (…) For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize the HARQ Process ID with the highest priority, where the priority of HARQ process is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. In this selection, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the additionalPriority if the logical channel has a priority adjustable PDCP SDU or a priority adjustable PDCP SDU from the logical channel was multiplexed in the MAC PDU in the HARQ buffer, otherwise it shall be the priority (see clause 5.4.3.1.3). If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection among initial transmission and retransmission with equal priority, the UE shall prioritize retransmissions before initial transmissions. The priority of a HARQ Process for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than the priority of a HARQ Process for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU. If the MAC entity is not configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize retransmissions before initial transmissions. The UE shall toggle the NDI in the CG-UCI for new transmissions and not toggle the NDI in the CG-UCI in retransmissions. (…)

# 3	Conclusion

This document has made the following proposals:

Proposal: Agree the following TP into TS 38.321:

### 5.4.1 UL Grant reception (…) For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize the HARQ Process ID with the highest priority, where the priority of HARQ process is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. In this selection, the priority of a logical channel configured with priorityAdjustmentThreshold shall be the additionalPriority if the logical channel has a priority adjustable PDCP SDU or a priority adjustable PDCP SDU from the logical channel was multiplexed in the MAC PDU in the HARQ buffer, otherwise it shall be the priority (see clause 5.4.3.1.3). If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection among initial transmission and retransmission with equal priority, the UE shall prioritize retransmissions before initial transmissions. The priority of a HARQ Process for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than the priority of a HARQ Process for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU. If the MAC entity is not configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize retransmissions before initial transmissions. The UE shall toggle the NDI in the CG-UCI for new transmissions and not toggle the NDI in the CG-UCI in retransmissions. (…)