3GPP TSG-RAN WG2 Meeting #131-Bis	R2-2507310

Prague, Czech, 13 – 17 October, 2025

Agenda item:	8.7.3

Source:	InterDigital

Title:	Remaining RLC open issues on avoiding unnecessary re-transmissions

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This TDoc discusses the remaining RLC open issues of XR wrt. avoiding unnecessary re-transmissions.

# 2	Discussion

## 2.1	Issue RLC-V01

In the post-meeting email discussion, it was noted that when t-RxDiscard expires, the Rx RLC entity stops waiting to receive the obsolete RLC SDUs, and the corresponding RLC SDUs will be positively acknowledged in the STATUS report. However, the PDCP t-Reordering may be still running and waiting for receiving the outdated SDUs from the RLC layer, which could lead to unnecessary delays in packet delivery to upper layers. Proponent advocates a solution where the Rx RLC entity would indicate this to the PDCP entity.

However, such proposal seems to come with issues: the Rx RLC entity cannot really know whether the RLC PDU that is regarded as obsolete would consist of a PDCP data PDU or a PDCP control PDU. Hence, such an indication to PDCP entity could lead to out-of-order delivered PDCP SDUs which is not the intention. Generally, we don’t see how such indication could be exploited by the PDCP entity.

Proposal 1: No indication from RLC entity to PDCP entity is specified where RLC entity would inform PDCP entity about the expiry of t-RxDiscard.

## 2.2	Issue RLC-N01

In the same email discussion, it was noted that the current description of RX_Next state variable is outdated after adding the support of Rx RLC entity ACKnowledging RLC SDUs based on the t-RxDiscard timer.

The state variable is currently described as follows in TS 38.322 v18.2.0:

## 7.1 State variables (…) a) RX_Next – Receive state variable This state variable holds the value of the SN following the last in-sequence completely received RLC SDU, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next.

The wording needs to be updated and the following is proposed.

Proposal 2: Update the RX_Next state variable description in the TS 38.322 as follows:

a) RX_Next – Receive state variable

If t-RxDiscard is not configured, this state variable holds the value of the SN following the last in-sequence completely received RLC SDU. If t-RxDiscard is configured, this state variable holds the value of the SN following either the last in-sequence completely received RLC SDU or the last discarded AMD PDU. It serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires.

## 2.3	Issue RLC-N02

For issue [RLC-N02], it was agreed and captured in a NOTE that the UE may stop and reset t-PollRetransmit timer when all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded by upper layers. This behaviour enables the UE to stop polling even when NACK has just been received to avoid unnecessary polling. The behaviour is also optional (“may stop and reset the timer”), which also allows different UE behaviour in scenarios where the polling may still be necessary. Therefore, we propose not to pursue further enhancement. However, we agree with the proposal to limit this behaviour only for the case stopReTxDiscardedSDU is configured.

Proposal 3: Add a restriction to the NOTE in 5.3.3.3 regarding stopping and resetting t-PollRetransmit that it is only applicable when stopReTxDiscardedSDU is configured.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: No indication from RLC entity to PDCP entity is specified where RLC entity would inform PDCP entity about the expiry of t-RxDiscard.

Proposal 2: Update the RX_Next state variable description in the TS 38.322 as follows:

a) RX_Next – Receive state variable

If t-RxDiscard is not configured, this state variable holds the value of the SN following the last in-sequence completely received RLC SDU. If t-RxDiscard is configured, this state variable holds the value of the SN following either the last in-sequence completely received RLC SDU or the last discarded AMD PDU. It serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires.

Proposal 3: Add a restriction to the NOTE in 5.3.3.3 regarding stopping and resetting t-PollRetransmit that it is only applicable when stopReTxDiscardedSDU is configured.