3GPP TSG-RAN WG2 Meeting #131-Bis	R2-2507311

Prague, Czech, 13 – 17 October, 2025

Agenda item:	8.7.3

Source:	InterDigital

Title:	Remaining RLC open issue on timely re-transmissions

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This TDoc discusses the remaining RLC open issues of XR wrt. timely RLC re-transmissions.

# 2	Discussion

### 2.1.1	Issue RLC-V02

The following was discussed in the previous RAN2#131 meeting:

R2-2505405 Discussion on RLC enhancement for XR vivo discussion Rel-19 NR_XR_Ph3-Core Proposal 8: The pending remaining time based RLC retransmission should be cancelled when the RLC entity receives a positively ACK for the corresponding RLC SDU, RAN2 to discuss whether there are any specification impacts. - Lenovo suggests to capture that UE will cancel the retransmission and we can try to have it covered together with the previous agreements. - Ericsson agrees with the intention, but we do not have to distinguish retransmission types. No change is needed. Can try to clarify together with the previous agreement.

However, no clarification in the running CR was adopted for the issue. In the legacy specification, the re-transmissions can only be triggered by receiving an RLC STATUS PDU from the peer RLC entity and the specification does not need to consider cancelling pending re-transmissions given if subsequently ACK is received for an RLC SDU, it is not considered for re-transmission – however, such RLC SDU would not be considered for re-transmission when receiving the ACK in the legacy specification.

With the autonomous re-transmission feature, the RLC entity may have already considered an RLC SDU (or a segment) for re-transmission and subsequently receives an ACK for the corresponding RLC SDU (or segment) before its re-transmission. In this case, the pending re-transmission should be cancelled, and this should be reflected in the normative behaviour. Otherwise, with the current specification text, the RLC entity may still re-transmit the RLC SDU (or segment) regardless of if an ACK has been issued by the Rx RLC entity.

Proposal 1: Clarify in the TS 38.322 that the pending re-transmission of an RLC SDU (segment) shall be cancelled in case ACK for the corresponding RLC SDU (segment) is received from the peer RLC entity in an RLC STATUS PDU.

Proposal 2: Adopt the following TP into TS 38.322:

### 5.3.2 Retransmission The transmitting side of an AM RLC entity can receive a positive acknowledgement or a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following: - STATUS PDU from its peer AM RLC entity. When receiving a positive acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall: - if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer; and - if the RLC SDU or the RLC SDU segment is considered for re-transmission: - cancel the retransmission of the RLC SDU or the RLC SDU segment for which a positive acknowledgement was received. When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall: - if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer: - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission. When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall: - if the RLC SDU or RLC SDU segment is considered for retransmission for the first time: - set the RETX_COUNT associated with the RLC SDU to zero. - else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU: - increment the RETX_COUNT. - if RETX_COUNT = maxRetxThreshold: - indicate to upper layers that max retransmission has been reached. When retransmitting an RLC SDU or an RLC SDU segment, the transmitting side of an AM RLC entity shall: - if needed, segment the RLC SDU or the RLC SDU segment; - form a new AMD PDU which will fit within the total size of AMD PDU(s) indicated by lower layer at the particular transmission opportunity; - submit the new AMD PDU to lower layer. When forming a new AMD PDU, the transmitting side of an AM RLC entity shall: - only map the original RLC SDU or RLC SDU segment to the Data field of the new AMD PDU; - modify the header of the new AMD PDU in accordance with the description in clause 6.2.2.4; - set the P field according to clause 5.3.3.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: Clarify in the TS 38.322 that the pending re-transmission of an RLC SDU (segment) shall be cancelled in case ACK for the corresponding RLC SDU (segment) is received from the peer RLC entity in an RLC STATUS PDU.

Proposal 2: Adopt the following TP into TS 38.322:

### 5.3.2 Retransmission The transmitting side of an AM RLC entity can receive a positive acknowledgement or a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following: - STATUS PDU from its peer AM RLC entity. When receiving a positive acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall: - if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer; and - if the RLC SDU or the RLC SDU segment is considered for re-transmission: - cancel the retransmission of the RLC SDU or the RLC SDU segment for which a positive acknowledgement was received. When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall: - if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer: - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission. When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall: - if the RLC SDU or RLC SDU segment is considered for retransmission for the first time: - set the RETX_COUNT associated with the RLC SDU to zero. - else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU: - increment the RETX_COUNT. - if RETX_COUNT = maxRetxThreshold: - indicate to upper layers that max retransmission has been reached. When retransmitting an RLC SDU or an RLC SDU segment, the transmitting side of an AM RLC entity shall: - if needed, segment the RLC SDU or the RLC SDU segment; - form a new AMD PDU which will fit within the total size of AMD PDU(s) indicated by lower layer at the particular transmission opportunity; - submit the new AMD PDU to lower layer. When forming a new AMD PDU, the transmitting side of an AM RLC entity shall: - only map the original RLC SDU or RLC SDU segment to the Data field of the new AMD PDU; - modify the header of the new AMD PDU in accordance with the description in clause 6.2.2.4; - set the P field according to clause 5.3.3.