3GPP TSG-RAN WG2 #131bis	R2-2507312

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda Item:	10.2

Source:	InterDigital, Inc.

Title:	Design of 6GR Radio Protocols

Document for:	Discussion

# Introduction

RANP #108 approved a study item (SI) Description (SID) for 6G Radio (6GR) [1].

The study of 6GR aims to define a standalone cellular system with a radio access technology characterized by superior performance, inherent scalability, extensibility, ease of integration of data-driven techniques and energy efficiency with a design that minimizes unnecessary optionality for a given functionality.

The 6GR design should aim to enable economies of scale in terms of modularity and reuse of existing hardware while improving parallelization of critical path and processing complexity of user plane data and control plane signalling. The design should enable reduction of MNO’s operational costs in terms of spectral efficiency and energy consumption. The design should also enable new capabilities related to sensing, AI/ML, management and transfer of related operational data.

From an end-user perspective, 6GR must deliver efficient support for emerging services such as immersive multimodal experiences and mobile Generative AI applications, ensuring both high Quality of Experience (QoE) and efficient use of system resources. Its success will depend on providing clear, tangible improvements in user experience alongside the resource efficient enablement of entirely new services.

This contribution discusses the scope of RAN2 work for the 6GR R20 study phase including design targets, design process and a high-level breakdown of possible focus areas.

# Design Principles for 6GR

## RAN2 scope and basic working assumptions

RAN2 Scope for 6GR Study

For the study on 6GR, RAN2 is responsible for defining the radio interface protocol architecture and procedures [1]. This includes the design of the User Plane architecture and L2 protocols including radio level QoS, as well as the Control Plane architecture and L3/RRC protocol (including RRC states). Its key objectives are to ensure low-latency mobility across all RRC states, address security for the Access Stratum in alignment with SA3 requirements, develop a simplified signaling framework for UE capability exchange and define data transfer mechanisms supporting diverse data types such as data collected for AI/ML and sensing data.

RAN2 will also contribute to the overall RAN architecture and internal functional splits under RAN3’s responsibility while also addressing design aspects of initial access procedures, system information, paging, and higher-layer procedures related to sensing in alignment with progress in other WGs e.g., RAN1 and SA2.

Observation 1: 	Timely inter-TSG coordination with SA2 and SA3 will be critical for RAN2’s progress.

For example, coordination with SA2 on paging, data collection, sensing, and user plane (including QoS) as well as with SA3 on access stratum security will directly impact RAN2’s ability to progress.

Baseline working assumptions for 6GR study

RANP will revisit the question of Dual Connectivity (DC) support for 6G when it begins its study in March 2026, considering both 5G/6G and 6G-only DC scenarios. Until then, the baseline assumption is to focus the 6GR study on a standalone system architecture, optimized to support both communication and non-communication functions such as AI/ML-based solutions and sensing. It should also be assumed that 6GR deployments may involve CU/DU splits.

## High-level design targets

Design targets for 6GR have been discussed at the 3GPP workshop on 6G in March 2025, as well as in various discussions and contributions to RANP.

The following design focus, listing topics to address earliest first, is expected to be central to RAN2’s R20 work on 6GR :

1) Efficient handling of advanced services and complex traffic patterns

This target aims at enabling differentiated service treatment and flexible, fine-grained QoS enforcement at the per-packet or per-flow level. The L2 protocol design must also explicitly support advanced traffic types, including bursty traffic with varying volumes and inter-burst intervals, delay-sensitive traffic, and potentially “tokenized” data (e.g., related to network coding or FEC).

Observation 2: 	Consistent end-user experience under varying radio conditions and system load is essential for advanced 6G services such as mobile generative AI and immersive multimodal XR.

Observation 3: 	A QoS framework with QoE awareness, per-subflow granularity, and parameter ranging is to support advanced 6G services in a resource- and energy-efficient manner.

Proposal 1: 	For 6GR, RAN2 initiates and prioritizes work on RAN-level QoE-aware QoS framework.

Contribution R2-2507313 [2] discusses 6G services, applications and their traffic characterization.

2) Energy efficiency

This target aims to design energy savings from both the network and the terminal perspective while overcoming the backward compatibility limitations of 5GA.

Pragmatically, RAN2 should begin by reviewing existing techniques and identifying those with the most tangible benefits. The initial focus should be on building the foundation around network-based energy savings and efficiency, prioritizing time-domain techniques to establish the operational baseline. Spatial- and frequency-domain techniques can then be studied in later phases, once the time-domain baseline is in place. This will impact design of cell selection, system information acquisition, paging and initial access as well as control channel operation.

Observation 4: 	Energy efficiency should first be addressed from gNB and radio access network operation perspective, to identify key baseline system-level design aspects as early as possible.

Observation 5: 	Energy efficiency will impact critical L2/L3 functions, including at least cell re/selection, system information acquisition, paging, initial access, and control channel operation.

Proposal 2: 	For 6GR, RAN2 prioritizes early and timely progress on identifying baseline network energy savings techniques common across any and all 6G device types.

Any UE requirements needed to support baseline 6G network energy savings will most likely become part of the minimum mandatory function set for all 6G UEs. Building on this, RAN2 can then introduce additional UE-based energy-saving techniques.

Observation 6: 	The design of network-side energy reduction techniques will largely determine the minimal set of mandatory functionalities that must be supported by all 6G devices and device types.

The priority should be to minimize always-on signal transmissions and receptions that consume significant power (e.g., SSB, SIBx), to the point where cell operation can even be fully suspended in certain deployments - for example, by separating cells providing coverage and those providing capacity.

This includes supporting signaling that indicates available common resources in time, frequency and space domains for both RRC_IDLE/INACTIVE and RRC_CONNECTED UEs, while adapting UE-based power saving mechanisms accordingly. Principles behind 5G’s LP-WUS, WUR, and C-DRX can be reused, with additional measures like faster transitions to the INACTIVE state and relaxed measurement requirements at the terminal side.

Contribution R2-2507180 [6] further elaborates on energy efficiency.

3) Simplification of user plane protocols

This target aims to reduce L2 processing overhead, particularly along the critical low-latency path at very high data rates. This includes addressing redundant functions, such as sequence numbering (SN) and retransmission mechanisms that currently exist in both PDCP and RLC.

Different approaches can be considered, such as PDCP SDU concatenation to exploit hardware processing and reduce the per-PDU overhead of security functions (ciphering/integrity protection); merging SDAP into PDCP to limit header overhead and applying sequencing per QoS flow with improved differentiation in PDCP in conjunction with QFI; integrating PDCP and RLC to enable SN reuse when dual connectivity is not supported; and introducing transmitter-initiated ARQ retransmissions upon HARQ feedback failure, or alternatively removing ARQ from RLC. The feasibility of these options may also be constrained by CU/DU split assumptions.

Fixed, or limited set of discrete and possibly configurable size(s) for baseline processing units can also contribute to more efficient hardware processing.

Contribution R2-2507313 [2] discusses the design of user plane protocols.

4) Control plane and mobility improvements

This target aims at consolidating mobility by integrating L3-based mobility, L1-Triggered Mobility (LTM), and Conditional Handover (CHO) within a common measurement framework; simplifying paging, for example by adopting either RAN-only or CN-only based paging; enhancing the INACTIVE state to support low-rate data transfers and enable faster transition to/from connected mode; and reducing both ASN.1 overhead and signaling complexity to allow parallelized processing.

Further optimizations using DU-based control and signaling for updating active configurations can also be considered to reduce interruption time and reconfiguration latency.

Contribution [5] discusses design of control plane while [4] addresses connected mobility.

5) Transfer of other types of data

This target aims at efficient transmission of large volumes of operational data such as collected datasets, AI/ML models, training data, or sensing results. RAN2’s priority should be to enable differentiated treatment of such data, allowing it to be scheduled and transferred opportunistically without disrupting other services for the same or other UEs. Whether this is achieved by extending radio bearer definitions, introducing a service layer, or creating a RAN-terminating function is largely a matter of modeling, where a lightweight approach enabling differentiated treatment and scheduling should be preferred that blends with other QoS-related improvements for differentiation of user-plane data.

Contribution [3] further discusses transfer of operational data.

6) Access Statum Security

This target aims to align with SA3 recommendations possibly including addressing requirements of new security algorithms with key lengths up to 256 bits to protect against quantum computing threats; applying Integrity Protection (IP) to some or all MAC Control Elements (CE) and broadcast system information to prevent attacks such as fake LTM cell switch commands and MIB/SIB spoofing; and potentially extending security with privacy for specific paging information and IP for user plane or other types of data.

Contribution R2-2507180 [6] further discusses security-related aspects.

7) AI/ML framework

This targets a generalized, use case–agnostic LCM that is extensible to support easier integration of AI/ML models in future releases, with priority given to one-sided models for the 6GR air interface. It also includes designing a generic data collection and management framework capable of supporting data exposure, applicable to any type of operational data e.g., for AI/ML and for sensing-related measurements for ISAC.

For AI/ML use cases in 6GR L2/L3, AI-based mobility solutions from 5GA can be supported with minimal additional evaluation. For new 6GR L2/L3 use cases, however, it is important to first verify that they truly involve an AI/ML aspect rather than being solvable through adding flexibility in some aspects of RRC configurations combined with enabling specific implementation choices. For new 6GR L2/L3 use case that truly require support of LCM aspects such as performance monitoring and fallback behavior, then RAN2 should first carefully assess their potential benefits in real deployments.

Given guidance in the 6GR SID stating that 6GR shall be capable of operating without use of AI/ML, RAN2 should prioritize progress on framework-related topics and generic enablers before studying any new use cases.

Contribution [3] further discusses how to address AI/ML and data management for 6GR in RAN2.

8) Device types, scalability and capabilities

This target aims at identifying the minimal set of mandatory functions common to all device types; defining UE capabilities in a compartmentalized manner with minimal cross-dependencies; and optimizing signaling through functional grouping, where multiple capabilities can be configured per group and indexed by IDs, while supporting continuity across gNBs.

For 6GR, scalability means adapting to increasing load while minimizing fixed costs, enabling support for diverse and even unforeseen device and traffic types without requiring dedicated spectrum or hardware per case. Achieving this also requires energy scalability - minimizing always-on signals and dynamically activating resources with fine granularity - so that energy consumption aligns with actual demand while maintaining seamless UE operation.

Observation 7: 	The minimum set of requirements that must be common to all 6G devices derives from network operation, network scalability, deployment scenarios, and energy efficiency.

Observation 8: 	RAN2 discussions on whether to define device types, UE categories, minimum capability set and possibly support dynamic capability updates can be deferred to later stages.

Observation 9: 	RAN2 can discuss the overall capability framework design including efficient processing and low signaling overhead once baseline requirements are clear.

Proposal 3: 	For 6GR R20, RAN2 identifies the minimum set of requirements common to all 6G device types while leaving capability related improvements and signaling for R21 normative phase.

Scalable design, mandatory features, optionality and capabilities are discussed in a later section below.

## Systematic design approach for L2/L3 protocols

When designing the L2/L3 architecture and protocols for 4G LTE in Release 8, RAN2 followed a highly systematic process, resulting in a robust design that was largely reused for 5G NR in Release 15. Since then, however, the L2/L3 protocols have evolved to add new capabilities, increasing complexity, while service and application QoS requirements have become more demanding. In addition, various shortcomings have been observed in field deployments.

Observation 10: Lessons learned from the implementation and deployment (or lack thereof) of 5G NR L2/L3 protocol features provide a valuable starting point for studying their design in 6GR.

In addition to meeting the design objectives outlined in the 6GR SID and the functional, deployment, and performance requirements specified in [RP-250810] and [TR38.914], the design of L2 protocols must also be grounded in a clear understanding of the traffic characteristics of current and anticipated services and applications.

Observation 11: Defining clear and well-motivated functional requirements for the design of the L3 and L2 protocols is a necessary step for RAN2 in the 6GR study.

RAN2 should follow a process with a design approach that is closer to what was done for 4G LTE L2/L3 protocols for the study of 6GR. LTE protocols were designed through a thorough bottom-up process, considering all functional requirements and architectural trade-offs. Their robustness is evident in the fact that they later continued to serve as the baseline for 5G NR L2/L3, even more than a decade later. By contrast, the design of 5G NR protocols was mostly incremental: LTE protocols were assumed as the starting point, and modifications were applied to meet new requirements. While this was effective in enabling timely deployment of 5G, it was not a comprehensive re-examination of the protocol space with limitations in its potential for extensibility and introduction of new features in a backward compatible manner. For 6G, it may be advisable to follow an LTE-style design approach to simplify, streamline and ensure that protocols are future-proofed.

For 6G, the objective should be to apply a systematic and complete design process as opposed to applying an incremental approach treating 5G NR as the baseline to decide what to retain, remove, or adjust risks perpetuating inefficiencies and overlooking issues in processing, energy use, scheduling, and QoS handling. Instead, each L2/L3 component should be revisited with 6G requirements in mind, following a rigorous methodology rather than inheritance by default.

Observation 12: A systematic review of L2/L3 functional elements will lead to a modular design approach, improve L2/L3 protocol design and help limit the number of candidate solutions for R21.

Proposal 4: 	For 6GR, RAN2 uses a systematic bottom-up design process aiming to minimize the number of candidate solutions and/or options without artificial constraints from existing 5G NR protocols.

Proposal 5: 	RAN2 first compiles lessons learned from NR, then defines functional requirements, studies and designs functional components and finally specifies procedures and protocols for 6G L2/L3.

Pragmatically, this process should lead to a sound design that reuses, adapts, augments and improves on proven components of the 5G NR user plane and control plane in a modular and extensible manner to meet the more stringent 6GR expectations aligned with requirements. It will also facilitate consensus by limiting the number of candidate solutions and the number of options retained for each function ahead of the normative phase.

# Design for 6GR

The motivations for taking a very systematic design approach include ensuring L2/L3 properly delivers on:

- QoE-driven QoS framework to effectively manage end user perceived experience for immersive applications and machine-based task success rate for mobile Generative AI, moving beyond throughput/latency as the sole performance metrics.

- Latency and delay bound guarantees to handle strict multimodality requirements, including delay, jitter, and media synchronicity, including for high data rate applications.

- Scheduling improvements for bursty traffic, inter-PDU prioritization, and dynamic traffic shaping.

- Application awareness to enable finer QoS granularity, QoS parameter ranging and to leverage real-time adaptation.

- Improved MAC control e.g., for faster RF/bandwidth adaptation and low-overhead reconfigurations.

- Improved security for MAC control information and for broadcast system information.

- Computational and energy efficiency to ensure scalability across all 6G device classes, from IoT nodes to high-performance terminals.

## Services, applications and traffic characteristics

An immersive multimodal XR application can adapt its date rate e.g., in terms of modalities, scene composition, resolution and compression ratio while maintaining acceptable QoE level and subjectively satisfactory end user perception.

A Mobile Generative AI application can exhibit mid-term, contextually driven changes in its dominantly uplink data rate for its periodic stream component while generating delay-bounded bursts of varying data volumes and inter-burst intervals where the timely and reliable transfer of data directly impacts the applications task’s success rate.

The 6GR design should enable tradeoff decisions between system load, congestion avoidance, energy consumption and QoS servicing while maintaining an acceptable QoE level, for human perception as well as machine-based task success rate. The focus should be on RAN-based adaptation of QoS servicing level to manage congestion, system load and energy efficiency within acceptable QoE for interactive, real-time services. The objective for R20 would be to identify RAN-level techniques and QoS requirements that will support a QoE-driven framework including studying QoS adaptation and ranging, sub-flow QoS granularity and Delay Critical Guaranteed Burst Rates.

More details on traffic characterization for 6G applications are provided in [2].

## Scalable design, mandatory features, optionality & capabilities

### Scalable design

Scalability can be defined as the ability of a system to adapt to increasing load without excessive cost. This implies that fixed costs (i.e., overhead independent of load) is minimized. For 6GR, the system must be scalable while supporting diverse device types (in terms of e.g. maximum supported bandwidth and maximum coupling loss) and diverse traffic types (in terms of e.g. latency and reliability requirements). Ideally, a scalable system has this ability not only for currently envisioned, but also for unanticipated device or traffic types. The “cost” dimension includes multiple aspects such as spectrum licensing, hardware, or site acquisition. For 6GR, there is increased emphasis on energy costs at the network side compared to previous generations.

Observation 13 : 	Scalability is the ability of a system to adapt to increasing load without excessive cost.

Scalability with respect to spectrum or hardware means that the network can serve multiple device types and/or traffic types using common spectrum and hardware. If the network needs to reserve separate carriers and hardware for each device or traffic type, the overhead is very high, and the design is not scalable. To achieve this implies that the design enables sharing uplink and downlink resources for initial access (e.g. RACH, system information, paging) across the device types at least when the load is low.

A scalable 6GR is characterized by a design enabling sharing resources for initial access across device types.

Observation 14: 	Scalability in relation to spectrum or hardware is the ability of a system to serve multiple device and/or traffic types using common spectrum and hardware.

Scalability with respect to energy means that the consumed energy during a period is commensurate with the actual demand during that period instead of being fixed. This implies that always-on signals are minimized and that the system can completely turn on or off transmission and reception of resources with high granularity in time, frequency, space and power/antenna domains without affecting UEs already camping or connected. For example, the design should enable the network turn off transmission/reception of a TRP on a certain carrier with minimal impact to connected or camped UEs. One possible design direction towards this target is that the protocol supports dynamic updates of cell resource information indicating available resources at a time. The design should also enable transmitting using a number of antenna elements and power not higher than required to achieve maximum performance when serving a UE.

A scalable 6GR is characterized by a design that minimizes transmission of always-on signals.

A scalable 6GR is characterized by a design that enables turning on and off transmission and reception of resources with high granularity in time, frequency, space and power domains without affecting UEs operating in the system.

Proposal 6: 	Scalability in relation to energy is the ability of a system to consume energy over time commensurate with actual demand, rather than being fixed or bounded by a specific value.

### UE capability Framework

UE capability framework

6GR must define a framework for managing UE capabilities. The definition and management of capabilities are central to interoperability, scalability, energy efficiency, and signalling optimization.

Minimum set of UE capabilities

The framework should define the smallest set of mandatory features common to all devices. As discussed earlier, this set will derive from network operation, network scalability, deployment scenarios, and energy efficiency.

A generalization of this approach should also consider means to further downscale the minimum set of UE capabilities defined for 6GR in future releases in a backward compatible manner.

Other UE capabilities

The framework should manage capabilities beyond the minimum set of capabilities, where a set of capability should include function or feature-based capability grouping with little or no interdependency. The definition of a “function” or a “feature” for signalling purposes should reflect what can be used to distinguishes one device type from another based on whether the device supports a given function - for example, mobility – or a feature specific to e.g., fixed wireless access, smartphones, wearables or IoT. For example, grouping capabilities for a given function under common signalling bits (e.g., bits to indicate support for mobility using a default, a minimum or one of many sets of capabilities) simplifies exchanges while ensuring UEs can always signal support beyond the minimum baseline.

Within the capabilities that are beyond the minimum set of UE capabilities, the framework should support mandatory and optional features as well as grouping e.g., to form device types, or feature sets.

Signalling optimizations

The framework should aim to minimize signalling complexity and overhead – in particular in relation with band combinations and bandwidth parts configurations. Possibly, RAN2 can study how to best support a dynamic change in UE capabilities initiated autonomously by the UE or by network-driven triggers.

In NR, UE capability signalling has become very large - often thousands of bytes - primarily due to per-band reporting. The framework also handles dynamic changes poorly (e.g., MUSIM, overheating, foldable devices), resulting in attach bottlenecks, higher failure risk, and interoperability issues.

For 6GR, capability reporting should be lighter and more flexible:

- Mandatory capability sets tied to device type to reduce baseline overhead.

- Capability IDs and compression to minimize signalling size.

- Dynamic profiles to support capability changes without full re-reporting.

Later in the study phase or in the normative phase, RAN2 should define UE capability signalling for 6GR that includes device type/category based mandatory feature sets, compressed reporting, and capability change during a connection.

## Integrated Sensing and Communications (ISAC)

For 6GR, sensing and ISAC should be addressed only once sufficient groundwork has been completed in 5GA. R19 has already focused on ISAC channel modeling, while R20 concentrates on gNB-only sensing for UAV detection and tracking. To avoid duplication and to build on these efforts, 6GR studies on ISAC should first begin earliest in RAN2#133bis (April 2026). This timeline ensures that progress on reference signals and measurement quantities in RAN1, along with architecture and configuration aspects in RAN3 and the core network, can inform RAN2’s work on ISAC.

When 6GR ISAC studies start in RAN2, they should focus on gNB-UE bi-static sensing modes i.e., prioritizing UE-involved scenarios such as TRP-UE bi-static sensing. Key topics in RAN1 include whether a dedicated sensing reference signal is needed or if CSI-RS/SRS can be reused, as well as frame structure and measurement/reporting design for velocity and Doppler estimation. Other topics related to communications, such as how to minimize or avoid the need for explicit support of measurement gaps for sensing tasks for UEs also active in communications. ISAC KPIs such as sensing success rate, false alarm rate, coverage, and latency should be clearly defined in RAN2.

In terms of evolution, 6G Day-1 should ensure baseline sensing capability across diverse use cases, with more advanced sensing-assisted communication deferred to Day-2. This phased approach helps focusing on a commercially relevant baseline with immediate value for applications like UAVs, smart factories, and public transportation, while ensuring the framework can extend to broader ISAC functions over time.

The system must explicitly support measurement gaps in communication for sensing tasks. Sensing data transfer is managed as operational data,

Proposal 7: 	RAN2#133bis will start studying ISAC/sensing for 6GR, to align with 5GA ISAC work.

## Non-Terrestrial Networks (NTN)

NTN is expected to play a central role as a complement to terrestrial networks (TN) in 6G, to meet coverage requirements in hard-to-reach areas. For 6GR, there is a clear desire towards a design where the air interface can both support TN and NTN operation, with minimal need for NTN-specific features and ensuring consistent system behavior across deployment scenarios.

Key study areas for such harmonization between TN/NTN include:

- SSB periodicity, addressing NTN-specific requirements.

- Repetition-native initial access, to improve robustness in NTN environments without relying on GNSS.

- Support for a large number of HARQ processes, with options for feedback disabling to accommodate NTN latency and channel characteristics.

This approach ensures that NTN integration into 6GR is efficient, streamlined, and fully interoperable with terrestrial systems from the outset.

# Observations and Proposals

[RAN2 scope and inter-TSG coordination]

Observation 1: 	Timely inter-TSG coordination with SA2 and SA3 will be critical for RAN2’s progress.

[Design targets for 6GR L2/L3 protocols]

Observation 2: 	Consistent end-user experience under varying radio conditions and system load is essential for advanced 6G services such as mobile generative AI and immersive multimodal XR.

Observation 3: 	A QoS framework with QoE awareness, per-subflow granularity, and parameter ranging is to support advanced 6G services in a resource- and energy-efficient manner.

Proposal 1: 	For 6GR, RAN2 initiates and prioritizes work on RAN-level QoE-aware QoS framework.

Observation 4: 	Energy efficiency should first be addressed from gNB and radio access network operation perspective, to identify key baseline system-level design aspects as early as possible.

Observation 5: 	Energy efficiency will impact critical L2/L3 functions, including at least cell re/selection, system information acquisition, paging, initial access, and control channel operation.

Proposal 2: 	For 6GR, RAN2 prioritizes early and timely progress on identifying baseline network energy savings techniques common across any and all 6G device types.

Observation 6: 	The design of network-side energy reduction techniques will largely determine the minimal set of mandatory functionalities that must be supported by all 6G devices and device types.

Observation 7: 	The minimum set of requirements that must be common to all 6G devices derives from network operation, network scalability, deployment scenarios, and energy efficiency.

Observation 8: 	RAN2 discussions on whether to define device types, UE categories, minimum capability set and possibly support dynamic capability updates can be deferred to later stages.

Observation 9: 	RAN2 can discuss the overall capability framework design including efficient processing and low signaling overhead once baseline requirements are clear.

Proposal 3: 	For 6GR R20, RAN2 identifies the minimum set of requirements common to all 6G device types while leaving capability related improvements and signaling for R21 normative phase.

[Design approach]

Observation 10: Lessons learned from the implementation and deployment (or lack thereof) of 5G NR L2/L3 protocol features provide a valuable starting point for studying their design in 6GR.

Observation 11: Defining clear and well-motivated functional requirements for the design of the L3 and L2 protocols is a necessary step for RAN2 in the 6GR study.

Observation 12: A systematic review of L2/L3 functional elements will lead to a modular design approach, improve L2/L3 protocol design and help limit the number of candidate solutions for R21.

[Design of 6GR]

Proposal 4: 	For 6GR, RAN2 uses a systematic bottom-up design process aiming to minimize the number of candidate solutions and/or options without artificial constraints from existing 5G NR protocols.

Proposal 5: 	RAN2 first compiles lessons learned from NR, then defines functional requirements, studies and designs functional components and finally specifies procedures and protocols for 6G L2/L3.

Observation 13 : 	Scalability is the ability of a system to adapt to increasing load without excessive cost.

Observation 14: 	Scalability in relation to spectrum or hardware is the ability of a system to serve multiple device and/or traffic types using common spectrum and hardware.

Proposal 6: 	Scalability in relation to energy is the ability of a system to consume energy over time commensurate with actual demand, rather than being fixed or bounded by a specific value.

Proposal 7: 	RAN2#133bis will start studying ISAC/sensing for 6GR, to align with 5GA ISAC work.

# References

- RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator), Prague, CR, June 9-13, 2025.

- R2-2507313, “Requirements for 6GR L2 protocols”, InterDigital Inc., RAN2#131b

- R2-2507314, “Framework for AI/ML and Transfer of Various Data Types”, InterDigital Inc., RAN2#131b

- R2-2507432, “Connected Mobility for 6GR”, InterDigital Inc., RAN2#131b

- R2-2507433, “Control plane for 6GR”, InterDigital Inc., RAN2#131b

- R2-2507180, "Energy efficiency and AS security for 6GR". InterDigital Inc., RAN2#131b