3GPP TSG-RAN WG2 Meeting #131bis	R2-2507318

Prague, Czech Republic, Oct. 13th 17th 2025

Agenda item:	9.2.2 (Ambient IoT Ph2)

Source:	Sony

Title:	Discussion on Topology 2 for Ambient IoT

Document for:	Discussion and Decision

# 1 Introduction

At RANP#108 the following study item for Ambient IoT Ph2 was agreed [1], with the following objectives.

- Specify necessary support for active Device(s), taking the Rel-19 specifications as the starting point, in indoor scenarios under the following conditions [RAN1]:

- Deployment scenario 1 with topology 1, according to D1T1-C.

- Traffic types DO-DTT, DT, DO-A.

  - For DO-A traffic, WGs begin discussions from Clause 6.10 “DO-A assessment” in TR 38.769.

- Representative use cases rUC1 (indoor inventory), rUC2 (indoor sensor), and rUC4 (indoor command).

- Assumption that A-IoT BS readers are deployed on the same sites as existing indoor NR micro BSs.

- FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum.

This objective begins after RAN#111 (March 2026)

This objective includes:

RAN1-led

- In specifying support for the above, re-use the evaluation assumptions of TR 38.769

- Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769

- Air interface for active device(s) studied for outdoor will be reused for supporting active device(s) in indoor scenarios

RAN2-led

- DO-A specific procedure design.

- Paging and random access for active device(s).

RAN3-led

- Specify DO-A specific NGAP procedure.

RAN4-led

- Study coexistence between active Device(s) and NR/LTE in indoor scenarios [RAN4-led]

- Specify RF requirements for Type 1-C Ambient IoT BS where differences are needed from Rel-19.

- Specify RF requirements for active Device(s) with IF/ZIF receiver architecture

- Specify RRM core requirements for active Device(s) in D1T1

- Use band n8 as an example band

- Study and specify the test methodology for AIoT active devices

NOTE: Strive to minimize evaluation cases in RAN1 and RAN4.

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]:

- According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769.

  - No specification support for CW control

- According to D2T2-C for active Device(s).

  - Start after March 2026

- Traffic types DO-DTT and DT, and DO-A (only for active Device(s)).

  - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026

- Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command).

- FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum.

- Specify a single architecture by down-selection between RRC-based architecture and UP-based architecture, in coordination with SA2.

- The following are not supported:

  - Coordination between UEs operating as UE readers

  - New Uu protocol layer(s)

  - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band

This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1.

The work is subject to the following constraints:

RAN1

- The objective includes specification of necessary power control and collision handling procedures between intermediate UE and BS.

- No physical layer impact for signaling A-IoT resources to the intermediate UE.

- No additional specification change on R2D and D2R for Device 1 in Rel-20

RAN2

- The objective includes specification of necessary Uu procedures to enable the A-IoT operations.

- RRC CONNECTED UE reader only, including RLF, without specific enhancement for other temporary out-of-connection scenarios.

- A-IoT radio resource of the UE reader is only valid within the serving cell.

- Resource allocation to the UE Reader shall be via Uu RRC signaling.

RAN3

- Specify signaling for Topology 2

- The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection.

  - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized.

- Inter-gNB resource coordination is not specified.

- Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

RAN4

- Specify RF requirements for intermediate UE towards the A-IoT device.

- Specify necessary updates to RF requirements for A-IoT device 1.

- Specify A-IoT active device(s) RF requirements in D2T2

- Specify RRM requirements for intermediate UE due to A-IoT operation, if any. Combination with existing RRM procedures/requirements is excluded.

- Specify RRM requirements for Device(s) in D2T2 if necessary

- Use band n8 as an example band.

- Specify device (un)availability via Direction 2 in TR 38.769, [RAN2-led, RAN1].

- This objective is applicable only for active Device(s)

- Solutions shall be unified for Topology 1 and Topology 2.

- This objective begins after RAN#111 (March 2026)

- NOTE: Device for outdoor scenarios may be added here, pending RAN#111 decision.

Note: Coexistence between Device 1 and Device 2b/C is not considered in the same deployment in the same band.

For objective 2, Deployment Scenario 2 with Topology 2 with intermediate UE as Reader, the following activities for RAN2 has been outlined.

- The objective includes specification of necessary Uu procedures to enable the A-IoT operations.

- RRC CONNECTED UE reader only, including RLF, without specific enhancement for other temporary out-of-connection scenarios.

- A-IoT radio resource of the UE reader is only valid within the serving cell.

- Resource allocation to the UE Reader shall be via Uu RRC signaling.

And for this upcoming meeting RAN2 should only consider DO-DTT and DT. (DO-A to be treated after March 2026).

In this paper, we discuss, down-selection between RRC-based architecture and UP-based architecture, Uu procedures, RLF and Resource allocation.

# 2 Discussion on Topology2

## 2.1 Down-selection between RRC- and UP-based architecture

During the Rel-19 study phase [2], also Topology 2 was discussed the following high-level architecture was defined, .Figure 1

Figure 1, architecture topology 2

Figure 6.4.2-1 Logical system architecture for topology 2

In this topology the gNB is able to support an AIoT-enabled intermediate UE in topology 2.

Furthermore, , a number of solutions were discussed.

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

In the objective the following statement was included in.

“Specify a single architecture by down-selection between RRC-based architecture and UP-based architecture, in coordination with SA2”.

But according to SA2, [3] they are only working on RRC based solution.

Observation 1	SA2 are only working on the RRC based solution and have not defined any other solutions like NAS or UP based.

Therefore, it seems logical also for RAN2 to focus on the RRC based solution.

Proposal 1	RAN2 should focus the work on the RRC solution.

## 2.2 Uu procedure (RRC based solution)

The RRC based solution defined during the study [2] has the following protocol structure, see Figure 2.

Figure 2, protocol stack RRC solution Topology 2

Figure 6.4.2.1.1-1: RRC based solution of Topology 2

The following basic procedure steps was also defined using the A-IoT enabled intermediate UE as a AIoT reader.

Figure 3, Message flow RRC based solution topology 2

Figure 6.5.3.1.1-1: Message flow for A-IoT Inventory in Topology 2 - RRC-based solution

1a.	The A-IoT CN sends an Inventory request message to the A-IoT enabled gNB1b.	The A-IoT enabled gNB allocates and coordinates usage of A-IoT radio resources.

1c/2a	RRC communication with the A-IoT enabled UE takes place.

NOTE 1: RRC based communication is only depicted schematically, details subject to RAN2.

2.	The A-IoT-enabled gNB sends an Inventory response message to the A-IoT CN.

NOTE 2: In step 2, the A-IoT-enabled gNB may instead send an Inventory failure message to the A-IoT CN indicating that the inventory procedure could not be initiated towards the A-IoT device(s).

3.	The A-IoT-enabled gNB requests the A-IoT-enabled UE(s) to trigger inventory procedure towards the A-IoT device(s).

4a/4b.	After receiving inventory result reported from the A-IoT enabled UEs, the A-IoT-enabled gNB may send one or multiple Inventory reports towards the A-IoT CN including the received inventory result.

NOTE 3: Steps 4a/4b may happen in parallel with Step 3 for different A-IoT devices.

NOTE 4: Step 4a/4b between A-IoT-enable UE and A-IoT-enabled gNB subject to RAN2.

The steps above is only high level as pointed out in Note 1 above. The signaling details between the gNB and AIoT enabled UE needs to be specified.

The specification needs to consider everything from finding suitable UE Reader to configure the selected UE Readers with relevant configuration parameters to be used when communicating with AIoT devices, e.g., resources to be used.

In Rel-19, SA2 [4] has defined one out of many, a solution #30, Support AIoT Devices using AIOTF for Topology 2. Here, it is the AIOTF who is in charge of the intermediate UE for the A-IoT operations, including determining or selecting intermediate UEs, sending operation commands to and receiving results from intermediate UEs.

As the licensed spectrum is owned by MNO, it is proposed to let network provide radio resource information to the intermediate UEs about the spectrum information for the over-the-air interface between Intermediate UEs and AIoT devices.

Based on this assumptions it seems that given spectrum information, it could be up to the gNB to provide necessary resources to the Intermediate UE´s, potentially in form of resource pools.

The RRC option is briefly described in chapter 8.1.3.3 [4], where it is clarified that a UE that has been authorized to act as a UE reader can be assigned radio resources for communication with the AIoT device.

Furthermore, no impact on Rel-19 AIoT devices is assumed.

Proposal 2	Discuss signaling impact for RRC based solution selecting and configuring UE based intermediate readers in order to be able to convey AIoT Inventory and Command messages to the AIoT devices via the intermediate UE reader without any impact on Rel-19 AIoT devices.

## 2.3 RLF

A-IoT enabled UE may encounter RLF and perform re-establishment either on the same cell or a different cell. One potential problem during RLF could be the pending RRC Inventory Report messages and whether A-IOT enabled UE should store these messages during RLF and send them when the connection has been established or discard these messages. The solution depends on if upper layers can recover or account for the missing inventory reports or not. We think RAN2 should discuss this scenario.

Proposal 3	RAN2 to discuss the handling of pending inventory reports during A-IoT enabled UE RLF.

## 2.4 Resource allocation

Several AIoT-enabled UEs may be configured as intermediate node readers to communicate with AIoT device. It is expected that the NW/gNB to assign certain resources to these AIoT-enabled UEs when communication with AIoT devices. In a dynamic area where multiple AIoT-enabled UEs are in the vicinity of each other and the number of AIoT devices may vary, a lack of scheduling management may result in severe interference and collisions between the AIoT-enabled UEs. In our view, it is necessary to study solutions where the assignment of the physical resources are performed such that it avoids interference among AIoT enabled UEs.

Proposal 4	RAN2 to study solutions on how to perform resource allocations for AIoT-enabled UEs to avoid interference among them.

# 3 Conclusion

In this contribution we have made the following observations and proposals.

Observation 1	SA2 are only working on the RRC based solution and have not defined any other solutions like NAS or UP based.

Proposal 1	RAN2 should focus the work on the RRC solution.

Proposal 2	Discuss signaling impact for RRC based solution selecting and configuring UE based intermediate readers in order to be able to convey AIoT Inventory and Command messages to the AIoT devices via the intermediate UE reader without any impact on Rel-19 AIoT devices.

Proposal 3	RAN2 to discuss the handling of pending inventory reports during A-IoT enabled UE RLF.

Proposal 4	RAN2 to study solutions on how to perform resource allocations for AIoT-enabled UEs to avoid interference among them.

# 4 References

- RP-251885. New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2

- TR 38.769 Study on solutions for ambient IoT (Internet of Things) (Release 19

- S2-2507761, Architectural Assumptions

- TR 23.700-13, V19.0.0 (2025-03), Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)