3GPP TSG-RAN WG2 Meeting #131bis	R2-2507319

Prague, Czech Republic, Oct. 13th 17th 2025

Agenda item:	10.2 (General aspects)

Source:	Sony

Title:	Discussion on 6GR Rel-20 general aspects

Document for:	Discussion and Decision

# 1 Introduction

At RANP#108 the study item Study on 6G Radio was agreed [1].

In this paper, related to Agenda Item 10.2, General aspects, we discuss the Architecture/Design principles for 6G Radio, Signaling of UE Capabilities, AI Agents as a Service and Uplink-heavy mobile data traffic.

# 2 Discussion

## 2.1 Architecture/ design principles

6G architecture and design shall ensure that 4G IoT (both NB-IoT and eMTC) can coexist along with 6G and no changes are required to 4G. This should include 6G-IoT as 6G IoT and 4G IoT shall address different use cases which should be discussed at RANP level. From RAN2 point of view, there should be no requirements for service continuity between 6G IoT and 4G-IoT. Also, spectrum sharing should not impact 4G and it was agreed in RANP#109 that it should be semi static.

Proposal 1	No service continuity between 6G IoT and 4G IoT should be studied.

For RAN architecture, reuse of 5G CP/UP split design should be used as baseline. Further on RAN side, RANP#109 already agreed a text related to fronthaul interface and its relation to ORAN. For gNB split architecture, we think CU-DU split should be supported as in NR. In NR 7 different options for the split was studied. We do not think that all 7 split options discussed during NR design should be discussed for 6G, the number of studied options should be narrowed down. RAN3 should lead the task but RAN2 may assist by providing the information related to different protocol stacks which will be under RAN2 control.

Proposal 2	RAN2 should study CU-DU split architecture options in coordination with RAN3.

## 2.2 UE capability

Small size of UE capabilities is always preferred to avoid signalling overhead but on the other hand, large size allows UE capability flexibility. A right balance is needed between both extremes and we propose RAN2 to study UE capability size.

In 5G a UE capability update framework, based on UE Assistance Information, is defined. The UE Assistance Information may be sent by the UE when it prefers an adjustment of the parameters from the NW. This can be based on overheating, adjustment of DRX timing etc.

The UE capability framework in 5G is based on that the full UE Capability is sent at registration when the NW requests it, then it is stored in the UE Context in RAN and/or the Core Network. Thereby it is available in all RRC states and is moved between the nodes in the network when the UE moves. The problem with this solution is that the UE Capability is big, which makes it difficult to send from the UE to the NW at initial registration. This may in certain cases also delay the time until the UE is registered to the NW.

This needs to be solved in 6G since the capabilities will increase further as indicated above. Parts of the UE Capabilities may not be relevant for the actual network. Therefore possible solutions of the capability framework should be studied. One proposal is to send a minimum set of capabilities during registration and more detailed capabilities in different areas when the UE or NW triggers more information on need basis.

Due to the introduction of new features in 6GR, e.g., AI/ML, UE power saving, a UE may switch their preferences that are related to these new features in a more dynamic manner. For example, UE may switch between different AI/ML models according to its real-time data traffic or battery level, or a UE may switch to a different power saving plan according to its on-going service requirements. The characteristics of these preferences/information are as follows: 1) They are UE’s initiative. 2) They are potentially dynamic. Without these information being available to the network, the network may not be aware of UE’s preference/changing situation, which will lead to the degradation of user experience.

Proposal 3	RAN2 to study the size of UE capabilities and how to define a UE Capability framework which is more flexible and requires less signalling e.g. where e.g. only a smaller (minimum) set or of capabilities are sent initially, and the need to address potentially more dynamic changes of UE’s capability/preference, due to the introduction of new features.

## 2.3 Services

In the latest version of 6G use cases and service requirements TR 22.870 v0.4.0 in SA1 [2], more than 10 use cases on AI agents are agreed. For example, 6G network providing on-demand networking with AI agent, flexible UE-Network coordination through AI agents, AI agent management, network-based intelligent assistance (e.g., for autonomous driving) by a network-native AI Agent, etc. They can be classified into two categories: 6G network for 3rd party AI agent services and AI agent for 6G network. The AI agents can be deployed both at the UE side and network side.

In 5G networks, AI agents can be deployed as applications, which is transparent to RAN2 functions. For 6G network design to be AI-native (e.g., AI agents for network), the 6G RAN may shift from “application-agnostic” to “application-aware” or “application-collaborative” to support native AI agents services. Many use cases require collaboration between AI agents at UE-side and NW-side which need to ensure interoperability. This requires standardizing the mechanisms such as AI agent capability negotiation, AI agent context signaling, AI-specific bearer configuration, AI task scheduling for collaboration, AI agent mobility and performance monitoring.

Proposal 4:	RAN2 to Study UE-Network collaboration for/through AI agents

## 2.4 Uplink-heavy mobile data traffic

With the UE initiated/centric data applications getting more and more popular, e.g., online content creator, gaming, AI agent, some of the applications will put high requirements on the efficient delivery of uplink data traffic. In NR/LTE, downlink data delivery is of the utmost priority to enhance, e.g., the radio link quality measurement is based on the downlink, handover decision is based on the downlink performance, UE power saving enhancement is mainly on the downlink. On the other hand, there is need/technical gap to enhance the uplink data delivery especially for the uplink-heavy data traffic, due to the emerging new applications/use cases.

A relay, which is closer to the cell center and potentially controlled by network, has the benefit to reduce the uplink transmission power of its associated devices (which are located further to cell center) as well as to aggregate the data for an efficient uplink data delivery. But we think that non-3GPP interfaces other than D2D between peer devices should be supported and therefore, RAN2 should be involved to further study any impacts in the UE data aggregation for an efficient uplink-heavy data delivery.

Proposal 5:	RAN2 to study enhancements for uplink-heavy data services.

# 3 Conclusion

In this contribution, discussing the Architecture/Design principles for 6G Radio, Signalling of UE Capabilities, AI Agents as a Service and Uplink-heavy mobile data traffic the following is proposed:

Proposal 1	No service continuity between 6G IoT and 4G IoT should be studied.

Proposal 2	RAN2 should study CU-DU split architecture options.

Proposal 3	RAN2 to study the size of UE capabilities and how to define a UE Capability framework which is more flexible and requires less signalling e.g. where e.g. only a smaller (minimum) set or of capabilities are sent initially, and the need to address potentially more dynamic changes of UE’s capability/preference, due to the introduction of new features.

Proposal 4	RAN2 to Study UE-Network collaboration for/through AI agents

Proposal 5	RAN2 to study enhancements for uplink-heavy data services.

# 4 References

- RP-251881. New WID: Study on 6G Radio

- S1-253370. TR22.870 v0.4.0, Study on 6G Use Cases and Service Requirements; Stage 1(Release 20)