3GPP TSG-RAN WG2 Meeting #131bis	R2-2507320

Prague, Czech Republic, Oct. 13th-17th 2025

Agenda item:	10.3.1 (User plane)

Source:	Sony

Title:	Discussion on 6GR Rel-20 User plane aspects

Document for:	Discussion and Decision

# 1 Introduction

At RANP#108 the following study item was agreed with following objectives focused on user plane[1]:

- At RANP#108, the 6G study item was agreed [1], where mobility related part of the study item is captured as follows: Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3].

In this paper, we share our views on user plane protocol stack, handling of ACK type of traffic, and efficient use of user plane resource.

# 2 Discussion

## 2.1 User plane protocol stack

LTE user plane protocol stack has PDCP, RLC, and MAC sublayers. These sublayers were also present in NR protocol stack with the addition of SDAP protocol stack. Some detailed changes were introduced to each protocol layer while migrating from LTE to NR. We think that 6GR should reuse NR 5G UP and rel19 could be the baseline. Enhancements to each protocol layer based on the services need to be supported for 6G can be studied.

Apart from the protocol stack, QoS handling, number of DRBs and Logical channels should also be studied on top of NR. For example, RAN2 should study defining a realistic number of DRBs. Do we need to reserve LCID space for a number of DRBs which are never used etc. To begin with, NR rel-19 user plane should be the baseline.

Proposal 1	NR rel-19 user plane protocol stack should be the baseline for 6GR. RAN2 should study enhancements related to each protocol sublayer and other user plane topics depending on the type of services to be supported for 6G.

## 2.2 Handling of ACK type of traffic

6G will support different services but one aspect is the amount of traffic generated by ACK type of messages. This traffic can be generated at different layers like e.g. application-level TCP ACK or RLC-AM ACK or even PDCP status report. In order to reduce or optimise the ACK traffic, we see that there are two possible solutions on high level:

- No ack or ack correlation from other layer(s)

- Fake ack generation

For No ack or ack correlation from other layer, Asymmetric RLC configuration was discussed during rel-19. gNB will provide UL grant and UE will perform uplink transmission related to this UL grant. If gNB did not receive uplink RLC packet corresponding to an UL grant, then gNB can correlate between layers and ask for retransmission in HARQ layer. If cross layer ack correlation can work out then RLC-AM mode for uplink is not required and RLC-UM mode can be configured for uplink. For DL, RLC-AM is needed as it may be difficult to predict the reception at the UE.

For fake ack generation, instead of transmitting e.g. TCP ACK, a PDCP or RLC SN status report can be used over the air interface and a fake TCP ACK may be generated between PDCP and application layer for both UL and DL. For DL, UE PDCP layer generates ACK/NACK based on PDCP or RLC or HARQ status. For uplink, gNB may generate and send it to UPF like entity.

Proposal 2 RAN2 to study handling of ACK type traffic using cross layer coordination.

## 2.3 Adaptive header format

NR user plane is of SDAP, PDCP, RLC and MAC sublayers where each layer performs its functions, with specified e.g., PDU format, PDU header, MAC subheader. Take the RLC AMD PDU header as an example, it contains a D/C, a P, a SI, and a SN. It provides necessary control information regarding the RLC PDU while on the other hand, it introduces control signalling overhead over the air interface. Due to the changing characteristics of the radio link quality as well as on-going service requirements, together with newly introduced features, RAN2 should study the need for a more adaptive format by considering the trade-off between effective user plane PDU delivery and control signalling overhead reduction.

Proposal 3	RAN2 to study the need for adaptive user plane PDU format by considering the trade-off between effective user plane PDU delivery and control signalling overhead reduction.

## 2.4 Efficient Use of Resources

Scheduling is the heart of gNB functionality to realise the link adaptation and resource assignment in multi-user scenario. Dynamic DL/UL scheduling via DCI from gNB provides full flexibility of allocating resources and can handle well the traffic profile. But this results in significant control overhead and power consumption. SPS and CG scheduling are well-known schemes to reduce control overhead and power consumption for the periodic traffic with predictable packet sizes. However, in some cases, the traffic may not be exactly periodic and/or packet sizes are not predictable and/or there may be a jitter.  In addition, there are multiple services with different QoS requirements which resulted in 5G to design multiple CG resources. This leads inefficient use of the physical resources, for example, resources are allocated but may not be used for all the time. It should be possible for the UE to have some control of its scheduling assignments autonomously, probably, within the pre-configured uplink resources.

Based on the above discussion, it is recommended to study a new mode of UL scheduling scheme where the UE have some control of its scheduling assignments dynamically, probably, within the pre-configured uplink resources.

Proposal 4	 RAN2 to study efficient use of UL physical resources .

## 2.5 Multimodality QoS requirements

Furthermore, Immersive Reality is one of the main features driving and basis for 6G RAN requirement study. The QoS framework needs to be able to handle the increasing need and requirements for timely delivery of time critical communication and in particular packets belonging to the same XR service that supports interrelated multimodality streams, both in uplink and downlink direction. In 5G, RAN is aware of the streams which are interrelated but not? the interrelated QoS requirements for these streams. In order to handle strict multimodality, e.g., relative latency, requirements between interrelated data streams from the application, RAN2 should study whether the information of requirements for the interrelated data streams should be known by RAN.

Proposal 5	RAN2 to study RAN Awareness of the interrelated QoS requirements for the multimodality streams.

# 3 Conclusion

In this contribution, we propose RAN2 to discuss and agree to the following proposals:

Proposal 1	NR rel-19 user plane protocol stack should be the baseline for 6GR. RAN2 should study enhancements related to each protocol sublayer and other user plane topics depending on the type of services to be supported for 6G

Proposal 2	RAN2 to study handling of ACK type traffic using cross layer coordination

Proposal 3	RAN2 to study the need for adaptive user plane PDU format by considering the trade-off between effective user plane PDU delivery and control signalling overhead reduction

Proposal 4	RAN2 to study efficient use of UL physical resources

Proposal 5	RAN2 to study RAN Awareness of the interrelated QoS requirements for the multimodality streams

# 4 References

- RP-251881. New WID: Study on 6G Radio