3GPP TSG-RAN WG2 Meeting #131bis	R2-2507322

Prague, Czech Republic, Oct. 13th 17th 2025

Agenda item:	10.3.3 (Common UP & CP)

Source:	Sony

Title:	Discussion on 6GR Rel-20 Common UP & CP aspects

Document for:	Discussion and Decision

# 1 Introduction

At RANP#108 a study item was agreed, Study on 6G Radio [1]. with the following objective related to Protocol aspects.

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this paper, related to Agenda Item 10.3.3 Common User plane and Control plane, we elaborate briefly on Security for L1/L2, AIML and Sensing.

# 2 Discussion

## 2.1 Security of L1/L2

Main features of NR security architecture and key derivation:

- RRC and user plane ciphering and integrity protection

- gNB is not aware of keys used in previous or future gNBs (Next hop is known between UE and AMF)

- AS SMC procedure for RRC and UP security algorithms negotiations and RRC security activation.

- Idle to connected transition requires new AKA run

- AS security context is deleted during Connected to idle transition but kept during transition to inactive state.

- On mobility with vertical key derivation the NH, target PCI and its frequency ARFCN-DL are used. With horizontal key derivation the currently active KgNB is further bound to the target PCI and its frequency ARFCN-DL

In our understanding, no issues have been identified with above NR security architecture and key derivation principles and could serve as a starting point for 6G security.

However, lack of protection of L2/L1 signalling has been identified as an issue and raised a few times in the past. Some companies in RANP#109 assumed the involvement of RAN1 for L1 security while discussing an LS to SA on security [2]. However, RAN1 is not copied to this LS. A check point in June 26 for SA3 was suggested. We think that security of RLC and MAC control elements should be discussed in RAN2 in coordination with SA3. RAN1 may look at the security L1 signaling in coordination with SA3 depending on the progress of discussion in RANP and RAN1. RAN2 involvement or awareness of RAN1 work is FFS at this stage as there may be an attempt to harmonize the security mechanism for L2 and L1 signaling. The bottom line is that RAN2 should study the security of lower layer signaling along with other WGs

Proposal 1	RAN2 to study the security of lower layers signaling along with other WGs

## 2.2 AIML

The AI/ML functions in 5G-Advanced rely on offline training, where models are trained on large datasets in the cloud and then deployed to the network. While useful, this approach has its limitations in more advanced 6G use cases, such as Integrated Sensing and Communication (ISAC), AI agents and extreme mobility. To unlock the full potential of 6G to fulfill a continuous, real-time learning loop where AI models/agents (at the UE and gNB) exchange data to refine their models, online training framework (e.g. reinforcement learning) should be supported, where models adapt to the live network environment.

Both the UP based and CP based solutions for data collection have been studied in 5G-Advanced. A new SRBx with low priority for AI/ML data collection is agreed in Rel-19. This new SRB can satisfy the less time-sensitive data collection for offline training but may not satisfy the requirements for online training in 6G. 5G DRBs are tailored for the QoS needs of the user’s application. The 5G QoS framework is designed to differentiate between applications like "video," "voice," and "internet traffic." However, AL/ML training data has unique characteristics compared to traditional user plane data. The legacy user plane solution lacks the granularity to understand and prioritize different types of AI/ML data, such as distinguishing between raw sensor data, processed features, or model update gradients. A DRB is "data-agnostic", i.e. it treats all data within it the same. It cannot intelligently peek inside the data pipeline to perform in-network processing or aggregation. It doesn't understand the content or purpose of the data it carries. For AI/ML use cases such as AI agents, metadata (e.g., data source, timestamp, sensor type, feature vector, model gradients) is crucial for training, but 5G treats it as just another part of the payload. The design of 6G data plane can be semantic-aware: The data plane can be able to understand the meaning and intent of the data it carries. For example, it can distinguish between a raw image, a feature vector extracted from that image, or a model weight update, and then apply different policies for routing, caching, or prioritization. As billions of devices report data, the sheer volume of MDT information can be overwhelming for a network. The data transmission based on the control plane is not suitable for transmitting large amounts of data, leading to inefficient data transmission. The legacy measurement data collection is periodic or event-triggered, which may not satisfy the requirements of on-demand data retrieval of AI agents. We propose to study a new data collection framework for AI/ML functions considering the uniqueness of AI/ML traffic. Therefore, in addition to UP-based and CP-based data collection solutions, it is also suggested to evaluate whether a dedicated AI/ML data collection solution is beneficial.

Proposal 2	RAN 2 to study life cycle management to support online training, and to evaluate whether a dedicated AI/ML data collection solution is needed except for UP-based and CP-based solutions.

## 2.3 Sensing/ISAC

ISAC has been identified as one of the key features for 6G. In 5G-A, ISAC is currently being studied, including evaluating ISAC operation with gNB based mono-static sensing. In 6G, ISAC should consider all 6 ISAC modes as illustrated in Figure 1. The gNB based mono-static sensing in 5GA is to support sensing for UAV detection and tracking. Supporting all of these sensing modes would support sensing for various object types, such as human in indoor/outdoor, UAV, AGV, vehicles (e.g., cars, trucks), and object creating hazards (e.g., animal, human). 6G day-1 should consider the sensing modes that involve gNB and/or UE. An exception can be applied to the case of UE-UE bistatic that can be introduced later, such as in 6G day-2.

ISAC is commonly known to detect passive objects, including detecting the presence of object and its location. ISAC can also be used for environmental monitoring, in which the output can be used to assist communication. For example, a passive object may block the communication between UE and gNB. ISAC can be used to assist such communication, for example to assist beam operation/management and beam management. Another sensing operation is the usage of ISAC to support gesture recognition. This could enrich and improve the user experience of XR use-cases aiming to obtain immersive user experience. ISAC in 6G shall consider all of the above sensing operations.

.

Figure 1, Illustration of 6 different sensing modes

SA2 has started to discuss and evaluate architecture options and alternatives, which includes introducing some sort of management function, The initial study is addressing gNB monostatic sensing for drones [3] [23.700-14], but eventually UE based and bi-static solutions will be addressed. This will then not only affect RAN3, but also RAN2 and RAN1 for UE interaction. Hence, RAN2 should eventually study signaling framework towards the gNB and Sensing function, e.g., whether to select a Control plane or User plane based way forward

Proposal 3	RAN2 should eventually study an ISAC signaling/protocol framework between the UE and the gNB and Sensing function, e.g., whether to select a Control plane or User plane based way forward.

# 3 Conclusion

In this contribution we have made the following proposals related to Security for L1/L2, AIML and Sensing.

Proposal 1	RAN2 to study the security of lower layers signaling along with other WGs

Proposal 2	RAN 2 to study life cycle management to support online training, and to evaluate whether a dedicated AI/ML data collection solution is needed except for UP-based and CP-based solutions.

Proposal 3	RAN2 should eventually study an ISAC signaling/protocol framework between the UE and the gNB and Sensing function, e.g., whether to select a Control plane or User plane based way forward.

# 4 References

- RP-251881. New WID: Study on 6G Radio

- LS in RP-252891

- TR 23.700-14 Study on Integrated Sensing and Communication; Stage 2 (Release 20)