3GPP TSG-RAN WG2 Meeting #131bis	R2-2507323

Prague, Czech Republic, Oct. 13th - 17th 2025

Agenda item:	10.4 (Mobility)

Source:	Sony

Title:	Discussion on 6GR Rel-20 mobility aspects

Document for:	Discussion and Decision

# 1 Introduction

At RANP#108, the 6G study item was agreed [1], where mobility related part of the study item is captured as follows:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3].

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3].

In this paper, we discuss mobility related part of the 6G study item, covering RRM measurements, HO types and cell selection/reselection.

# 2 Discussion

## 2.1 RRM measurements

The Radio Resource Management (RRM) in the UE has since 3G time been developed jointly between RAN4, RAN2 and RAN1. The basis of the RRM framework has been stable, supporting both Cell selection/Reselection in RRC_Idle and RRC Inactive states and measurement reports to RAN for handover in RRC_Connected mode. The main idea is that the UE measures periodically on serving cell and when the quality of the serving cell degrades the UE measures on intra frequency and inter frequency cells respectively. Thereby the UE shall always be able camp on the best cell, at least when entering in connected mode.

RRM measurements consume a portion of UE power. Hence, the UE power consumption has been an important topic and RAN2 has introduced mechanisms like relaxed measurements for idle/inactive UEs in 5G NR. For connected mode RRM measurements, a UE normally relies on network configuration. For the same reason, the UE power consumption will be an important topic for 6G as well and there should be an effort to further reduce UE power consumption. We think there are at least two solution directions:

- DL RS in connected mode periodicity and its impacts on UE energy efficiency (EE) and performance

- UL RS measurements at the base station (EE) as discussed in the later section of this document.

DL RS periodicity and RS design should be discussed in RAN1 and RAN4 first, taking into account impacts on UE energy efficiency (EE) and performance.

Proposal 1, RAN2 should together with RAN4 and RAN1 study Energy Efficiency improvements in the RRM framework.

## 2.2 HO Types

L3 Handover: L3 handover procedures have been supported in 4G and 5G standards where the mobility of the connected mode UE is controlled by the network, as shown in Figure 1. The UE measures the signal quality/strength of the serving cell and neighbouring cells and then reports to the serving cell. Based on the measurements, the serving cell makes decision whether to handover the UE to one of the neighbouring cells termed as target cell. The gNB’s decision of sending handover message to the UE could be based on some predefined conditions, for example if the signal quality of the target cell is better than the signal quality of the serving cell. As soon as UE receives the handover command/RRCReconfiguration message, it synchronises and switches to the new cell by sending RRCReconfiguration complete message.

Figure 1. Basic Handover Procedure.

One of the drawbacks of the L3 handover is the long data interruption time due to poor radio link condition in some scenarios as the UE is mostly at the cell edge, which needs a number of retransmissions and even the radio link may sometimes be lost which results delays and significant overhead. The typical L3 handover latency is about 75ms which has significant impacts on some services like URLLC and XR. Regardless of drawbacks, in our view, L3 HO has been around for several generations, and it works well in the field, so it should be supported in 6G as well.

Conditional Handover (CHO): As said earlier, the UE is mostly at the cell edge when handover procedure is triggered which is not always in a favorite radio link condition. CHO has been specified in Rel-16 for 5G NR. Initially a UE provides preliminary measurement reports which identifies the potential target cells whilst still in good radio link condition with the serving cell, and handover preparations/configurations are performed for the UE with the identified potential target cells where some specific execution conditions are included in the configurations. The UE starts evaluating these conditions, and in a later stage if one or more conditions are met, the UE carries out HO execution and completion without instant signaling exchange with the current serving cell. This leads a more successful handover with lower chance of radio link failure.

L1L2 Triggered Mobility (LTM): With similar motivation for L3 CHO, to reduce the overhead and interruption time, LTM has been recently specified in Rel-18 and Rel-19 for 5G NR where L1/L2 signals and messages are employed to speed up the HO procedure, as shown in Figure 2. Initially a UE provides preliminary L3 measurement reports to the gNB where measurement reports identify the potential candidate cells. Then the gNB decides to configure LTM and initiates LTM preparation for multiple candidate cells for the UE. The UE makes DL and UL synchronizations with multiple candidate cells in advance. In a later stage, the UE provides L1 or L3 measurement reports to the gNB where measurement reports identify fewer candidate cells for handover. Based on these latest measurements, the gNB makes decision to handover the UE to a single target cell by issuing a cell switch command (CSC) signaled via a MAC CE. Then the UE applies the target cell’s LTM configuration according to the cell switch command and switches to the target cell. During cell switching, if valid timing advance (TA) value is available, the UE performs RACH-less cell switch, and if no valid TA value is available the UE performs RACH-based cell switch. The UE accesses the new cell by sending sending RRCReconfiguration complete message. In LTM, the interruption time is approximately reduced to 30ms.

Figure 2. LTM Procedures [2]

Conditional LTM (CLTM): CLTM cell has been developed in Rel-19 5G NR to achieve high robustness whereby the CLM procedure is executed without immediate signalling exchange between the UE and its serving cell. CLM is executed by the UE when L1-based or L3-based LTM cell switch execution conditions are met. CLTM offers shorter interruption time.

Uplink-based Mobility: Uplink-based mobility was somewhat introduced and discussed in 5G NR study item, although it was not specified at the end. Traditionally, the handover (HO) is triggered by a UE to transmit DL measurement report to the serving cell and then the serving cell decides to initiate the HO to the target cell. Another alternative is that a UE transmits UL Reference Signal to the target cell(s) and then the target cell sends measurement reports to the serving cell/source cell and source cell decides to initiate the handover procedure. This may mean that the UE transmits one or multiple Reference signals to multiple target cells. In this case, the network compares the measurements from different candidate cells and decides which TRP/cell(s) best serve the UE. Alternatively, the target cell can initiate a context fetch from serving cell and admit the UE in its cell straight away.

The scenario that this UL-based mobility is very beneficial is the dense network deployment which is intended to improve the capacity in dense urban areas as shown on Figure 3. In this scenario, the UL-based mobility would reduce power consumption at the UE due to decrease of DL measurements and reporting. The requirement from the network side is to coordinate and configure the UL Reference Signals to all possible candidate cells.

Figure 3: UL Mobility between different TRPs/cells.

In our view, UL-based mobility should be studied at least for connected mode as the network is able to coordinate the UL Reference Signals between the serving cell and candidate target cells. Note that there are already some UL signals that are coordinated between the serving cell and neighbouring cells, such as LTM RACH preamble coordination for UL synchronisation purpose, NES UL wake-up signals employed to wake-up NES cells and coordination of SRS configuration for positioning purpose. Hence, in 6G, these various uplink signals can be unified to one signal that serves to multiple purposes if possible.

In addition, legacy Inactive mode follows NR TS 38.304 but Inactive with small data transmission (SDT) might require UL mobility due to data transmission, so it could be studied as well.

Proposal 2. Rel-19 LTM should be the baseline for any HO work in 6G as LTM exhibited short interruption time and simple design. In addition,

- L3 HO has been around for many generations, and it works well in the field, in our view, L3 HO should be supported.

- UL-based mobility should be studied at least for connected mode mobility, FFS for Inactive mode.

NTN-TN tight integration: Fast switching between TN and NTN:

5G NTN does not support seamless handover from 5G NTN to TN and vice versa. Sharing of UE capabilities between TN and NTN is a technical problem. However, lack of coordination between TN and NTN MNOs is the cause. The main use case for 6G NTN, from our point of view, is to provide global coverage and NTN complementing TN coverage. To achieve this, a tight coordination is required between TN and NTN MNOs. NTN coverage can cover TN coverage holes as shown in Figure 4. We therefore propose that a tighter integration of NTN-TN should be studied.

Figure 4: NTN coverage covering TN coverage holes.

Proposal 3. RAN2 to study NTN-TN tighter integration.

## 2.3 Cell selection/reselection

Cell selection/reselection has a long history in all generations of cellular generation from NMT, and GSM, to WCDMA, LTE and NR. This will surely continue to be supported in the 6th generation. The procedure involves the UE performing measurement to make sure it is camping on a suitable cell, to be prepared to receive potential paging message. The procedure can be power consuming, and therefore different methods involving DRX and Power save states have been introduced to reduce the UE power consumption.

Nevertheless, are the current solutions that we have up to 5G NR the most optimal or are there any room for further improvements?

But related to the overall Energy Efficiency objectives, potentially further optimization can be discussed for 6G, e.g. reducing the UE effort/power consumption for RRM measurements in idle mode for a certain type of devices like e.g., IOT devices with predictable traffic, involving Low-Power wake up radio (LP-WUR) also for neighbor cell measurements.

Cell selection and cell reselection are performed by UEs in order to camp on a suitable cell in general and to camp on the best available cell when entering connected mode. The measurement on the serving cell is normally performed on every paging occasion, therefore it does not consume very much power. Searches for and measurements on neighbor cells are performed when the serving cell measurements are degrading.

In order to improve the Energy Efficiency (EE) in the UE, it has been proposed that the LP-WUS is used to send the paging indication to the UE, at least when in good coverage. If the LP-WUR is used by the UE for detecting paging, LP-WUR should also be used for doing the serving cell measurement when in good coverage. Otherwise, the normal receiver still needs to synchronize to the normal reference signals from the serving cell and the LP-WUS signalling would not improve the power consumption.

Proposal 4. RAN2 to study the energy efficiency enhancements like reduced RRM measurements in idle mode for certain device types, usage of LP-WUR for paging reception and serving cell measurements for cell reselection in idle and inactive mode.

# 3 Conclusion

In this contribution, we have discussed mobility related part of the 6G study item, covering RRM measurements, HO types and cell selection/reselection. We have the following proposals:

Proposal 1. RAN2 should together with RAN4 and RAN1 study Energy Efficiency improvements in the RRM framework.

Proposal 2. Rel-19 LTM should be the baseline for any HO work in 6G as LTM exhibited short interruption time and simple design. In addition,

- L3 HO has been around for many generations, and it works well in the field, in our view, L3 HO should be supported.

- UL-based mobility should be studied at least for connected mode mobility, FFS for Inactive mode.

Proposal 3. RAN2 to study NTN-TN tighter integration.

Proposal 4. RAN2 to study the energy efficiency enhancements like reduced RRM measurements in idle mode for certain device types, usage of LP-WUR for paging reception and serving cell measurements for cell reselection in idle and inactive mode.

# 4 References

- RP-251881, New WID: Study on 6G Radio, RAN#108.

- TS 38.300 V19.0.0