3GPP RAN WG2 Meeting #131bis	R2-2507324

Prague, Czech Republic, Oct 13th – 17th 2025

Agenda Item:	9.7.2

Source:	InterDigital

Title:	Consideration on IMT voice over IoT NTN

Document for:	Discussion, Decision

# Introduction

Support of IMT voice over ioT NTN is required in Rel-20. The corresponding WID [1] has the following objective.

-	Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2]

-	Coordination with SA is required

-	The objective can be updated with further details based on the outcome of the above down-selection

This contribution discusses about the down-selection between the CP and the UP solution.

# Discussion

One RAN plenary document [2] shows the comparison between the CP and the UP solutions.

According to [2], the following observations were shown.

|  | CP solution | UP solution |
| --- | --- | --- |
| Spec impact | - Introduce new dedicated SRB for IMS voice - Introduce one or two dedicated SRB to distinguish the legacy NAS message; - Support UM RLC mode for SRB carrying voice packet. - QoS info from CN is needed to guarantee GBR of the dedicated SRB, priority of new SRB should be specified - Introduce NB-IoT eNBcapability - indication in SI; | - Legacy DRB/QoS management for IMS voice in TN can be reused based on SA input; - Introduce NB-IoT eNBcapability indication in SI; |
|  | Observation: CP solution has more RAN2 impact. |  |
| Protocol overhead | C-plane (NAS + RRC) overhead (13 bytes) is always present per packet. Protocol overhead ratio: Legacy: 82.4% Optimised: 55.6% | Protocol overhead ratio: without RoHC: 78.6% with RoHC: 36.8% |
|  | Observation: CP solution has more protocol overhead. |  |

Observation1: CP solution has more RAN2 impact.

Observation2: CP solution has larger protocol overhead.

Proposal 1: RAN2 considers the spec impact and the protocol overhead to down-select between the CP and the UP solutions.

# Conclusion

In this contribution the following observations and proposal are made regarding the down-selection between the CP and the UP solutions.

Observation1: CP solution has more RAN2 impact.

Observation2: CP solution has larger protocol overhead.

Proposal 1: RAN2 considers the spec impact and the protocol overhead to down-select between the CP and the UP solutions.

# References

RP-252473	Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4	vivo

RP-251209	Support of voice call over GEO satellite based on NB IOT in 5G A Rel 20	vivo, CMCC, KT Corp Corp.,Verizon , Spreadtrum , UNISOC, ZTE Corporation, Sanechips