3GPP TSG-RAN WG2 Meeting #131bis	R2-2507332

Prague, Czech Republic, Oct. 13th-17th

Agenda item:		10.3.2

Source:	Rakuten

Title:	Discussion on Radio Protocol Architecture – Control Plane

Document for:		Discussion and Decision

# Introduction

The SID of Study on 6G Radio in RAN #108 includes the following objectives [1]

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this contribution, we present our analysis and views regarding the Control plane architecture and protocol design for 6GR.

# Discussion

## RRC Modelling

In our understanding, the scope of RRC modelling includes the design and structure of RRC states, messages, procedures, and interactions between the UE and the 6GNB including UE capability. It plays a central role in managing the radio connection, mobility, security, and QoS.

### Key Components of RRC Modeling in 5G

#### RRC States

- RRC_IDLE: UE is not connected to the gNB but can receive system information and perform cell (re)selection.

- RRC_INACTIVE: Bearers are released, UE maintains context at the gNB, but avoids retaining full connection, enabling fast resume to RRC_CONNECTED.

- RRC_CONNECTED: UE has an active connection with the gNB; used for UL/DL data transfer, mobility, and control signalling.

#### RRC Procedures

- Connection Establishment: UE transitions from IDLE/INACTIVE to CONNECTED.

- Connection Release: UE transitions back to IDLE or INACTIVE.

- Handover: Seamless mobility between cells/gNBs.

- Security Activation: Setup of encryption and integrity protection.

- Measurement Reporting: UE reports configured signal quality for mobility decisions.

- Configuration: Setup of DRBs, SRBs, QoS flows, etc.

#### RRC Messages

- RRC messages used to realize the RRC procedures

- Examples include:

  - RRCSetup, RRCSetupComplete

  - RRCReconfiguration, RRCRelease

  - MeasurementReport, RRCResume, RRCResumeComplete

We think that the 5G RRC modelling is effective in some of the above aspects and probably deserves some scrutiny in others like ASN.1 encoding/decoding and UE capability management.

5G RRC modelling is used as baseline for 6GR.

RRC modelling aspects that can be retained from 5G and RRC aspects that needs re-design in 6GR are identified in the beginning.

RRC modelling identified for re-design in 6GR is studied with the following objectives
* Efficiency: Reduce signaling overhead.  
* Flexibility: Support diverse services (eMBB, URLLC, mMTC).
* Mobility Optimization: Fast handovers and resume procedures.
* Security: Robust authentication and encryption.
* Context Management: Maintain UE context across states and transitions.

## RRC Protocol Architecture

The RRC (Radio Resource Control) protocol architecture in 5G NR is a key component of the control plane that manages the connection between the UE (User Equipment) and the gNB (Next Generation Node B). It operates above the PDCP (Packet Data Convergence Protocol) layer and interacts with both NAS (Non-Access Stratum) and lower layers like MAC and PHY.

Below provided is the list of important RRC functions.

#### RRC Layer Functions

- Connection Management: Establish, modify, and release RRC connections.

- Mobility Management: Handover, cell reselection, and beam management.

- Security Configuration: Ciphering and integrity protection setup.

- QoS Management: Configuration of QoS flows and DRBs.

- Measurement Control: Configure and report radio measurements.

- System Information Broadcast: MIB and SIB delivery.

- UE Capability Exchange: Communicate UE features to the network.

The 5G RRC protocol architecture is shown below

Figure 1: 5G RRC protocol architecture

Here, we would like to highlight that RLC, MAC and PHY protocols are controlled by the gNB-DU and hence has a big role in encoding RRC parameters necessary for any configuration. Though RRC parameter encoding is performed by the gNB-DU, the gNB-DU is not eligible to send RRC messages. This implies the following disadvantages

- Limited Control Over UE Signaling: This means the gNB-DU is more a passive entity, relying heavily on the Central Unit (CU) for control plane decisions, despite some of them being in the realm of gNB-DU managed functionalities. Some examples of this are RRC Reconfiguration involving parameters of gNB-DU only, like in an intra-cell or intra-DU scenario.

- Increased Latency: Without RRC at the DU, the handover decision-making is centralized, potentially slowing down the process and increasing the risk of dropped connections. For eg, in an intra-DU cell change based on L3 mobility, all the decisions have to be performed by the gNB-CU which adds additional delays. Similarly, in an intra-DU LTM candidate cell addition, gNB-CU involvement causes additional delays.

- Security and Authentication Delays: RRC handles security key exchanges and authentication procedures. Delaying these processes by routing them through the CU can expose the network to timing vulnerabilities or security risks. For eg, DU initiated UE context modification with no parameter from CU getting changed doesn’t require CU involvement. Therefore, RRC split without localised security is meaningless.

- Limited Local intelligence: DUs without RRC cannot make autonomous decisions based on local radio conditions. This limits the ability to implement edge intelligence. For eg, when UE sends an UL RRC message, even if the message is meant for gNB-DU, it has to be first decoded at gNB-CU and then forwarded as an F1 message to the gNB-DU. Depending on the scenario, the decision making could be localized at gNB-DU instead of always seeking permission from gNB-CU.

An alternative is an RRC protocol architecture design where RRC is split into RRC-High(H) and RRC-Low(L) and is associated with gNB-CU and gNB-DU respectively. In a split architecture, RRC-H could be located at CU and RRC-L located at DU, with a proper segregation of functionality based on the requirements and other functions performed by CU and DU.

Figure 2: Illustrative 6G RRC protocol architecture

Study split RRC architecture for 6GR with the objective of addressing the disadvantages of 5G RRC protocol architecture.

## Initial access and system information

The four steps of Initial access procedure are

- Cell Search and Synchronization,

- System Information Acquisition,

- Random Access Procedure (RACH)

- RRC Connection Establishment

A split RRC architecture introduced in the previous section has a definite bearing RRC Connection Establishment, if not the other steps. Hence, we propose to study how the RRC Connection Establishment is performed in the new protocol architecture.

Similarly, there could be some impacts on System Information encoding and broadcast as well. Therefore, we propose to include this also under the study.

Study the impact of split RRC architecture for 6GR on Initial Access and System Information management.

In 5G, SI is transmitted periodically and without encryption so that any device can receive it and initiate connection. Unsecured SI could lead to fake base station attacks, misleading configuration and denial of service. Hence, we propose to study security for SI in 6GR.

Study protection and encryption for system information to ensure that sensitive configuration data is protected without compromising network accessibility.

# Conclusion and Proposals

Based on the discussion above, we conclude this contribution with the following observations and proposals:.

Proposal 1	5G RRC modelling is used as baseline for 6GR.

Proposal 2	RRC modelling aspects that can be retained from 5G and RRC aspects that needs re-design in 6GR are identified in the beginning.

Proposal 3	RRC modelling identified for re-design in 6GR is studied with the following objectives
* Efficiency: Reduce signaling overhead.  
* Flexibility: Support diverse services (eMBB, URLLC, mMTC).
* Mobility Optimization: Fast handovers and resume procedures.
* Security: Robust authentication and encryption.
* Context Management: Maintain UE context across states and transitions.

Proposal 4	Study split RRC architecture for 6GR with the objective of addressing the disadvantages of 5G RRC protocol architecture.

Proposal 5	Study the impact of split RRC architecture for 6GR on Initial Access and System Information management.

Proposal 6	Study protection and encryption for system information to ensure that sensitive configuration data is protected without compromising network accessibility.

# References

- RP-251881. (n.d.). New SID: Study on 6G Radio. NTT Docomo.