3GPP TSG-RAN WG2 Meeting #131bis	R2-2507339

Prague, Czech Republic, Oct 13th - 17th, 2025

Agenda item:		8.18.3

Source:		Samsung

Title:			Consideration on cyclic shift for PMCH

Document for:	Discussion & Decision

# 1 Introduction

In this paper, we discuss on the need to specify the enhancement for the UE capability on the cyclic shift for PMCH.

# 2 Discussion

## In RAN1#122, the below agreement is reached specifying a third value alpha3 for the cyclic shift for PMCH.

Agreement The input to the cyclic shift is the block of bits b0, …, bMbit-1 defined in clause 6.3.1 and the ouput is a block of bits b0, …, bMbit-1. The block of bits b0, …, bMbit-1 shall be cyclically shifted by Xi bits to form the block b0, …, bMbit-1 according to bn=b(n-Xi mod Mbit) where - Xi is given by Xi=r=C-SiC-1Er Si=iα mod C - i∈0, 1, …, N-1 denotes the ith subframe to which the transport block is mapped - N is the number of subframes to which the transport block is mapped - Er is the number of bits in the rth codeblock within a subframe as defined in TS 36.212 [3] - C is the number of codeblocks in the transport block - α is given by - if the higher-layer parameter pmch-CyclicShiftAlpha is set to alpha1 α=1 - if the higher-layer parameter pmch-CyclicShiftAlpha is set to alpha2 α=CNL - if the higher-layer parameter pmch-CyclicShiftAlpha is set to alpha3 Xi= Ainf ,nSF×floor(NSC ×QmN), Ainf ,nSF ∈{0,…N-1} where Ainf ,nSF= m=07c8nf mod 12810+nSF+m2mmod N where nf is the system frame number and nSF is subframe number within the frame. The pseudo-random sequence ci is defined by clause 7.2 and shall be initialized with cinit=NIDMBSFN at the beginning of each radio frame for which nf mod 128=0, where NSC is the number of subcarriers available in 1 OFDM symbol for PMCH, NSC =Mbit/(QmL), Qm is the modulation order. - where L is the number of OFDM symbols used for the PMCH transmission.

Accordingly, there is a need to specify a UE capability for supporting “alpha3” for cyclic shift for PMCH in TS 36.306 and TS 36.331 specifications, quite inline for the UE capability previously specified for alpha1 and alpha2.

Proposal 1: Specify UE capability to support “alpha3” for cyclic shift for PMCH in TS 36.306 and TS36.331. Adopt TP1 and TP2 respectively.

# 3 Conclusion

Based on the discussion above, request RAN2 to consider the following proposal:

Proposal 1: Specify UE capability to support “alpha3” for cyclic shift for PMCH in TS 36.306 and TS36.331. Adopt TP1 and TP2 respectively.

# 4 Annex (Text Proposals)

TP1

TS 36.306 #### 4.3.17.14 pmch-cyclicShiftAlpha1-r19 This parameter defines, for the corresponding E-UTRA band, whether the UE supports cyclic shift of PMCH with alpha1 for the bit sequence in clause 6.3.1 of TS 36.211 [17] for the ith subframe of the time-interleaved TB by X_i bits. A UE which supports this feature shall also support timeInterleaving-r19 as specified in TS 36.331 [5]. #### 4.3.17.15 pmch-cyclicShiftAlpha2-r19 This parameter defines, for the corresponding E-UTRA band, whether the UE supports cyclic shift of PMCH with alpha2 for the bit sequence in clause 6.3.1 of TS 36.211 [17] for the ith subframe of the time-interleaved TB by X_i bits. A UE which supports this feature shall also support timeInterleaving-r19 as specified in TS 36.331 [5]. #### 4.3.17.16 pmch-cyclicShiftAlpha3-r19 This parameter defines, for the corresponding E-UTRA band, whether the UE supports cyclic shift of PMCH with alpha3 for the bit sequence in Section 6.3.1 of TS 36.211 for the ith subframe of the time-interleaved TB by X_i bits. A UE which supports this feature shall also support timeInterleaving-r19 as specified in TS 36.331 [5]. #### 4.3.17.17 freqInterleavingKhz15-r19, freqInterleavingKhz7dot5-r19, freqInterleavingKhz2dot5-r19, freqInterleavingKhz1dot25-r19 This parameter defines, for the corresponding E-UTRA band, whether the UE supports MBMS reception from an MBMS-dedicated cell configured with frequency interleaving shall support frequency-interleaving for MTCH for the corresponding PMCH numerology. A UE which supports this feature shall also support fembmsDedicatedCell-r14 as specified in TS 36.331 [5].

TP2

TS 36.331 MBMS-SupportedBandInfo-v1900 ::= SEQUENCE { cas-Muting-5GB-r19 ENUMERATED {supported} OPTIONAL, timeInterleaving-r19 SEQUENCE { timeInterleavingKhz15-r19 ENUMERATED {supported} OPTIONAL, timeInterleavingKhz7dot5-r19 ENUMERATED {supported} OPTIONAL, timeInterleavingKhz2dot5-r19 ENUMERATED {supported} OPTIONAL, timeInterleavingKhz1dot25-r19 ENUMERATED {supported} OPTIONAL } OPTIONAL, pmch-CyclicShiftAlpha1-r19 ENUMERATED {supported} OPTIONAL, pmch-CyclicShiftAlpha2-r19 ENUMERATED {supported} OPTIONAL, pmch-CyclicShiftAlpha3-r19 ENUMERATED {supported} OPTIONAL, freqInterleaving-r19 SEQUENCE { freqInterleavingKhz15-r19 ENUMERATED {supported} OPTIONAL, freqInterleavingKhz7dot5-r19 ENUMERATED {supported} OPTIONAL, freqInterleavingKhz2dot5-r19 ENUMERATED {supported} OPTIONAL, freqInterleavingKhz1dot25-r19 ENUMERATED {supported} OPTIONAL } OPTIONAL }