3GPP TSG-RAN WG2 Meeting #131bis	R2-2507342

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	8.7.3

Source:	OPPO

Title:	Discussion on PDCP open issues

Document for:	Discussion, Decision

Introduction

This paper presents our views on the open issues related to PDCP.

- [PDCP-H001]. Discuss whether to change the definition of “Non-delay-reporting PDCP SDU” so that each of a non-delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is exclusive.

Discussion

Regarding the definition of “Non-delay-reporting PDCP SDU”, some companies have expressed their concerns and prefer an update to align this definition with the corresponding procedure text.

Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the DSR-ReportingThreshold is a PDCP SDU that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold and that is not a delay-reporting or non-delay reporting PDCP SDU associated with any of the k:th DSR-ReportingThreshold where k<i.

When the discussion progressed, an alternative text was proposed by some proponents and received more support.

Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is a PDCP SDU that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold but after all delay-reporting PDCP SDUs associated with the i-1:th DSR-ReportingThreshold, and that is not a delay-reporting or non-delay reporting PDCP SDU associated with any of the k:th DSR-ReportingThreshold where k<=i.

However, we feel hesitant to implement the proposed change in PDCP for the following reasons,

- We do not identify any technical issues with following the logic of the current CR.

- Further discussions in RAN2 are necessary to determine whether the proposed update to the definition of “Non-delay-reporting PDCP SDU” is appropriate. For example, if the delay-reporting data exists for the i:th DSR-ReportingThreshold but not for the i-1:th DSR-ReportingThreshold, is the proposed alternative text still accurate?

- [PDCP-H001] keep the current definition of “Non-delay-reporting PDCP SDU” in PDCP (No spec change).

Conclusion

We have the following proposals:

Proposal 1	[PDCP-H001] keep the current definition of “Non-delay-reporting PDCP SDU” in PDCP (No spec change).

Reference

[1] RP-250107 Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur)