# 3GPP TSG-WG2 Meeting #131bis                                       			            R2-2507350

Prague, Czech Republic, Oct. 13th-17th

Source:                     Qualcomm Incorporated

Title:                         Discussion on RIL in LP-WUS RRC

Agenda Item:	8.4.2

Document for:	Discussion and Decision

# Introduction & Background

This contribution discusses leftover issues on RRC.

# Discussion

- RIL(H050): UAI for disabling LP-WUS

As discussed in IDLE/Inactive state, it is possible the LP-WUS coverage is smaller than MR coverage. In this case, UE should be able to fall back to PDCCH monitoring on MR using the configured C-DRX configuration.

Considering different LP-WUS options and different legacy PDCCH there are two fallback cases to be considered.

Case 1: UE is using LP-WUS option 1-1 and fallback to MR PDCCH monitoring with/without DCP configured

In this case, the UE is using LP-WUS option 1-1, when the UE detects moving outside of LP-WUS coverage, UE can automatically switch to legacy C-DRX PDCCH monitoring. There is no essential need for the network to know the exact moment that the UE autonomously fallback to PDCCH monitoring using C-DRX configuration. In option 1-1, the network anyway transmits both LP-WUS and PDCCH in the corresponding C-DRX active time. The UE monitors either LP-WUS or PDCCH in the corresponding C-DRX active time, depending on fallback status. Therefore, the network does not need to know the particular moment that the UE switches back to legacy C-DRX operation. However, it would be beneficial for network to know whether the UE is able to detect the LP-WUS or not, then gNB can determine whether to send LP-WUS any more.

Observation 1: When UE is using LP-WUS option 1-1 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB can stop sending LP-WUS indication and enable DCP functionality if configured.

Case 2: UE is using LP-WUS option 1-2 and fallback to MR PDCCH monitoring with/without DCP configured

For Option 1-2, if UE autonomous fallback to legacy C-DRX is enabled for Option 1-2, network does not know whether/when the UE leaves LP-WUS mode. There can be misalignment between the UE and the gNB when the UE monitors legacy PDCCH and gNB will continue to send LP-WUS indication and PDCCH in the new active time window, and then PDCCH could be missed. So in this case, UE should report to the gNB before switching to legacy PDCCH monitoring. And gNB should send command to the UE to switch to legacy PDCCH monitoring.

Observation 2: When UE is using LP-WUS option 1-2 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on PDCCH monitoring to avoid potential PDCCH missing.

Case 3: UE is using legacy PDCCH monitoring with/without DCP configured and enters LP-WUS mode

In this case, the UE is monitoring PDCCH as legacy, and UE detects entering LP-WUS coverage. Since for LP-WUS, gNB should firstly indicate to UE in LP-WUS and then send PDCCH on active time window. Then UE should report to gNB. And gNB should send switching command to the UE to enable LP-WUS. Then there is no misalignment between the gNB and the UE, and there is no potential PDCCH monitoring missed.

Observation 3: When UE is using legacy PDCCH monitoring and enters LP-WUS, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on LP-WUS enable to avoid potential PDCCH missing.

Considering the above cases, UE should indicate to gNB when enters or leave LP-WUS mode. The condition for entering or leaving LP-WUS mode could be based on MR measurement, can be left UE implementation.

- For both of Option 1-1 and Option 1-2, UE needs to indicate to the gNB when the UE enters or leave LP-WUS mode.

- For option 1-2, gNB needs to send a switching command to UE in order for UE switching from LP-WUS mode to legacy PDCCH monitoring with/without DCP.

- RIL(C026): Co-existence of LP-WUS in idle/inactive and paging adaptation

In C026, the co-existence of of LP-WUS in IDLE/Inactive state and paging adaptation was raised. RAN2 has not discussed this issue. Then it is worthy to be discussed.

In NES paging adaptation, the value of N and Ns are extended to increase the number of POs per PF with sparser PFs. The UE supporting paging adaptation shall monitor PDCCH in POs separately signalled for paging adaptation, if configured. The UE supporting paging adaptation and PEI can monitor PEIs separately signalled for paging adaptation, if configured. For LP-WUS occasion, UE applies one offset prior to the PO. That means, once the PO is determined regardless of using legacy PO or adapted PO, the LP-WUS occasion will always be the offset prior to the determined PO. Then LP-WUS and paging adaptation can be co-existence without specification change from RAN2 perspective.

- LP-WUS and paging adaptation can be co-existence without Stage-3 specification change from RAN2 perspective. It can be captured in Stage-2 specification.

# Conclusion

- For both of Option 1-1 and Option 1-2, UE needs to indicate to the gNB when the UE enters or leave LP-WUS mode.

- For option 1-2, gNB needs to send a switching command to UE in order for UE switching from LP-WUS mode to legacy PDCCH monitoring.

- LP-WUS and paging adaptation can be co-existence without Stage-3 specification change from RAN2 perspective. It can be captured in Stage-2 specification.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)