# 3GPP TSG-WG2 Meeting #131bis                                        			            R2-2507351

Prague, Czech Republic, Oct. 13th-17th

Source:                     Qualcomm Incorporated

Title:                         Remaining issues on LP-WUS MAC

Agenda Item:	8.4.3

Document for:	Discussion and Decision

# Introduction & Background

This contribution proposes to capture RAN2 agreement into specification.

# Discussion

In last RAN2 meeting, RAN2 agreed that if secondary DRX group is configured, UE monitors LP-WUS only when both DRX groups are not in DRX active time.

If secondary DRX group is configured, UE monitors LP-WUS only when both DRX groups are not in DRX active time.

However, in current MAC specification, this agreement is not captured. AS current specification, UE will monitor LP-WUS if not in Active Time, so the UE will monitor LP-WUS even the though the secondary DRX group is in Active Time. Then it should be clarified in MAC specification.

- Capture in MAC specification that UE monitors PDCCH on both DRX groups as legacy when any of the two DRX groups is in DRX active time, and TP is in Annex.

# Conclusion

- Capture in MAC specification that UE monitors PDCCH on both DRX groups as legacy when any of the two DRX groups is in DRX active time, and TP is in Annex.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

# Annex

## 5.7	Discontinuous Reception (DRX)

(omit)

1>	if LP-WUS monitoring is configured and the lpwus-PDCCH-MonitoringTimer for this DRX group is configured:

2>	if LP-WUS indication received from lower layer indicates to start lpwus-PDCCH-MonitoringTimer, as specified in TS 38.213 [6]:

3>	start lpwus-PDCCH-MonitoringTimer from the beginning of the subframe, as specified in TS 38.213 [6].

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the DRX duration.

NOTE 1c:	In case LP-WUS monitoring is configured and the lpwus-PDCCH-MonitoringTimer for this DRX group is configured, if UE is unable to monitor LP-WUS monitor occasion(s), UE does not start the lpwus-PDCCH-MonitoringTimer.

1> if secondary DRX group is configured:

2> if both DRX groups are not in Active Time:

5> indicates lower layer to monitor LP-WUS occasion(s), as specified in TS 38.213 [6].

1>	if a DRX group is in Active Time:

2>	monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission; or

2>	if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or

2>	if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]:

3>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

4>	if at least one of the corresponding HARQ process(es) is configured with HARQ feedback enabled:

5>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process(es) equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

5>	if the UE is configured with one-shot HARQ Feedback:

6>	start or restart the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process(es) whose HARQ feedback is enabled and reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

5>	else:

6>	start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.