3GPP TSG-RAN WG2 Meeting #131bis                                                               R2-2507353

Prague, Czech Republic, Oct. 13th-17th

Source:	Qualcomm Incorporated

Title:	Remaining issue on U2N multi-hop U2N relay control plane

Agenda Item:	8.13.2

Document for:	Discussion and Decision

# Introduction & Background

This contribution discusses the raiming issues of control plane.

# Discussion

In last RAN2 meeting, sl-PagingInfo-RemoteUE-List was introduced for the intermedirate relay UE to forward paging request from multiple child UEs. However the legacy IE sl-PagingInfo-RemoteUE could also be used by the intermediate relay UE when then there is only paging request from only child UE at the same time.

Then it is proposed that the intermediate Relay UE can include the received paging information into sl-PagingInfo-RemoteUE or sl-PagingInfo-RemoteUE-List. And it is up to intermediate Relay UE implementation to determine a paging information list in a paging request message or independant paging request messages are used when forwarding paging request for multiple child UEs.

- The intermediate Relay UE can include the received paging information into sl-PagingInfo-RemoteUE or sl-PagingInfo-RemoteUE-List based on intermediate relay UE implementation.

--------------------------------------------TP for Proposal 1-------------------------------------------------------------

1>	if the UE has paging related information to provide (e.g. the UE has not sent sl-PagingInfo-RemoteUE in the RemoteUEInformationSidelink message to the parent L2 U2N Relay UE before), set sl-PagingInfo-RemoteUE/sl-PagingInfo-RemoteUE-List as follows:

2>	if the L2 U2N Remote UE is in RRC_IDLE:

3>	include ng-5G-S-TMSI in the sl-PagingIdentityRemoteUE;

3>	if the UE specific DRX cycle is configured by upper layer, set sl-PagingCycleRemoteUE to the value of UE specific Uu DRX cycle configured by upper layer;

2>	else if the L2 U2N Remote UE is in RRC_INACTIVE:

3>	include ng-5G-S-TMSI and fullI-RNTI in the sl-PagingIdentityRemoteUE;

3>	if the UE specific DRX cycle is configured by upper layer,

4>	set sl-PagingCycleRemoteUE to the minimum value of UE specific Uu DRX cycles (configured by upper layer and configured by RRC);

3>	else:

4>	set sl-PagingCycleRemoteUE to the value of UE specific DRX cycle configured by RRC;

2>	if any paging information is received from the Child UE:

3>	include the received paging information in the sl-PagingInfo-RemoteUE/sl-PagingInfo-RemoteUE-List;

NOTE:	It is up to intermediate relay UE implementation to include the received paging information in the sl-PagingInfo-RemoteUE or sl-PagingInfo-RemoteUE-List.

----------------------------------------------------------------------------------------------------------------------------------

In service continuity, if the path switch is from multi-hop to multi-hop-hop, then the candidate relay UE have to be in connected state. Currently, remote UE will report all target paths regardless the candidate relay UE are in connected state or not, and it relies on gNB implementation to select the target path on which all the candidate relay UEs are connected state. gNB can determine whether the candidate relay UEs are in connected state based on received L2 ID of the intermediate relay UE. And gNB could know the target path is multi-hop path or single-hop path when the relay UEs on the target path are connected state.

However, if the path switch is from multi-hop to single-hop, then the candidate relay UE can be in IDLE/Inactive state. If the first intermediate relay UE is in IDLE/Inactive state, the gNB does not know whether the target path is single-hop or multi-hop U2N relay, and then the gNB cannot determine whether to select the target path.

Observation 1: As current spec, if the reported candidate relay UE in the target paths are in IDLE/Inactive state, the gNB does not know whether the target path is single-hop or multi-hop U2N relay.

So, in order to support path switch to single-hop U2N relay with candidate relay UEs in IDLE/Inactive state, Remote UE indicates the indication or hop number in measurement report to the gNB.

- Adding an indication or hop number in the measurement report to indicate whether the candidate Relay UE is on a single-hop or multi-hop target path.

# Conclusion

- The intermediate Relay UE can include the received paging information into sl-PagingInfo-RemoteUE or sl-PagingInfo-RemoteUE-List based on intermediate relay UE implementation.

Observation 1: As current spec, if the reported candidate relay UE in the target paths are in IDLE/Inactive state, the gNB does not know whether the target path is single-hop or multi-hop U2N relay.

- Adding an indication or hop number in the measurement report to indicate whether the candidate Relay UE is on a single-hop or multi-hop target path.

# Reference

[1]. RP-241609    New WID on NR sidelink multi-hop relay    LG Electronics, InterDigital, FirstNet