3GPP TSG RAN WG2 #131-bis	R2-2507357

Prague, Czechia, 13th – 17th October 2025

Agenda item	: 10.3.3

Source	: Jio Platforms Limited

Title	: Enhancements on New Type of Data in 6G

Document for	: Discussion

# Introduction

The new SID “Study on 6G Radio” [1] has been agreed in RAN#108 and revised [2] in RAN#109 meeting and objective 3 of the study item is as shown below:

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

(a) User Plane architecture and protocol design [RAN2]

(b) Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

(c) Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

(d) Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

(e) Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this contribution, we introduce AI/ML related control plane and data transfer design aspects that needs to be discussed in RAN2.

# Discussion

With the rapid integration of Artificial Intelligence (AI) and Machine Learning (ML) into modern communication networks, there is a growing need to support efficient, reliable, high-throughput, and low-latency data transfer mechanisms between User Equipment (UE) and the network infrastructure (RAN, CN, OAM etc.) to support the requirements. As AI/ML models are increasingly trained and refined using real-time or near-real-time data collection and validation at the UE, RAN, Core and any other third block entity need to have a proper transport mechanism to transfer those models whenever is needed by any entity in the network for any specific task, specifically the bearer configuration either default or dedicated bearer with assigned QoS flow with defined QoS profile to ensure the data transfer/delivery with high throughput, low latency, and reliability.

Figure 1 below illustrates the four different interfaces for different use-cases to send/receive the AI/ML specific data, sensing data etc. referred to as Special Data (SpD) between the UE and the Network.

The table below summarizes the categories and the data flow illustrated in the diagram above:

Table 1 - Summary of categories and the data flow units for special data types

| Scenario 1 | Data categories/Scenarios | Plane | Data Provider | Data Consumer |
| --- | --- | --- | --- | --- |
| 1 | AI/ML, sensing data is generated by an application in the UE | User Plane | UE | Application server |
| 2 | Data is destined for Network Data Analytics function (NWDAF) in Core Network e.g. Location Measurements similar to Sensing data | Control Plane | UE, AF(NWDAF) | UE, AF(NWDAF) |
| 3 | AI/ML, Sensing Data similar to Measurement Report generated at the RRC Layer | Control Plane | UE | RAN RAN Intelligent Controller (RIC) |
| 4 | AI/ML data generated at MAC Layer e.g. CSI data, Beam Management data etc. | Control Plane | UE | RAN Scheduler RAN Intelligent Controller (RIC) |

Figure 1 and Table 1 describe the framework for data transfer where UE has an active role and where provisions need to be made in 6GR radio specifications. However, there are other types of data such as performance counters accumulated at RAN, i.e., KPIs, or the traces generated by the RAN node which can also be used for various types of analytics and AI/ML model training. Such data will be transferred between RAN node and the external element like Radio Intelligent Controller (RIC) over other interfaces which are beyond RAN2’s domain of work.

With reference to Figure 1 and Table 1 above, we discuss transferring the Special Data (AIML Data, sensing etc.) that needs to be considered in 6G.

At present all the user plane data of a device maps to a DRB. DRB is mapped to LC IDs which are used for scheduling different kinds of data differently. In UL, currently the UE can send BSRs per Logical Channel Group (LCG) wherein the LCs within the group are configured by gNB.

This framework is well established for user data. If we use the same framework for special data, it may be difficult for schedulers to differentiate special data for AI/ML, sensing etc from the user data. Hence RAN2 should study about adding new type of Radio Bearer and corresponding Logical Channel which is used exclusively for special data. For use-cases involving AI/ML data transfer between UE and corresponding Application server, the existing data bearers would not be able to ensure the prioritization and reliability of such Data

For use-cases involving AI/ML data transfer between UE and corresponding Application server, the existing data bearers would not be able to ensure the prioritization and reliability of such Data.

RAN 2 should discuss introducing a dedicated special radio bearer (SpRB) in 6GR so that the AI/ML data between the UE and Application server with assigned priority where this data can be delivered with consistency and reliability of the data can be ensured.

An appropriate/New QoS Flow with suitable QoS profile (6QI value, Priority Level, Packet link budget, error rate), and mapping to the separate DRB can be defined as shown in below diagram to facilitate scheduling of this type of special data separately compared to the end-user application data.

Within the AI/ML framework, there are several use-cases where the exchange of Data is required between the UE and the Core Network (for example: the NWDAFD node). The gNB is not required to interact with such data. The current NAS architecture in 5G does not support the exchange of such AI/ML special data. Although this scenario is beyond the scope of RAN2, it is discussed as it is important in the overall framework of handling the transfer of special data such as AI/ML, Sensing etc.

In case of legacy Measurement Reports, the current measurement quantities (RSRP, RSRQ) may not be suitable to send sensing related data periodically. For such data, the Measurement Reports need to have a different set of report objects which will be transmitted over an RRC message. The use cases where special data (AIML, sensing) needs to be transported over RRC interface, there is no specified RRC procedure/Information in existing TS38.331 to support the RAN intelligence functions of these type of special data (AIML data, Sensing)

The use cases where special data (AIML, sensing) needs to be transported over RRC interface, there is no specified RRC procedure/Information in existing TS38.331 to support the RAN intelligence functions of these type of special data (AIML data, Sensing).

It is proposed that RAN2 should introduce the new AIML-specific RRC messages (e.g AI-ML-Config, AL-ML Data-report, AIML-control etc) or new Information elements inside existing RRC messages (Measurement Report for AI/ML, sensing data, RRC Reconfiguration, UE capability information) for transfer of special data between UE and RAN and further RAN can transfer these data to RAN intelligence entity RIC for Model/AIML function purpose.

The current MAC CE protocol does not have any suitable Control elements which can be utilized for exchange of this Special Data between the UE and the gNB MAC layer. The use cases where special data (AIML, sensing) needs to be transported between the UE and RAN MAC layer, there is no MAC CE in existing TS 38.321 to support the RAN intelligence functions of these type of special data (AIML data, Sensing).

The use cases where special data (AIML, sensing) needs to be transported between the UE and RAN MAC layer, there is no MAC CE in existing TS 38.321 to support the RAN intelligence functions of these type of special data (AIML data, Sensing).

RAN2 should discuss to introduce new MAC Control Elements (AIML-Report CEs) to efficiently transport AI-ML related short control payloads or measurement information( e.g enhanced CSI feedback, Beam Managements etc) between UE and 6GR RAN.

# Conclusion

Below is the summary of proposals and observations:

Observation 1:	 For use-cases involving AI/ML data transfer between UE and corresponding Application server, the existing data bearers would not be able to ensure the prioritization and reliability of such Data.

Proposal 1:	RAN 2 should discuss introducing a dedicated special radio bearer (SpRB) in 6GR so that the AI/ML data between the UE and Application server with assigned priority where this data can be delivered with consistency and reliability of the data can be ensured.

Observation 2: The use cases where special data (AIML, sensing) needs to be transported over RRC interface, there is no specified RRC procedure/Information in existing TS38.331 to support the RAN intelligence functions of this type of special data (AIML data, Sensing).

Proposal 2:	 It is proposed that RAN2 should introduce the new AIML-specific RRC messages (e.g AI-ML-Config, AL-ML Data-report, AIML-control etc) or new Information elements inside existing RRC messages (Measurement Report for AI/ML, sensing data, RRC Reconfiguration, UE capability information) for transfer of special data between UE and RAN and further RAN can transfer these data to RAN intelligence entity RIC for Model/AIML function purpose.

Observation 3: The use cases where special data (AIML, sensing) need to be transported between the UE and RAN MAC layer, there is no MAC CE in existing TS 38.321 to support the RAN intelligence functions of this type of special data (AIML data, Sensing).

Proposal 3: RAN2 should discuss introducing new MAC Control Elements (AIML-Report CEs) to efficiently transport AI-ML related short control payloads or measurement information (e.g., enhanced CSI feedback, Beam Managements etc.) between UE and 6GR RAN.

# References

3GPP RAN#108, RP-251881, New SID: Study on 6G Radio

3GPP RAN#109, RP-252912, Revised SID: Study on 6G Radio