3GPP TSG-RAN WG2 #131bis	R2-2507363

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	8.11.2

Source:	Ericsson

Title:	MAC remaining issues

Document for:	Discussion

# 1	Introduction

In this document, we discuss some remaining SBFD RA issues in MAC specification.

# 2	Discussion

## 2.1 Open Issue MAC-1

MAC-1: RO Type and RA Type Selections

Issue description (Ericsson):

Given the consensus that SBFD RO is not supported for 2-step RA, and the RO type selection is performed before RA type selection, there are largely two options in implementing the expected UE behaviour, i.e., UE selecting SBFD RO performs 4-step RA. Option 1) (as specified in SBFD MAC CR [1]) - Explicitly capturing UE behaviour for the case when RO type is set to SBFD RO, 4-step RA is selected, i.e., maintaining the RO type condition in RA selection (in yellow) captured in SBFD MAC CR [1]. CR#R2-2506606 5.1.1 Random Access procedure initialization … 1> if the RO_TYPE is set to 2nd-RO: - 2> set the RA_TYPE to 4-stepRA. …. Option 2) (proposed by Ericsson) - Relying on a proper network configuration (which ensures expected UE behaviour) without explicitly capturing the relevant UE behavior in MAC spec, i.e., removing the RO type condition in RA selection captured in SBFD MAC CR [1].

For this issue, it is a bad example to include “correction text” in the MAC spec for a misconfiguration given by the NW. The network will always ensure to provide a correct configuration to the UE, i.e., the network will ensure that 2-step RA and SBFD will not be present in the same cell/BWP.

The network will ensure that 2-step RA and SBFD will not be present in the same cell/BWP.

In addition, in stage 2, there is already text that says “Only the 4-step RA type using SBFD RACH resources can be supported”. Therefore, no additional spec change is really needed.

In stage 2, there is already text that says “Only the 4-step RA type using SBFD RACH resources can be supported”. Therefore, no additional spec change is really needed.

Relying on a proper network configuration (which ensures expected UE behaviour) without explicitly capturing the relevant UE behavior in MAC spec, i.e., removing the RO type condition in RA selection captured in the MAC spec.

## 2.2 Open Issue MAC-2

According to RAN1#120bis agreement captured below,

RAN1#120bis For determination of the Msg3 PUSCH transmission power for RACH configuration Option 2: - preambleReceivedTargetPower configured for legacy-RO is used if Msg3 PUSCH is transmitted in non-SBFD symbols; - preambleReceivedTargetPower configured for additional-RO is used if Msg3 PUSCH is transmitted in SBFD symbols;

the Rapporteur captured the corresponding UE behavior in SBFD MAC CR [1] by:

CR#R2-2506606 5.1.4 Random Access Response reception … 5> if the received UL grant indicates that the corresponding PUSCH transmission is in SBFD symbols as specified in clause 11.1 of TS 38.213 [6]: 6> if sbfd-RACH-SingleConfig (see TS 38.331 [5]) is configured for the Random Access procedure: 7> indicate the sbfd-RACH-SingleConfig-preambleReceivedTargetPower if configured, or the preambleReceivedTargetPower otherwise, and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE). 6> else if sbfd-RACH-DualConfig (see TS 38.331 [5]) is configured for the Random Access procedure: 7> indicate the preambleReceivedTargetPower included in the sbfd-RACH-DualConfig, and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE). 6> else: 7> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE). 5> else (i.e., the received UL grant indicates that the corresponding PUSCH transmission is in non-SBFD symbols as specified in clause 11.1 of TS 38.213 [6]): 6> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE). …

Note that, based on the RAN1 agreement, the preambleReceivedTargetPower indicated to lower layer for Msg3 tx power calculation solely depends on the type of symbols (SBFD or non-SBFD) the corresponding Msg3 is transmitted in, irrespective of the RO type used in preamble transmission, and hence, is not necessarily the same parameter (re)initialized for the preamble transmission(s).

From the offline discussions, companies’ views are summarized in the below

- five companies prefer negative statement ("not in xx"), considering the tricky point in explicitly indicating a commonly applicable parent IE for different contexts inclusively, and

- four companies prefer positive statement ("in yy"), for clearer and simpler UE implementation.

In our opinion, it is necessary to introduce additional changes. In this case, positive statement is preferred since it is a typical manner to capture things in specs.

For the case that Msg3 is transmitted in non-SBFD symbols and SBFD RACH config option 2 is configured, adopt the following TP:

5> else (i.e., the received UL grant indicates that the corresponding PUSCH transmission is in non-SBFD symbols as specified in clause 11.1 of TS 38.213 [6]):

- 6> indicate the preambleReceivedTargetPower included in rach-ConfigCommon and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE).

## 2.3 MAC-3: UE transmit power continuity during RO type switching

Issue description (Nokia)

During RAN2#131, companies agreed to compensate for the UE transmit power difference between legacy RO and SBFD RO by introducing a power offset: Proposal: For the RO type fallback between legacy RO and additional RO for RACH configuration option 2, a power offset given by the difference between the two quantities of preamble power ramping steps is added.. One of the arguments for supporting this power offset compensation is that the UE should not change its transmit power by a value higher than 6 dB between mgs1 transmissions, as described in TS 38.331: TS 38.331: powerRampingStep ENUMERATED {dB0, dB2, dB4, dB6},. By using this approach, the UE power continuity is ensured given that the preamble received target power is the same. However, in Nokia's view, the current proposal is not completely solving the UE transmit power continuity issue when switching between SBFD RO to legacy RO or vice versa. The reason is that current proposal only compensates for the potential differences in the power ramping step but doesn’t take into account the potential differences in the preamble received power. In Configuration 2, the UE will receive 2 independent RACH-ConfigCommon configurations, which among others, contain different preambleTargetReceivedPower and/or different powerRampingStep (among others). Therefore, the UE transmit power continuity is not ensured. This can lead to a larger power ramp-down or power ramp-up of 6 dB, which is not acceptable from UE vendors point of view as per last meeting’s discussion. Therefore, it is required to find a new way of compensating for both the differences in power ramping offsets and preamble received target powers.

For this issue, we think the issue is valid. However, the TP can be implemented differently than the TP proposed by Nokia. Nokia TP is implemented using negative statement. We think the TP shall be implemented using a positive statement.

Address the issue MAC-3 using positive statement.

We have also prepared TPs for all the three issues captured in clause 5.

Adopt the TPs captured in section 5.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	The network will ensure that 2-step RA and SBFD will not be present in the same cell/BWP.

Observation 2	In stage 2, there is already text that says “Only the 4-step RA type using SBFD RACH resources can be supported”. Therefore, no additional spec change is really needed.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Relying on a proper network configuration (which ensures expected UE behaviour) without explicitly capturing the relevant UE behavior in MAC spec, i.e., removing the RO type condition in RA selection captured in the MAC spec.

Proposal 2	For the case that Msg3 is transmitted in non-SBFD symbols and SBFD RACH config option 2 is configured, adopt the following TP

6> indicate the preambleReceivedTargetPower included in rach-ConfigCommon and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE).

Proposal 3	Address the issue MAC-3 using positive statement.

Proposal 4	Adopt the TPs captured in section 5.

# 4	References

- [1] 	RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei,

- Shanghai, China, June 17-20, 2024, RAN#104.

# 5	Text proposals

The below TPs are based on the draft TS 38.321 v19.0.0 (file Draft_38321-j00.docx) with changes not accepted.
Our proposed text changes are marked with Word comments.

## 5.1 Text proposal for issue MAC-1

### 5.1.1	Random Access procedure initialization

The Random Access procedure described in this clause is initiated by a PDCCH order, by the MAC entity itself, or by RRC for the events in accordance with TS 38.300 [2]. There is only one Random Access procedure ongoing at any point in time in a MAC entity. The Random Access procedure on an SCell or an LTM candidate cell shall only be initiated by a PDCCH order with ra-PreambleIndex different from 0b000000.

NOTE 1:	If a new Random Access procedure is triggered while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request).

NOTE 2:	If there was an ongoing Random Access procedure that is triggered by a PDCCH order while the UE receives another PDCCH order for the same serving cell indicating the same Random Access Preamble, PRACH mask index and uplink carrier, the Random Access procedure is considered as the same Random Access procedure as the ongoing one and not initialized again.

When a Random Access procedure is initiated, UE selects a set of Random Access resources as specified in clause 5.1.1b and initialises the following parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources:

-	prach-ConfigurationIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble for Msg1. These are also applicable to the MSGA PRACH if the PRACH occasions are shared between 2-step and 4-step RA types;

-	prach-ConfigurationPeriodScaling-IAB: the scaling factor defined in TS 38.211 [8] and applicable to IAB-MTs, extending the periodicity of the PRACH occasions baseline configuration indicated by prach-ConfigurationIndex;

-	prach-ConfigurationFrameOffset-IAB: the frame offset defined in TS 38.211 [8] and applicable to IAB-MTs, altering the ROs frame defined in the baseline configuration indicated by prach-ConfigurationIndex;

-	prach-ConfigurationSOffset-IAB: the subframe/slot offset defined in TS 38.211 [8] and applicable to IAB-MTs, altering the ROs subframe or slot defined in the baseline configuration indicated by prach-ConfigurationIndex;

-	msgA-PRACH-ConfigurationIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble for MSGA in 2-step RA type;

-	preambleReceivedTargetPower: initial Random Access Preamble power for 4-step RA type;

-	sbfd-RACH-SingleConfig-preambleReceivedTargetPower: initial Random Access Preamble power for 4-step RA type associated with the second PRACH occasions as defined in TS 38.213 [6];

-	msgA-PreambleReceivedTargetPower: initial Random Access Preamble power for 2-step RA type;

-	rsrp-ThresholdSSB: an RSRP threshold for the selection of the SSB for 4-step RA type. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdSSB used for the selection of the SSB within candidateBeamRSList refers to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE;

-	rsrp-ThresholdCSI-RS: an RSRP threshold for the selection of CSI-RS for 4-step RA type. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdCSI-RS is equal to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE;

-	msgA-RSRP-ThresholdSSB: an RSRP threshold for the selection of the SSB for 2-step RA type;

-	rsrp-ThresholdSSB-SUL: an RSRP threshold for the selection between the NUL carrier and the SUL carrier;

-	msgA-RSRP-Threshold: an RSRP threshold for selection between 2-step RA type and 4-step RA type when both 2-step and 4-step RA type Random Access Resources are configured in the UL BWP;

-	rsrp-ThresholdMsg1-RepetitionNum2: an RSRP threshold for Msg1 repetition with repetition number 2 (see clause 5.1.1b);

-	rsrp-ThresholdMsg1-RepetitionNum4: an RSRP threshold for Msg1 repetition with repetition number 4 (see clause 5.1.1b);

-	rsrp-ThresholdMsg1-RepetitionNum8: an RSRP threshold for Msg1 repetition with repetition number 8 (see clause 5.1.1b);

-	sbfd-RSRP-ThresholdMsg1-RepetitionNum2: an RSRP threshold for Msg1 repetition with repetition number 2 associated with the second PRACH occasions as defined in TS 38.213 [6] (see clause 5.1.1b);

-	sbfd-RSRP-ThresholdMsg1-RepetitionNum4: an RSRP threshold for Msg1 repetition with repetition number 4 associated with the second PRACH occasions as defined in TS 38.213 [6] (see clause 5.1.1b);

-	sbfd-RSRP-ThresholdMsg1-RepetitionNum8: an RSRP threshold for Msg1 repetition with repetition number 8 associated with the second PRACH occasions as defined in TS 38.213 [6] (see clause 5.1.1b);

-	rsrp-ThresholdMsg3: an RSRP threshold for Msg3 repetition (see clause 5.1.1b);

-	sbfd-RSRP-ThresholdRO-Type: an RSRP threshold for the selection of the initial RO type between the first PRACH occasions and the second PRACH occasions as defined in TS 38.213 [6] in contention-based Random Access procedure;

-	sbfd-RSRP-ThresholdRO-TypeUsage: indicates how sbfd-RSRP-ThresholdRO-Type is used in initial RO type selection;

-	sib1-rsrp-ThresholdSSB: an RSRP threshold for the selection of the SSB for SIB1 request.

-	FeatureCombination: feature or a combination of features associated with a set of Random Access resources;

-	featurePriorities: priorities for features, such as (e)RedCap, Slicing, etc. (see clause 5.1.1d);

-	msgA-TransMax: The maximum number of MSGA transmissions when both 4-step and 2-step RA type Random Access Resources are configured;

-	candidateBeamRSList: a list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated Random Access parameters;

-	recoverySearchSpaceId: the search space identity for monitoring the response of the beam failure recovery request;

-	powerRampingStep: the power-ramping factor;

-	msgA-PreamblePowerRampingStep: the power ramping factor for MSGA preamble;

-	powerRampingStepHighPriority: the power-ramping factor in case of prioritized Random Access procedure;

-	scalingFactorBI: a scaling factor for prioritized Random Access procedure;

-	ra-PreambleIndex: Random Access Preamble;

-	ra-ssb-OccasionMaskIndex: defines PRACH occasion(s) associated with an SSB in which the MAC entity may transmit a Random Access Preamble (see clause 7.4);

-	msgA-SSB-SharedRO-MaskIndex: Indicates the subset of 4-step RA type PRACH occasions shared with 2-step RA type PRACH occasions for each SSB. If 2-step RA type PRACH occasions are shared with 4-step RA type PRACH occasions and msgA-SSB-SharedRO-MaskIndex is not configured, then all 4-step RA type PRACH occasions are available for 2-step RA type (see clause 7.4);

-	ssb-SharedRO-MaskIndex: defines PRACH occasions, on which preambles are allocated for a feature or a combination of features, associated with an SSB in which the MAC entity may transmit a Random Access Preamble (see clause 7.4);

-	ra-OccasionList: defines PRACH occasion(s) associated with a CSI-RS in which the MAC entity may transmit a Random Access Preamble;

-	ra-PreambleStartIndex: the starting index of Random Access Preamble(s) for on-demand SI request;

-	sib1-RA-PreambleStartIndex: the starting index of Random Access Preamble(s) for SIB1 request;

-	startPreambleForThisPartition: the first preamble associated with the set of Random Access Resources applicable to the Random Access procedure;

-	preambleTransMax: the maximum number of Random Access Preamble transmission;

-	preambleTransMax-Msg1-Repetition: the maximum number of Random Access Preamble transmissions with a given Msg1 repetition number before switching to Msg1 repetition with the next available higher Msg1 repetition number;

-	preambleTransMaxRO-Type: the maximum number of Random Access Preamble transmissions before switching RO type between the first PRACH occasions and the second PRACH occasions as defined in TS 38.213 [6];

-	ssb-perRACH-OccasionAndCB-PreamblesPerSSB: defines the number of SSBs mapped to each PRACH occasion for 4-step RA type and the number of contention-based Random Access Preambles mapped to each SSB;

-	msgA-CB-PreamblesPerSSB-PerSharedRO: defines the number of contention-based Random Access Preambles for 2-step RA type mapped to each SSB when the PRACH occasions are shared between 2-step and 4-step RA types;

-	msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB: defines the number of SSBs mapped to each PRACH occasion for 2-step RA type and the number of contention-based Random Access Preambles mapped to each SSB;

-	numberOfPreamblesPerSSB-ForThisPartition: defines the number of consecutive preambles for a feature or a combination of features mapped to each SSB;

-	msgA-PUSCH-ResourceGroupA: defines MSGA PUSCH resources that the UE shall use when performing MSGA transmission using Random Access Preambles group A;

-	msgA-PUSCH-ResourceGroupB: defines MSGA PUSCH resources that the UE shall use when performing MSGA transmission using Random Access Preambles group B;

-	msgA-PUSCH-Resource-Index: identifies the index of the PUSCH resource used for MSGA in case of contention-free Random Access with 2-step RA type;

-	if groupBconfigured is configured, then Random Access Preambles group B is configured for 4-step RA type.

-	Amongst the contention-based Random Access Preambles associated with an SSB (as defined in TS 38.213 [6]), the first numberOfRA-PreamblesGroupA included in groupBconfigured Random Access Preambles belong to Random Access Preambles group A. The remaining Random Access Preambles associated with the SSB belong to Random Access Preambles group B (if configured).

-	if groupB-ConfiguredTwoStepRA is configured, then Random Access Preambles group B is configured for 2-step RA type.

-	Amongst the contention-based Random Access Preambles for 2-step RA type associated with an SSB (as defined in TS 38.213 [6]), the first numberOfRA-PreamblesGroupA included in GroupB-ConfiguredTwoStepRA Random Access Preambles belong to Random Access Preambles group A. The remaining Random Access Preambles associated with the SSB belong to Random Access Preambles group B (if configured).

NOTE 3:	If Random Access Preambles group B is supported by the cell Random Access Preambles group B is included for each SSB.

-	if Random Access Preambles group B is configured for 4-step RA type:

-	ra-Msg3SizeGroupA: the threshold to determine the groups of Random Access Preambles for 4-step RA type;

-	msg3-DeltaPreamble: ∆PREAMBLE_Msg3 in TS 38.213 [6];

-	messagePowerOffsetGroupB: the power offset for preamble selection included in groupBconfigured;

-	numberOfRA-PreamblesGroupA: defines the number of Random Access Preambles in Random Access Preamble group A for each SSB included in groupBconfigured.

-	if Random Access Preambles group B is configured for 2-step RA type:

-	msgA-DeltaPreamble: ∆MsgA_PUSCH in TS 38.213 [6];

-	messagePowerOffsetGroupB: the power offset for preamble selection included in GroupB-ConfiguredTwoStepRA;

-	numberOfRA-PreamblesGroupA: defines the number of Random Access Preambles in Random Access Preamble group A for each SSB included in GroupB-ConfiguredTwoStepRA;

-	ra-MsgA-SizeGroupA: the threshold to determine the groups of Random Access Preambles for 2-step RA type.

-	the set of Random Access Preambles and/or PRACH occasions for SI request, if any;

-	the set of Random Access Preambles and/or PRACH occasions for SIB1 request, if any;

-	the set of Random Access Preambles and/or PRACH occasions for beam failure recovery request, if any;

-	the set of Random Access Preambles and/or PRACH occasions for reconfiguration with sync, if any;

-	ra-ResponseWindow: the time window to monitor RA response(s) (SpCell only);

-	ra-ContentionResolutionTimer: the Contention Resolution Timer (SpCell only);

-	msgB-ResponseWindow: the time window to monitor RA response(s) for 2-step RA type (SpCell only).

In addition, the following information for related Serving Cell is assumed to be available for UEs:

-	if Random Access Preambles group B is configured:

-	if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5], and SUL carrier is selected for performing Random Access Procedure:

-	PCMAX,f,c of the SUL carrier as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16].

-	else:

-	PCMAX,f,c of the NUL carrier as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16].

The following UE variables are used for the Random Access procedure:

-	PREAMBLE_INDEX;

-	PREAMBLE_TRANSMISSION_COUNTER;

-	PREAMBLE_POWER_RAMPING_COUNTER;

-	PREAMBLE_POWER_RAMPING_STEP;

-	PREAMBLE_RECEIVED_TARGET_POWER;

-	PREAMBLE_BACKOFF;

-	PCMAX;

-	SCALING_FACTOR_BI;

-	TEMPORARY_C-RNTI;

-	RA_TYPE;

-	POWER_OFFSET_2STEP_RA;

-	MSGA_PREAMBLE_POWER_RAMPING_STEP;

-	RO_TYPE;

-	POWER_OFFSET_RO_TYPE;

-	PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP.

When the Random Access procedure is initiated on a Serving Cell or for an LTM candidate cell, the MAC entity shall:

1>	flush the Msg3 buffer;

1>	flush the MSGA buffer;

1>	set the PREAMBLE_TRANSMISSION_COUNTER to 1;

1>	if the Random Access procedure is initiated on a Serving Cell; or

1>	if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell and the PDCCH order indicates preamble initial transmission; or

1>	if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell, which is different from the cell to which the MAC entity performed the last Random Access Preamble transmission, and the PDCCH order indicates preamble re-transmission:

2>	set the PREAMBLE_POWER_RAMPING_COUNTER to 1;

1>	set the PREAMBLE_BACKOFF to 0 ms;

1>	set POWER_OFFSET_2STEP_RA to 0 dB;

1>	set POWER_OFFSET_RO_TYPE to 0 dB;

1>	if the carrier to use for the Random Access procedure is explicitly signalled:

2>	select the signalled carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the signalled carrier.

1>	else if the carrier to use for the Random Access procedure is not explicitly signalled; and

1>	if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5]; and

1>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

2>	select the SUL carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the SUL carrier.

1>	else:

2>	select the NUL carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the NUL carrier.

NOTE 4:	Void.

1>	perform the BWP operation as specified in clause 5.15, except when the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell;

1>	if the Random Access procedure is initiated by PDCCH order and if the ra-PreambleIndex explicitly provided by PDCCH is not 0b000000 and if the RACH occasion indicator is set to 1 (as specified in TS 38.212 [9]); or

1>	if the Random Access procedure was initiated for SpCell beam failure recovery (as specified in clause 5.17) and if the contention-free Random Access Resources for beam failure recovery request for 4-step RA type have been explicitly provided by RRC for the BWP selected for Random Access procedure and if the ra-OccasionType is set to sbfd for the Random Access procedure (as specified in TS 38.331 [5]); or

1>	if the Random Access procedure was initiated for reconfiguration with sync not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3 and if the contention-free Random Access Resources for 4-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure and if the ra-OccasionType is set to sbfd for the Random Access procedure (as specified in TS 38.331 [5]):

2>	set the RO_TYPE to 2nd-RO.

1>	else if neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure and either sbfd-RACH-SingleConfig or sbfd-RACH-DualConfig is configured by RRC for the Random Access procedure (as specified in TS 38.331 [5]):

2>	if the sbfd-RO-Type is set to sbfd for the Random Access procedure (as specified in TS 38.331 [5]):

3>	set the RO_TYPE to 2nd-RO.

2>	else if the sbfd-RO-Type is set to non-sbfd for the Random Access procedure (as specified in TS 38.331 [5]):

3>	set the RO_TYPE to 1st-RO.

2>	else if the sbfd-RO-Type is not configured for the Random Access procedure:

3>	if sbfd-RSRP-ThresholdRO-Type and sbfd-RSRP-ThresholdRO-TypeUsage are configured for the Random Access procedure (see TS 38.331 [5]):

4>	if the RSRP of the downlink pathloss reference is below sbfd-RSRP-ThresholdRO-Type, and sbfd-RSRP-ThresholdRO-TypeUsage is set to below (as specified in TS 38.331 [5]); or

4>	if the RSRP of the downlink pathloss reference is above sbfd-RSRP-ThresholdRO-Type, and sbfd-RSRP-ThresholdRO-TypeUsage is set to above (as specified in TS 38.331 [5]):

5>	set the RO_TYPE to 2nd-RO.

4>	else:

5>	set the RO_TYPE to 1st-RO.

NOTE 5:	If sbfd-RO-Type, sbfd-RSRP-ThresholdRO-Type, and sbfd-RSRP-ThresholdRO-TypeUsage are not configured for the Random Access procedure, it is up to UE implementation how to set the RO_TYPE between 1st-RO and 2nd-RO as the initial RO type for the Random Access procedure.

1>	else:

2>	set the RO_TYPE to 1st-RO.

1>	select the set of Random Access resources applicable to the current Random Access procedure according to clause 5.1.1b;

1>	if the Random Access procedure is initiated by PDCCH order and if the ra-PreambleIndex explicitly provided by PDCCH is not 0b000000; or

1>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC; or

1>	if the Random Access procedure was initiated for SpCell beam failure recovery (as specified in clause 5.17) and if the contention-free Random Access Resources for beam failure recovery request for 4-step RA type have been explicitly provided by RRC for the BWP selected for Random Access procedure; or

1>	if the Random Access procedure was initiated for reconfiguration with sync not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3 and if the contention-free Random Access Resources for 4-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure; or

1>	if the contention-free Random Access Resources have been explicitly provided in the (Enhanced) LTM Cell Switch Command MAC CE;

or

1>	if the Random Access procedure was initiated for SIB1 request (as specified in TS 38.331 [5]) and the Random Access Resources for SIB1 request have been provided by RRC:

2>	set the RA_TYPE to 4-stepRA.

1>	else if the BWP selected for Random Access procedure is configured with both 2-step and 4-step RA type Random Access Resources within the selected set of Random Access resources (as specified in clause 5.1.1b) and the RSRP of the downlink pathloss reference is above msgA-RSRP-Threshold; or

1>	if the BWP selected for Random Access procedure is only configured with 2-step RA type Random Access resources within the selected set of Random Access resources according to clause 5.1.1b; or

1>	if the Random Access procedure was initiated for reconfiguration with sync not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3 and if the contention-free Random Access Resources for 2-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure:

2>	set the RA_TYPE to 2-stepRA.

1>	else:

2>	set the RA_TYPE to 4-stepRA.

1>	perform initialization of variables specific to Random Access type as specified in clause 5.1.1a;

1>	if RA_TYPE is set to 2-stepRA:

2>	perform the Random Access Resource selection procedure for 2-step RA type (see clause 5.1.2a).

1>	else:

2>	perform the Random Access Resource selection procedure (see clause 5.1.2).

## 5.2 Text proposal for issue MAC-2

### 5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted on a non-terrestrial network:

3>	start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the PDCCH occasion as specified in TS 38.213 [6].

2>	else:

3>	start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission.

2>	monitor for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while ra-ResponseWindow is running.

1>	else:

2>	if the Random Access Preamble was transmitted on a non-terrestrial network:

3>	if the Random Access Preamble is transmitted with repetitions:

4>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the PDCCH occasion from the end of all repetitions of the Random Access Preamble transmission as specified in TS 38.213 [6].

3>	else:

4>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the PDCCH occasion as specified in TS 38.213 [6].

2>	else if the Random Access Preamble is transmitted with repetitions:

3>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion from the end of all repetitions of the Random Access Preamble transmission as specified in TS 38.213 [6].

2>	else if the Random Access Preamble is transmitted for SIB1 request:

3>	start the ra-ResponseWindow configured in RACH-ConfigSIB1 at the first PDCCH occasion from the end of the Random Access Preamble transmission as specified in TS 38.213 [6].

2>	else:

3>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission.

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running.

1>	if notification of a reception of a PDCCH transmission on the search space indicated by recoverySearchSpaceId is received from lower layers on the Serving Cell where the preamble was transmitted; and

1>	if PDCCH transmission is addressed to the C-RNTI; and

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	consider the Random Access procedure successfully completed.

1>	else if a valid (as specified in TS 38.213 [6]) downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded:

2>	if the Random Access Response contains a MAC subPDU with Backoff Indicator:

3>	if the Random Access procedure was initiated for SIB1 request:

4>	set the PREAMBLE_BACKOFF to 0 ms.

3>	else:

4>	set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI.

2>	else:

3>	set the PREAMBLE_BACKOFF to 0 ms.

2>	if the Random Access Response contains a MAC subPDU with Random Access Preamble identifier corresponding to the transmitted PREAMBLE_INDEX (see clause 5.1.3):

3>	consider this Random Access Response reception successful.

2>	if the Random Access Response reception is considered successful:

3>	if the Random Access Response includes a MAC subPDU with RAPID only:

4>	consider this Random Access procedure successfully completed;

4>	if the Random Access procedure was initiated for SIB1 request:

5>	indicate the reception of an acknowledgement for SIB1 request to upper layers.

4>	else if the Random Access procedure was initiated for SI request:

5>	indicate the reception of an acknowledgement for SI request to upper layers.

3>	else:

4>	apply the following actions for the Serving Cell where the Random Access Preamble was transmitted:

5>	process the received Timing Advance Command (see clause 5.2);

5> if the received UL grant indicates that the corresponding PUSCH transmission is in SBFD symbols as specified in clause 11.1 of TS 38.213 [6]:

6>	if sbfd-RACH-SingleConfig (see TS 38.331 [5]) is configured for the Random Access procedure:

7>	indicate the sbfd-RACH-SingleConfig-preambleReceivedTargetPower if configured, or the preambleReceivedTargetPower otherwise, and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE).

6>	else if sbfd-RACH-DualConfig (see TS 38.331 [5]) is configured for the Random Access procedure:

7>	indicate the preambleReceivedTargetPower included in the sbfd-RACH-DualConfig, and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE).

6>	else:

7>	indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE).

5>	else (i.e., the received UL grant indicates that the corresponding PUSCH transmission is in non-SBFD symbols as specified in clause 11.1 of TS 38.213 [6]):

6>	indicate the preambleReceivedTargetPower (included in rach-ConfigCommon) and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE).

5>	if the Random Access procedure for an SCell is performed on uplink carrier where pusch-Config is not configured:

6>	ignore the received UL grant.

5>	else:

6>	process the received UL grant value and indicate it to the lower layers.

4>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s):

5>	consider the Random Access procedure successfully completed.

4>	else:

5>	set the TEMPORARY_C-RNTI to the value received in the Random Access Response;

5>	if this is the first successfully received Random Access Response within this Random Access procedure:

6>	if the transmission is not being made for the CCCH logical channel:

7>	indicate to the Multiplexing and assembly entity to include a C-RNTI MAC CE in the subsequent uplink transmission.

6>	if the Random Access procedure was initiated for SpCell beam failure recovery and spCell-BFR-CBRA with value true is configured:

7>	if there is at least one Serving Cell of this MAC entity configured with two BFD-RS sets:

8>	indicate to the Multiplexing and assembly entity to include an Enhanced BFR MAC CE or a Truncated Enhanced BFR MAC CE in the subsequent uplink transmission.

7>	else:

8>	indicate to the Multiplexing and assembly entity to include a BFR MAC CE or a Truncated BFR MAC CE in the subsequent uplink transmission.

6>	else if the Random Access procedure was initiated for beam failure recovery of both BFD-RS sets of SpCell:

7>	indicate to the Multiplexing and assembly entity to include an Enhanced BFR MAC CE or a Truncated Enhanced BFR MAC CE in the subsequent uplink transmission.

6>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the Msg3 buffer.

NOTE 1:	If within a Random Access procedure, an uplink grant provided in the Random Access Response for the same group of contention-based Random Access Preambles has a different size than the first uplink grant allocated during that Random Access procedure, the UE behavior is not defined.

1>	if ra-ResponseWindow configured in BeamFailureRecoveryConfig expires and if a PDCCH transmission on the search space indicated by recoverySearchSpaceId addressed to the C-RNTI has not been received on the Serving Cell where the preamble was transmitted; or

1>	if ra-ResponseWindow configured in RACH-ConfigCommon or RACH-ConfigSIB1 expires, and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received:

2>	consider the Random Access Response reception not successful;

2>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

2>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

3>	if the Random Access Preamble is transmitted on the SpCell:

4>	indicate a Random Access problem to upper layers;

4>	if this Random Access procedure was triggered for SI request or SIB1 request:

5>	consider the Random Access procedure unsuccessfully completed.

3>	else if the Random Access Preamble is transmitted on an SCell:

4>	consider the Random Access procedure unsuccessfully completed.

2>	if the Random Access procedure is not completed:

3>	if preambleTransMaxRO-Type is applied, and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure, and PREAMBLE_TRANSMISSION_COUNTER = preambleTransMaxRO-Type + 1:

4>	if the RO_TYPE is set to 2nd-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same or higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), than the current set of Random Access resources, is available for the first PRACH occasions as defined in TS 38.213 [6]:

5>	set the RO_TYPE to 1st-RO;

5>	select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure;

5>	if sbfd-RACH-DualConfig is configured for the Random Access procedure (see TS 38.331 [5]):

6>	set PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP to PREAMBLE_POWER_RAMPING_STEP;

6>	(re-)initialize the parameters specified in clause 5.1.1 for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources;

6>	re-initialize PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI as specified in clause 5.1.1a;

6>	set POWER_OFFSET_RO_TYPE to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP).

4>	else if the RO_TYPE is set to 1st-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same or higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), than the current set of Random Access resources, is available for the second PRACH occasions as defined in TS 38.213 [6]:

5>	set the RO_TYPE to 2nd-RO;

5>	select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure;

5>	if sbfd-RACH-DualConfig is configured for the Random Access procedure (see TS 38.331 [5]):

6>	set PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP to PREAMBLE_POWER_RAMPING_STEP;

6>	(re-)initialize the parameters specified in clause 5.1.1 for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources;

6>	re-initialize PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI as specified in clause 5.1.1a;

6>	set POWER_OFFSET_RO_TYPE to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP).

3>	if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure:

4>	if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or

4>	if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1:

5>	if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available:

6>	select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure;

6>	initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

3>	select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF;

3>	if the criteria (as defined in clause 5.1.2) to select contention-free Random Access Resources is met during the backoff time:

4>	perform the Random Access Resource selection procedure (see clause 5.1.2).

3>	else if the Random Access procedure for an SCell is performed on uplink carrier where pusch-Config is not configured:

4>	delay the subsequent Random Access transmission until the Random Access Procedure is triggered by a PDCCH order with the same ra-PreambleIndex, ra-ssb-OccasionMaskIndex, and UL/SUL indicator TS 38.212 [9].

3>	else:

4>	perform the Random Access Resource selection procedure (see clause 5.1.2) after the backoff time.

The MAC entity may stop ra-ResponseWindow (and hence monitoring for Random Access Response(s)) after successful reception of a Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX.

HARQ operation is not applicable to the Random Access Response reception.

NOTE 2:	For the case that RAR PDSCH bandwidth is larger than the bandwidth the eRedCap UE can receive or process per slot, and the UL grant in RAR indicates that the time is not enough for Msg3 transmission, as specified in TS 38.213 [6], it is up to UE implementation, e.g. either to consider the Random Access Response reception not successful, or transmit Msg3.

## 5.3 Text proposal for issue MAC-3

### 5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted on a non-terrestrial network:

3>	start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the PDCCH occasion as specified in TS 38.213 [6].

2>	else:

3>	start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission.

2>	monitor for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while ra-ResponseWindow is running.

1>	else:

2>	if the Random Access Preamble was transmitted on a non-terrestrial network:

3>	if the Random Access Preamble is transmitted with repetitions:

4>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the PDCCH occasion from the end of all repetitions of the Random Access Preamble transmission as specified in TS 38.213 [6].

3>	else:

4>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the PDCCH occasion as specified in TS 38.213 [6].

2>	else if the Random Access Preamble is transmitted with repetitions:

3>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion from the end of all repetitions of the Random Access Preamble transmission as specified in TS 38.213 [6].

2>	else if the Random Access Preamble is transmitted for SIB1 request:

3>	start the ra-ResponseWindow configured in RACH-ConfigSIB1 at the first PDCCH occasion from the end of the Random Access Preamble transmission as specified in TS 38.213 [6].

2>	else:

3>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission.

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running.

1>	if notification of a reception of a PDCCH transmission on the search space indicated by recoverySearchSpaceId is received from lower layers on the Serving Cell where the preamble was transmitted; and

1>	if PDCCH transmission is addressed to the C-RNTI; and

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	consider the Random Access procedure successfully completed.

1>	else if a valid (as specified in TS 38.213 [6]) downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded:

2>	if the Random Access Response contains a MAC subPDU with Backoff Indicator:

3>	if the Random Access procedure was initiated for SIB1 request:

4>	set the PREAMBLE_BACKOFF to 0 ms.

3>	else:

4>	set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI.

2>	else:

3>	set the PREAMBLE_BACKOFF to 0 ms.

2>	if the Random Access Response contains a MAC subPDU with Random Access Preamble identifier corresponding to the transmitted PREAMBLE_INDEX (see clause 5.1.3):

3>	consider this Random Access Response reception successful.

2>	if the Random Access Response reception is considered successful:

3>	if the Random Access Response includes a MAC subPDU with RAPID only:

4>	consider this Random Access procedure successfully completed;

4>	if the Random Access procedure was initiated for SIB1 request:

5>	indicate the reception of an acknowledgement for SIB1 request to upper layers.

4>	else if the Random Access procedure was initiated for SI request:

5>	indicate the reception of an acknowledgement for SI request to upper layers.

3>	else:

4>	apply the following actions for the Serving Cell where the Random Access Preamble was transmitted:

5>	process the received Timing Advance Command (see clause 5.2);

5> if the received UL grant indicates that the corresponding PUSCH transmission is in SBFD symbols as specified in clause 11.1 of TS 38.213 [6]:

6>	if sbfd-RACH-SingleConfig (see TS 38.331 [5]) is configured for the Random Access procedure:

7>	indicate the sbfd-RACH-SingleConfig-preambleReceivedTargetPower if configured, or the preambleReceivedTargetPower otherwise, and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE).

6>	else if sbfd-RACH-DualConfig (see TS 38.331 [5]) is configured for the Random Access procedure:

7>	indicate the preambleReceivedTargetPower included in the sbfd-RACH-DualConfig, and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE).

6>	else:

7>	indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE).

5>	else (i.e., the received UL grant indicates that the corresponding PUSCH transmission is in non-SBFD symbols as specified in clause 11.1 of TS 38.213 [6]):

6>	indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE).

5>	if the Random Access procedure for an SCell is performed on uplink carrier where pusch-Config is not configured:

6>	ignore the received UL grant.

5>	else:

6>	process the received UL grant value and indicate it to the lower layers.

4>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s):

5>	consider the Random Access procedure successfully completed.

4>	else:

5>	set the TEMPORARY_C-RNTI to the value received in the Random Access Response;

5>	if this is the first successfully received Random Access Response within this Random Access procedure:

6>	if the transmission is not being made for the CCCH logical channel:

7>	indicate to the Multiplexing and assembly entity to include a C-RNTI MAC CE in the subsequent uplink transmission.

6>	if the Random Access procedure was initiated for SpCell beam failure recovery and spCell-BFR-CBRA with value true is configured:

7>	if there is at least one Serving Cell of this MAC entity configured with two BFD-RS sets:

8>	indicate to the Multiplexing and assembly entity to include an Enhanced BFR MAC CE or a Truncated Enhanced BFR MAC CE in the subsequent uplink transmission.

7>	else:

8>	indicate to the Multiplexing and assembly entity to include a BFR MAC CE or a Truncated BFR MAC CE in the subsequent uplink transmission.

6>	else if the Random Access procedure was initiated for beam failure recovery of both BFD-RS sets of SpCell:

7>	indicate to the Multiplexing and assembly entity to include an Enhanced BFR MAC CE or a Truncated Enhanced BFR MAC CE in the subsequent uplink transmission.

6>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the Msg3 buffer.

NOTE 1:	If within a Random Access procedure, an uplink grant provided in the Random Access Response for the same group of contention-based Random Access Preambles has a different size than the first uplink grant allocated during that Random Access procedure, the UE behavior is not defined.

1>	if ra-ResponseWindow configured in BeamFailureRecoveryConfig expires and if a PDCCH transmission on the search space indicated by recoverySearchSpaceId addressed to the C-RNTI has not been received on the Serving Cell where the preamble was transmitted; or

1>	if ra-ResponseWindow configured in RACH-ConfigCommon or RACH-ConfigSIB1 expires, and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received:

2>	consider the Random Access Response reception not successful;

2>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

2>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

3>	if the Random Access Preamble is transmitted on the SpCell:

4>	indicate a Random Access problem to upper layers;

4>	if this Random Access procedure was triggered for SI request or SIB1 request:

5>	consider the Random Access procedure unsuccessfully completed.

3>	else if the Random Access Preamble is transmitted on an SCell:

4>	consider the Random Access procedure unsuccessfully completed.

2>	if the Random Access procedure is not completed:

3>	if preambleTransMaxRO-Type is applied, and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure, and PREAMBLE_TRANSMISSION_COUNTER = preambleTransMaxRO-Type + 1:

4>	if the RO_TYPE is set to 2nd-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same or higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), than the current set of Random Access resources, is available for the first PRACH occasions as defined in TS 38.213 [6]:

5>	set the RO_TYPE to 1st-RO;

5>	select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure;

5>	if sbfd-RACH-DualConfig is configured for the Random Access procedure (see TS 38.331 [5]):

6>	set PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP to PREAMBLE_POWER_RAMPING_STEP;

6>	(re-)initialize the parameters specified in clause 5.1.1 for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources;

6>	re-initialize PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI as specified in clause 5.1.1a;

6>	set POWER_OFFSET_RO_TYPE to preambleReceivedTargetPower (included in rach-ConfigCommon) - preambleReceivedTargetPower (included in sbfd-RACH-DualConfig) +  (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_ (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP).

4>	else if the RO_TYPE is set to 1st-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same or higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), than the current set of Random Access resources, is available for the second PRACH occasions as defined in TS 38.213 [6]:

5>	set the RO_TYPE to 2nd-RO;

5>	select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure;

5>	if sbfd-RACH-DualConfig is configured for the Random Access procedure (see TS 38.331 [5]):

6>	set PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP to PREAMBLE_POWER_RAMPING_STEP;

6>	(re-)initialize the parameters specified in clause 5.1.1 for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources;

6>	re-initialize PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI as specified in clause 5.1.1a;

6>	set POWER_OFFSET_RO_TYPE to preambleReceivedTargetPower (included in sbfd-RACH-DualConfig) - preambleReceivedTargetPower (included in rach-ConfigCommon)+ (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP).

3>	if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure:

4>	if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or

4>	if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1:

5>	if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available:

6>	select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure;

6>	initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

3>	select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF;

3>	if the criteria (as defined in clause 5.1.2) to select contention-free Random Access Resources is met during the backoff time:

4>	perform the Random Access Resource selection procedure (see clause 5.1.2).

3>	else if the Random Access procedure for an SCell is performed on uplink carrier where pusch-Config is not configured:

4>	delay the subsequent Random Access transmission until the Random Access Procedure is triggered by a PDCCH order with the same ra-PreambleIndex, ra-ssb-OccasionMaskIndex, and UL/SUL indicator TS 38.212 [9].

3>	else:

4>	perform the Random Access Resource selection procedure (see clause 5.1.2) after the backoff time.

The MAC entity may stop ra-ResponseWindow (and hence monitoring for Random Access Response(s)) after successful reception of a Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX.

HARQ operation is not applicable to the Random Access Response reception.

NOTE 2:	For the case that RAR PDSCH bandwidth is larger than the bandwidth the eRedCap UE can receive or process per slot, and the UL grant in RAR indicates that the time is not enough for Msg3 transmission, as specified in TS 38.213 [6], it is up to UE implementation, e.g. either to consider the Random Access Response reception not successful, or transmit Msg3.