3GPP TSG-RAN WG2 #131bis	R2-2507364

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	8.11.3

Source:	Ericsson

Title:	Remaining issue for Stage 2 spec

Document for:	Discussion

# 1	Introduction

In this paper, we discuss one of the remaining issues for Stage 2 spec, as listed in [Post131][219][SBFD] CR for TS 38.300. Below is our input to this email disccussion:

| Company | Issue description and potential resolution | Rapporteur comment |
| --- | --- | --- |
| Ericsson | More cases on RO type selection is missing, including 1) CFRA triggered by dedicated RRC signaling 2) CBRA RO selection for initial PRACH transmissions. The motivation to include description on the rest cases are: In stage 2 spec, we need to keep a consistent description on the feature, i.e., RO type selection. We can either remove all texts on RO type selection, or include complete and consistent description for all cases, for better readability perspective. |  |

-

# 2	Discussion

Random access procedure for SBFD has been captured in the draft TS 38.300 (file Draft_38300-j00_v3.docx) as shown below:

## 23.2	SBFD Random Access

Random access procedure in SBFD symbols is supported for all existing RACH trigger events as described in clause 9.2.6, except for the event of Request for Other SI and Early UL synchronization with an LTM candidate cell. For the event of RACH-based LTM cell switch, random access procedure in SBFD symbols is only supported for intra-DU case.

Both CBRA and CFRA can be supported on SBFD sub-bands. Only the 4-step RA type using SBFD RACH resources can be supported.

Two RACH configuration options are specified for SBFD RA operation in TS 38.331 [12]. A cell can configure only one RACH configuration option. This can be either: 1) A single RACH configuration that supports both non-SBFD RA operation and SBFD RA operation, or 2) A dual RACH configuration where a RACH configuration is used for non-SBFD RA operation and an additional RACH configuration is designated for SBFD RA operation, as specified in TS 38.331 [12]. An SBFD aware UE that supports the RACH configuration option configured in the cell applies the corresponding RACH configuration. Otherwise, the SBFD aware UE applies the non-SBFD RA operation.

For CFRA triggered by PDCCH order, an SBFD aware UE can be explicitly indicated in the PDCCH order whether to use either the first PRACH occasions or the second PRACH occasions as specified in TS 38.213 [38], for the PRACH transmission.

For CBRA, an SBFD aware UE is permitted to switch from the first PRACH occasions to the second PRACH occasions and vice versa as specified in TS 38.213 [38] during a random access procedure. However, no further switch of PRACH occasions type (the first or second PRACH occasions) is allowed in the same random access procedure.

If a fall-back from CFRA to CBRA occurs, an SBFD aware UE uses the CBRA resource associated with the same PRACH occasions type.

From the above text, it is seen that the RO type selection/indication via the PDCCH order has been captured in the spec. However, more cases for the RO type selection are missing from the stage 2 spec, such as:

1) CFRA triggered by dedicated RRC signalling 2) CBRA RO selection for initial PRACH transmissions.

Therefore, we suggest to include specification text capturing the missing cases.

Update TS 38.300 to include text on RO type selection for the following cases:

  - CFRA triggered by dedicated RRC signalling and MAC CE (i.e., the LTM Cell Switch Command MAC CE)

  - CBRA RO selection for initial PRACH transmissions.

We have also prepared corresponding TP for the changes captured in section 5.

Adopt the TP to TS 38.300 captured in section 5.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Update TS 38.300 to include text on RO type selection for the following cases:

a. CFRA triggered by dedicated RRC signalling and MAC CE (i.e., the LTM Cell Switch Command MAC CE)

b. CBRA RO selection for initial PRACH transmissions.

Proposal 2	Adopt the TP to TS 38.300 captured in section 5.

# 4	References

- [1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Shanghai, China, June 17-20, 2024, RAN#104.

# 5	TP to TS 38.300

The text below is from draft TS 38.300 v19.0.0 (file Draft_38300-j00_v3.docx, with changes accepted).

## 23.2 SBFD Random Access

Random access procedure in SBFD symbols is supported for all existing RACH trigger events as described in clause 9.2.6, except for the event of Request for Other SI and Early UL synchronization with an LTM candidate cell. For the event of RACH-based LTM cell switch, random access procedure in SBFD symbols is only supported for intra-DU case.

Both CBRA and CFRA can be supported on SBFD sub-bands. Only the 4-step RA type using SBFD RACH resources can be supported.

Two RACH configuration options are specified for SBFD RA operation in TS 38.331 [12]. A cell can configure only one RACH configuration option. This can be either: 1) A single RACH configuration that supports both non-SBFD RA operation and SBFD RA operation, or 2) A dual RACH configuration where a RACH configuration is used for non-SBFD RA operation and an additional RACH configuration is designated for SBFD RA operation, as specified in TS 38.331 [12]. An SBFD aware UE that supports the RACH configuration option configured in the cell applies the corresponding RACH configuration. Otherwise, the SBFD aware UE applies the non-SBFD RA operation.

For CFRA triggered by PDCCH order, an SBFD aware UE can be explicitly indicated in the PDCCH order, the LTM Cell Switch Command MAC CE or dedicated RRC signalling whether to use either the first PRACH occasions or the second PRACH occasions as specified in TS 38.213 [38], for the PRACH transmission.

For CBRA, the network can configure a cell specific indication in SI whether to use either the first PRACH occasions or the second PRACH occasions. If there is no cell specific indication, the UE selects the PRACH occasions based on an SSB RSRP threshold provided in SI.

For CBRA, an SBFD aware UE is permitted to switch from the first PRACH occasions to the second PRACH occasions and vice versa as specified in TS 38.213 [38] during a random access procedure. However, no further switch of PRACH occasions type (the first or second PRACH occasions) is allowed in the same random access procedure.

If a fall-back from CFRA to CBRA occurs, an SBFD aware UE uses the CBRA resource associated with the same PRACH occasions type.