3GPP TSG-RAN WG2 #131bis	R2-2507375

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:		10.4	Mobility

Title:		Overview of mobility procedures in 6G

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN#109, the revised SID has been approved, which includes many aspects as RAN2 objectives [1]. In this contribution, we discuss overview of 6G mobility schemes and RRM measurement reporting framework.

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

# 2. Discussion

2.1	Overview of mobility schemes in 6G

In the 5G NR, various mobility schemes were specified in addition to the legacy L3-based handover procedure.

In Rel-16, CHO and DAPS are introduced in NR, targeting for higher robustness and 0ms service interruption separately. In CHO, the UE is configured with an execution condition, and triggers CHO procedure upon determination of fulfilment of the execution condition by the UE. In DAPS, the UE continues the downlink user data reception from the source gNB until releasing the source cell and continues the uplink user data transmission to the source gNB until successful random access procedure to the target gNB.

In Rel-18, intra-CU LTM procedure was introduced for mobility latency reduction and signalling overhead reduction. The intra-CU LTM cell switch procedure is based on RRC measurement or L1 measurement report from the UE and triggered by a MAC CE from the network, and supports serval funcations including early uplink and downlink synchronization, RACH-less LTM cell switch, and subsequent LTM cell switch. In the same release, the handover procedure is also enhanced to support RACH-less handover procedure.

In Rel-19 inter-CU LTM and conditional intra-CU LTM were introduced for further extension of the applicable scenario and robustness of the LTM cell switch.From our perspective, in the first release of 6G, both handover and LTM cell switch procedure should be supported. Because the handover, which is the baseline mobility procedure of previous generations of wireless mobility procedure, has been proved in the real implementation of its usability. And the LTM cell switch procedure, which is a rising star in the mobility families and has the benefit of lower latency, overhead and interruption time, can be supported to meet the high requirement of the 6G system.

Proposal 1: RAN2 aim to support both handover and LTM in the first release of 6G.

Compared with CHO and LTM, DAPS procedure is much more complex in the user plane design, and so far there is less interest to implement it in the 5G network, so we suggest to not consider DAPS-like scheme in the first release of 6G.

Proposal 2: DAPS-like mobility scheme is not considered at least in the first release of 6G

In the 5G NR, the handover procedure and LTM cell switch procedure use different RRC signalling branches and different RRC level procedures, which increases the signalling overhead and makes it complicated to implement the same enhancement to both of handover and LTM procedure. For example, if the network wants to configure CHO and LTM for the same UE, the network needs to signal the UE two set of signalling corresponding to each of the CHO and LTM separately. And if enhancement is applied to both handover and LTM, the signalling and procedure part should be designed and specified for CHO and LTM separately.

For 6G mobility, it would be good to start with study of a unified mobility architecture, including a single candidate configuration embedded in the RRC container for the candidate cell, and a single procedure description.

Proposal 3. RAN2 study a unified mobility architecture with single RRC signalling branch and procedure for the 6G mobility.

2.2	RRM measurement reporting framework

In current 5G NR measurement framework, the configuration and reporting of L1 measurement and L3 measurement procedure are completely independent.

Specifically, the L1 and L3 measurement are configured through different RRC signaling and the measurement results reported at different levels. For L3 measurement reporting, the cell-based triggering condition with RRC reporting is used, while for L1 measurement reporting, the beam-based triggering condition with MAC CE is used. Those duplicated RRM measurement procedures increase UE complexity and UE processing overhead, especially when L1 and L3 measurements and reporting are configured simultaneously, the UE must concurrently maintain different RRC measurement configurations and reporting logical (i.e., RRC and MAC CE reporting).

In addition, for the same cell, the L3 measurement results may be derived based on the L1 measurement results, or the L3 measurement resources may be a subset of the L1 measurement resources. However, based on the 5G NR measurement configuration, even if the resources (e.g., RS) for L1 and L3 measurements are the same, it still need to be configured separately by RRC signaling, which is redundant and increases the signaling overhead.

For 6G RRM measurement, it is beneficial to study a single unified RRM measurement procedure that supports L1 and/or L3 measurement and reporting control.

Proposal 4. RAN2 study a single unified measurement framework to support the configuration and reporting of L1 and L3 measurement.

# 3. Conclusion

In this contribution we discussed overview of 6G mobility schemes and RRM measurement reporting framework. We made the following proposals.

Proposal 1: RAN2 aim to support both handover and LTM in the first release of 6G.

Proposal 2: DAPS-like mobility scheme is not considered at least in the first release of 6G

Proposal 3. RAN2 study a unified mobility architecture with single RRC signalling branch and procedure for the 6G mobility.

Proposal 4. RAN2 study a single unified measurement framework to support the configuration and reporting of L1 and L3 measurement.

# References

[1] RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone