3GPP TSG-RAN WG2 Meeting #131-bis	R2-2507378

Prague, Czech Republic, 13th - 17th October 2025

Agenda Item:	8.6.2

Source:	Huawei, HiSilicon

Title:	RRC issues for LTM

Document for: Discussion and Decision

# 1	Introduction

The Rel-19 Mobility Enhancement WI was finalized in the previous meeting. The overall procedure for Rel-19 LTM enhancement including inter-CU LTM, L1 event-triggered measurement reporting, and intra-CU conditional LTM, has been captured in the current CRs for RRC and MAC.

This contribution discusses two RRC issues.

# 2	Discussion

## 2.1	[H150] C-LTM execution conditions

In TS 38.331 v19.0.0, execution conditions are provided as a list of execution conditions, per LTM-Candidate:

#### –	LTM-ExecutionConditionList

The IE LTM-ExecutionConditionList is used to configure LTM cell switch conditions.

LTM-ExecutionConditionList information element

LTM-ExecutionConditionList-r19 ::= SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-ExecutionCondition-r19

LTM-ExecutionCondition-r19 ::=     SEQUENCE {

ltm-CandidateId-r19                LTM-CandidateId-r18,

executionCondition-r19             CHOICE {

l1-Conditions-r19                  LTM-CSI-ReportConfigId-r18,

l3-Conditions-r19                  SEQUENCE (SIZE (1..2)) OF MeasId

}                                                                     OPTIONAL,   -- Need R

...

}

Such a list can be signalled in LTM-Config, as the ltm-ServingCellExecutionCondition field:

LTM-Config-r18 ::=   SEQUENCE {

-- field omitted here

ltm-ServingCellNoResetID-r18      INTEGER (1..maxNrofLTM-Configs-plus1-r18)OPTIONAL, -- Need N

-- field omitted here

...,

[[

ltm-ServingCellNoSecurityChangeID-r19   LTM-NoSecurityChangeId-r19       OPTIONAL,   -- Need N

ltm-ServingCellExecutionCondition-r19   SetupRelease {LTM-ExecutionConditionList-r19}

OPTIONAL    -- Need M

]]

}

When the UE receives ltm-ServingCellExecutionCondition in LTM-Config, the UE procedure is as follows:

The UE shall perform the following actions based on the received LTM-Config IE:

[...]

1>	if the received LTM-Config includes the field ltm-ServingCellExecutionCondition set to setup:

2>	for each LTM-ExecutionCondition included within ltm-ServingCellExecutionCondition:

3>	if the UE is performing LTM cell switch conditions evaluation based on L1 measurements:

4>	request lower layers to stop the LTM cell switch conditions evaluation based on L1 measurements for all the LTM candidate configurations;

3>	if the UE is performing LTM cell switch conditions evaluation based on L3 measurements:

4>	stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations as specified in 5.3.5.18.8;

3>	if l3-Conditions is included within ltm-ServingCellExecutionCondition:

4>	perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.8 according to the received ltm-ServingCellExecutionCondition;

3>	else if l1-Conditions is included within ltm-ServingCellExecutionCondition:

4>	request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the received field ltm-ServingCellExecutionCondition;

1>	else (ltm-ServingCellExecutionCondition set to release):

2>	release the ltm-ServingCellExecutionCondition.

The network can also provide a list of execution conditions in each LTM-Candidate, as the ltm-ExecutionCondition field:

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

-- field omitted here

...,

[[

ltm-NoSecurityChangeID-r19    LTM-NoSecurityChangeId-r19                    OPTIONAL, -- Need M

ltm-ExecutionCondition-r19    SetupRelease {LTM-ExecutionConditionList-r19} OPTIONAL, -- Need M

-- field omitted here

]]

}

Using this field, the following UE actions are specified when executing LTM cell switch:

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

-- field omitted here

...,

[[

ltm-NoSecurityChangeID-r19    LTM-NoSecurityChangeId-r19                    OPTIONAL, -- Need M

-- field omitted here

]]

}

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or upon the fulfilment of LTM cell switch execution conditions, the UE shall:

[...]

1>	if ltm-ExecutionCondition is configured within the LTM-Candidate IE for the selected LTM candidate configuration:

2>	if the field l3-Conditions is included within ltm-ExecutionCondition:

3>	perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.8 according to the received ltm-ExecutionCondition once this procedure is completed;

2>	else if the field l1-Conditions is included within ltm-ExecutionCondition:

3>	request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the received field ltm-ExecutionCondition once this procedure is completed.

In summary:

1)	when receiving ltm-ServingCellExecutionCondition,

-	the UE applies the received execution conditions immediately;

2)	when receiving ltm-ExecutionCondition in an LTM-Candidate,

-	the UE just stores the received ltm-ExecutionCondition, and will only apply when it performs LTM cell switch to this LTM-Candidate.

In the RRC PDU definitions:

-	ltm-ServingCellExecutionCondition is a SetupRelease Need M field, but when ltm-ExecutionCondition from an LTM-Candidate is applied, whatever remains in ltm-ServingCellExecutionCondition should not be applied anymore;

-	ltm-ExecutionCondition is applied when the UE applies a specific UE configuration.

Then, the specification would be clearer and shorter by doing the following:

-	in LTM-Candidate, remove ltm-ExecutionCondition;

-	remove the corresponding procedure text in 5.8.5.18.6;

-	in the RRCReconfiguration message contained in ltm-CandidateConfig, allow including ltm-Config but with ltm-ServingCellExecutionCondition as the only present field.

In this way, the UE always uses the execution conditions in ltm-ExecutionConditionServingCell, which significantly simplifies the specification.

For the issue Z151, the following TP was proposed:

[Proposed Change]: To add a condition to exclude the report configuration and measurement object configuration associated with L3 based CLTM. And also fix the wrong indentation. For example:

3>	for each measId of the MCG measConfig, if configured, and for each measId of the SCG measConfig, if configured, if the associated reportConfig has a reportType set to condTriggerConfig:

4>	if the reportConfigId is not associated with any measId indicated by the condExecutionCond or the condExecutionCondSCG in an entry of condReconfigList in VarConditionalReconfig in which subsequentCondReconfig is included; and

4>	if the reportConfigId is not associated with any measId indicated by the LTM-ExecutionCondition in an entry of LTM-ExecutionConditionList:

5>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

4>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig; and

4>	if the measObjectId is not associated with any measId indicated by the condExecutionCond or the condExecutionCondSCG in an entry of condReconfigList in VarConditionalReconfig in which subsequentCondReconfig is included; and

4>	if the measObjectId is not associated with any measId indicated by the LTM-ExecutionCondition in an entry of LTM-ExecutionConditionList:

5>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

4>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

TP for Z151

The problem in this TP is: which LTM-ExecutionConditionList, since there are two fields, and which one is applied depends on past UE behaviour, not captured in any variable. If there is a single field, it can be used directly without any ambiguity.

Proposal 1: Use a single field for execution conditions i.e.,

-	in LTM-Candidate, remove ltm-ExecutionCondition;

-	remove the corresponding procedure text in 5.8.5.18.6;

-	in the RRCReconfiguration message contained in ltm-CandidateConfig, allow including ltm-Config but with ltm-ServingCellExecutionCondition as the only present field.

In addition, we consider 5.3.5.18.1:

The UE shall perform the following actions based on the received LTM-Config IE:

[...]

1>	if the received LTM-Config includes the field ltm-ServingCellExecutionCondition set to setup:

2>	for each LTM-ExecutionCondition included within ltm-ServingCellExecutionCondition:

3>	if the UE is performing LTM cell switch conditions evaluation based on L1 measurements:

4>	request lower layers to stop the LTM cell switch conditions evaluation based on L1 measurements for all the LTM candidate configurations;

3>	if the UE is performing LTM cell switch conditions evaluation based on L3 measurements:

4>	stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations as specified in 5.3.5.18.8;

3>	if l3-Conditions is included within ltm-ServingCellExecutionCondition:

4>	perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.8 according to the received ltm-ServingCellExecutionCondition;

3>	else if l1-Conditions is included within ltm-ServingCellExecutionCondition:

4>	request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the received field ltm-ServingCellExecutionCondition;

1>	else (ltm-ServingCellExecutionCondition set to release):

2>	release the ltm-ServingCellExecutionCondition.

This raises three points:

-	there is no need to stop LTM cell switch evaluation "for all the LTM candidate configurations", for each LTM-ExecutionCondition, once is enough;

-	this should also be executed when release is received;

-	the "else" should be at 2>, otherwise, ltm-ExecutionConditionServingCell is effectively a Need R field.

This can be corrected as follows:

The UE shall perform the following actions based on the received LTM-Config IE:

[...]

1>	if the received LTM-Config includes the field ltm-ServingCellExecutionCondition:

set to setup:

2>	for each LTM-ExecutionCondition included within ltm-ServingCellExecutionCondition:

32>	if the UE is performing LTM cell switch conditions evaluation based on L1 measurements:

3>	request lower layers to stop the LTM cell switch conditions evaluation based on L1 measurements for all the LTM candidate configurations;

2>	if the UE is performing LTM cell switch conditions evaluation based on L3 measurements:

3>	stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations as specified in 5.3.5.18.8;

2>	if ltm-ServingCellExecutionCondition is set to setup:

3>	for each LTM-ExecutionCondition included within ltm-ServingCellExecutionCondition:

4>	if l3-Conditions is included within ltm-ServingCellExecutionCondition:

5>	perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.8 according to the received ltm-ServingCellExecutionCondition;

4>	else if l1-Conditions is included within ltm-ServingCellExecutionCondition:

5>	request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the received field ltm-ServingCellExecutionCondition;

2>	else (ltm-ServingCellExecutionCondition set to release):

3>	release the ltm-ServingCellExecutionCondition.

the bullet partially highlighted in yellow affects all previously ongoing execution conditions, but it is executed

Proposal 2: Correct the handling of ltm-ServingCellExecutionCondition as in the TP in 2.1.

## 2.2	[H151] Configuration of L1 event-trigger report & execution condition

In TS 38.331 v19.0.0, L1 event-triggered report transmitted on MAC CE and execution conditions are specified using the LTM-CSI-ReportConfig structure, which is also used in Rel-18 for reports on PUCCH and PUSCH.

#### –	LTM-CSI-ReportConfig

The IE LTM-CSI-ReportConfig is used to configure gNB-scheduled measurement report on the cell in which the LTM-CSI-ReportConfig is included when the field ltm-ReportConfigType is configured as periodic, semi-persistentOnPUCCH, or semi-persistentOnPUSCH/aperiodic.

The IE LTM-CSI-ReportConfig is also used to configure events for the conditional execution of an LTM cell switch procedure.

The IE LTM-CSI-ReportConfig is also used to configure event-triggered measurement report when the field ltm-ReportConfigType is configured as eventTriggered, within which the criteria for triggering measurement report by MAC CE as in TS 38.321 [3] is specified.

[...]

LTM-CSI-ReportConfig information element

LTM-CSI-ReportConfig-r18 ::=  SEQUENCE {

ltm-CSI-ReportConfigId-r18               LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18   LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                     CHOICE {

periodic-r18                                 SEQUENCE {},

semiPersistentOnPUCCH-r18                    SEQUENCE {},

semiPersistentOnPUSCH-r18                    SEQUENCE {},

aperiodic-r18                                SEQUENCE {},

...,

eventTriggered-r19                   SEQUENCE {

eventId-r19                          CHOICE {

eventLTM2-r19                        SEQUENCE {},

eventLTM3-r19                        SEQUENCE {},

eventLTM4-r19                        SEQUENCE {},

eventLTM5-r19                        SEQUENCE {},

...

},

ltm-CandidateReportConfigList-r19    SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18))

OF LTM-CandidateReportConfig-r19  OPTIONAL,--Need R

ltm-EventTriggeredReportContent-r19  LTM-EventTriggeredReportContent-r19  OPTIONAL,--Need R

reportOnLeave-r19                    ENUMERATED {enabled}                 OPTIONAL,--Need R

ltm-EventTriggeredPeriodicReport-r19 LTM-EventTriggeredPeriodicReport-r19 OPTIONAL,--Need S

candidateSpecificOffsetS-r19         MeasTriggerQuantityOffset OPTIONAL, -- Cond onlyLTM3

...

}

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...,

[[

ltm-ReportContent-v1900                        LTM-ReportContent-v1900      OPTIONAL,--Need R

ltm-ResourceForInterferenceMeasurements-r19    LTM-CSI-ResourceConfigId-r18 OPTIONAL --Need R

]]

}

In TS 38.331, the only list of LTM-CSI-ReportConfig is in CSI-MeasConfig:

CSI-MeasConfig ::=                  SEQUENCE {

nzp-CSI-RS-ResourceToAddModList     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources))

OF NZP-CSI-RS-Resource          OPTIONAL, -- Need N

nzp-CSI-RS-ResourceToReleaseList    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources))

OF NZP-CSI-RS-ResourceId        OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToAddModList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets))

OF NZP-CSI-RS-ResourceSet       OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToReleaseList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets))

OF NZP-CSI-RS-ResourceSetId     OPTIONAL, -- Need N

csi-IM-ResourceToAddModList         SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources))

OF CSI-IM-Resource              OPTIONAL, -- Need N

csi-IM-ResourceToReleaseList        SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources))

OF CSI-IM-ResourceId            OPTIONAL, -- Need N

csi-IM-ResourceSetToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets))

OF CSI-IM-ResourceSet           OPTIONAL, -- Need N

csi-IM-ResourceSetToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets))

OF CSI-IM-ResourceSetId         OPTIONAL, -- Need N

csi-SSB-ResourceSetToAddModList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets))

OF CSI-SSB-ResourceSet          OPTIONAL, -- Need N

si-SSB-ResourceSetToReleaseList    SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets))

OF CSI-SSB-ResourceSetId        OPTIONAL, -- Need N

csi-ResourceConfigToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations))

OF CSI-ResourceConfig           OPTIONAL, -- Need N

csi-ResourceConfigToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations))

OF CSI-ResourceConfigId         OPTIONAL, -- Need N

csi-ReportConfigToAddModList        SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations))

OF CSI-ReportConfig             OPTIONAL, -- Need N

csi-ReportConfigToReleaseList       SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations))

OF CSI-ReportConfigId           OPTIONAL, -- Need N

reportTriggerSize                   INTEGER (0..6)                        OPTIONAL, -- Need M

aperiodicTriggerStateList           SetupRelease { CSI-AperiodicTriggerStateList }

OPTIONAL, -- Need M

semiPersistentOnPUSCH-TriggerStateList SetupRelease {CSI-SemiPersistentOnPUSCH-TriggerStateList}

OPTIONAL, -- Need M

...,

-- fields omitted

ltm-CSI-ReportConfigToAddModList-r18 SEQUENCE(SIZE(1..maxNrofLTM-CSI-ReportConfig-r18))

OF LTM-CSI-ReportConfig-r18       OPTIONAL, -- Need N

ltm-CSI-ReportConfigToReleaseList-r18 SEQUENCE(SIZE(1..maxNrofLTM-CSI-ReportConfig-r18))

OF LTM-CSI-ReportConfigId-r18 OPTIONAL,  -- Need N

-- fields omitted

}

CSI-MeasConfig field descriptions

ltm-CSI-ReportConfigToAddModList Configured CSI report settings for LTM as specified in TS 38.214 [19].

This list is defined as "Configured CSI report settings for LTM as specified in TS 38.214 [19]", which describes reports on PUCCH and on PUSCH, but not reports on MAC CE.

These items are included in CSI-MeasConfig, which is included in ServingCellConfig.

ServingCellConfig ::=           SEQUENCE {

tdd-UL-DL-ConfigurationDedicated TDD-UL-DL-ConfigDedicated              OPTIONAL,   -- Cond TDD

initialDownlinkBWP               WP-DownlinkDedicated                   OPTIONAL,     -- Need M

downlinkBWP-ToReleaseList    SEQUENCE (SIZE(1..maxNrofBWPs)) OF BWP-Id  OPTIONAL,     -- Need N

downlinkBWP-ToAddModList     SEQUENCE (SIZE(1..maxNrofBWPs)) OF BWP-Downlink OPTIONAL,-- Need N

firstActiveDownlinkBWP-Id           BWP-Id                     OPTIONAL, -- Cond SyncAndCellAdd

bwp-InactivityTimer                 ENUMERATED { }    OPTIONAL,   --Need R

defaultDownlinkBWP-Id               BWP-Id                              OPTIONAL,     -- Need S

uplinkConfig                        UplinkConfig                        OPTIONAL,   -- Need M

supplementaryUplink                 UplinkConfig                        OPTIONAL,   -- Need M

pdcch-ServingCellConfig             SetupRelease {PDCCH-ServingCellConfig} OPTIONAL, -- Need M

pdsch-ServingCellConfig             SetupRelease {PDSCH-ServingCellConfig} OPTIONAL, -- Need M

csi-MeasConfig                      SetupRelease {CSI-MeasConfig}        OPTIONAL,   -- Need M

-- other fields omitted

}

For the reports on PUSCH and PUCCH, the reports are included in the cell on which the PUCCH is to be transmitted, or in the cell on which the DCI triggering the report on PUSCH is transmitted.

However, L1 event-triggered reports transmitted on MAC CE and execution conditions have no relationship with any serving cell, they are per cell group. If their configuration would be kept per serving cell, this would create the possibility of several configurations on different serving cells, possibly with the same ID value, which is confusing.

This situation is unprecedented, there is no example of something configured per serving cell, while it is actually per cell group. To avoid confusions, items that are per CG should be configured at CG level.

In addition, L1 event-triggered reports and L1 execution conditions are two different items, for which different procedures in TS 38.321 are executed, so it would be clearer to configured them in separate lists.

Proposal 3a: Create a new ToAddModList and ToReleaseList in MAC-CellGroupConfig for LTM L1 event-triggered reports.

Proposal 3b: Create a new ToAddModList and ToReleaseList in MAC-CellGroupConfig for L1 events used for execution conditions.

# 3	Conclusion

We have the following proposals:

Proposal 1: Use a single field for execution conditions i.e.,

-	in LTM-Candidate, remove ltm-ExecutionCondition;

-	remove the corresponding procedure text in 5.8.5.18.6;

-	in the RRCReconfiguration message contained in ltm-CandidateConfig, allow including ltm-Config but with ltm-ServingCellExecutionCondition as the only present field.

Proposal 2: Correct the handling of ltm-ServingCellExecutionCondition as in the TP in 2.1.

Proposal 3a: Create a new ToAddModList and ToReleaseList in MAC-CellGroupConfig for LTM L1 event-triggered reports.

Proposal 3b: Create a new ToAddModList and ToReleaseList in MAC-CellGroupConfig for L1 events used for execution conditions.