3GPP TSG-RAN WG2 Meeting #131bis	R2-2507380

Prague, Czech Republic, 13 - 17 October 2025

Source: 	Huawei, HiSilicon

Title: 	Remaining issues for MBS broadcast over NTN

Agenda Item:	8.8.4

Document for:	Discussion and decision

# 1	Introduction

In previous meeting, RAN2 agreed to consider the Target Service Area for the purpose of geofencing MBS broadcast services in NTN. Th open issues are highlighted in yellow [1][2].

38.304 CR Open issue 1: The TAI/Cell list received in target service area in USD could be used by UE for service continuity purpose.

38.331 CR RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. C003; NTN; 1; clarify how UE determines the service area if the Target Service Area in the USD or the ISA in SIBXX are not totally aligned; Yes, R2-250xxxxx; CATT (Da Wang); v008; ToDo.

# 2	Discussion

There is an issue in 38.304 open issue list[1] need to be discussed:

When the "target service area" in USD is defined using a TAI/cell list (rather than a geographical shape), whether the UE can utilize this explicit list to determine whether it is within the service area, thereby achieving service continuity.

Open issue 1: The TAI/Cell list received in target service area in USD could be used by UE for service continuity purpose. Resolution: Remove the ‘(i.e., an area specified as a list of polygon or circle shapes)’ in below note. NOTE 0j: The ISA(s) can be ISA(s) provided in SIBxx and/or target service area (i.e., an area specified as a list of polygon or circle shapes) in USD. It is up to UE’s implementation to decide whether it is inside ISA(s) or not.

We believe that it is reasonable to remove the qualifier regarding "polygonal or circular shapes" from Note 0j.

Before the modification, "The target service area" primarily refers to an area defined by geographical shapes (polygons/circles). The UE needs to calculate whether its position is within the shape.

After the modification, "The target service area" can be an area defined in any manner (including TAI/cell lists and geographical shapes). The UE needs to determine whether it is within the area based on its own implementation method.

Moreover, UE directly checks whether the TAI/Cell is in the list to determine service availability, eliminating the need to always rely on power-consuming GNSS positioning, thereby achieving greater efficiency and energy savings. When the UE moves, it can know whether the next target cell supports the MBS service in advance, thereby enabling seamless handover.

Proposal 1: Remove the “(i.e., an area specified as a list of polygon or circle shapes)” in Note.

Additionally, there is a remaining issue on ISA for 38.331CR[2]:

38.331 CR

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C003 | NTN | 1 | clarify how UE determines the service area if the Target Service Area in the USD or the ISA in SIBXX are not totally aligned | Yes, R2-250xxxxx | CATT (Da Wang) |  | v008 | ToDo |

There are two methods to provide the intended service area for an MBS broadcast services:

1)	target service area info in the USD;

2)	ISA IDs in the MBSBroadcastConfiguration message + the ISA definitions in SIB27.

The target service area in the USD may cover many cells, and can be a list of circle areas, whose radius is determined by  where K is a value from 0 to 127. While such a coding is rather efficient (only 7 bits), the granularity is rather coarse. SIB27 only provides areas for once cell and can provide circle areas with a linear granularity of the radius, so it can be much finer. Therefore, if SBI27 is provided, it will provide much more accurate information on the relevant area.

If, for some cell, the granularity provided by the target service area in the USD is sufficient, the network may not broadcast SIB27. Otherwise, the network can provide SIB27 and the UE should use it to avoid receiving the service while located in areas where it is actually not relevant.

Then, if the network broadcasts SIB27 in a cell, the UE should acquire SIB27, the MCCH and, read the MBSBroadcastConfiguration message and see if the service is provided in the cell and what the intended service area (one or more ISA) is. If the UE sees that it is not in the intended service area of the MBS broadcast service of interest, for as long as it remains in the current cell, it can skip reading MCCH again.

If the network does not broadcast SIB27, then the UE can use the target service area indicated in the USD of the MBS broadcast service of interest, if provided.

Proposal 2: To determine the intended service area of an MBS broadcast service of interest:
	- when SIB27 is not broadcast, the UE can use the target area info in the USD;
	- when SIB27 is broadcast, the UE shall use SIB27 and the MBSBroadcastConfiguration message..

# 3	Conclusion

In this contribution the following observations and proposals are made:

Proposal 1: Remove the “(i.e., an area specified as a list of polygon or circle shapes)” in Note.

Proposal 2: To determine the intended service area of an MBS broadcast service of interest:
	- when SIB27 is not broadcast, the UE can use the target area info in the USD;
	- when SIB27 is broadcast, the UE shall use SIB27 and the MBSBroadcastConfiguration message.

# 4	References

- Open issues on Rel-19 NR NTN 38.304 CR.

- NTN Review file v015.