3GPP TSG-RAN WG2 Meeting #131bis	R2-2507385

Prague, Czechia, 13th – 17th of October 2025

Agenda item:	10.4

Source:	Nokia

Title:	Initial Thoughts on 6G MMM (Mobility, Measurements and Migration)

WID/SID:	FS_6G_Radio

Document for:	Discussion and Decision

# 1	Introduction

This paper presents our initial view on 6G mobility and measurements which RAN2 shall study based on the following objectives of the RAN 6G SID [1]:

(4) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

We also explain our view on the migration from NR to 6GR and inter-RAT mobility, as per the following objective [1]:

(7) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: a) 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] b) Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. c) Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

# 2	Discussion

## 2.1	Connected mode mobility

### 2.1.1	Baseline procedure

The baseline handover procedure in 5G (as defined in 3GPP Release 15) has demonstrated effective support for enhanced Mobile Broadband (eMBB) services across a wide range of deployment scenarios. Building on this foundation, subsequent 5G releases have introduced key enhancements focused on improving handover robustness, reducing service interruption time, and enabling support for emerging use cases beyond eMBB. 5G baseline handover defined in 3GPP Release 15 was sufficient for eMBB scenario. Thus, a similar mechanism should be studied for 6G mobility, at least to cover basic scenarios such as eMBB.

To cater for other use-cases such as private network mobility that demands reliability and low latency, Release 16 of 5G has introduced enhanced mobility procedures such as conditional handover (CHO) that improves mobility robustness, dual active protocol stack (DAPS) that reduces interruption time. In Release 18, an integration of beam management procedure and mobility procedure is strived for with the procedure of layer 1/layer 2 triggered mobility (LTM). The LTM feature also introduced the possibility to use either L3 or L1 measurements for deciding whether to trigger the handover. Also in Release 18, specifically for mobile integrated access and backhaul (mIAB) and non-terrestrial network (NTN) deployments, a Release 15 HO was enhanced with RACH-less possibility. Having numerous separate mobility procedures creates implementation challenges for UE and NW vendors and creates many inter-working scenarios to be dealt with.  That, in turn, slows down the actual technical progress in 3GPP. It is dubious if the effort spent in 3GPP versus the market value created by such complex design was justified.

Observation 1: 5G has introduced many mobility enhancements to cater distinct use-cases for some of which market value is not clear.

A unified mobility procedure further denoted as unified handover (UHO) shall be the primary focus of 6G mobility studies. It should consider the functionalities that clearly possess a market value and will not lead to implementation complexity. UHO shall consider two main parts, one is the baseline features to be supported by all UEs in the network and procedures that can be supported for specific use-cases optionally. To simplify the use-case specific mobility procedure support without creating additional complexity with capability handling, we suggest that this aspect is taken into consideration of device types. We do not discuss the framework to support optional capabilities in this paper, and it can be taken under device type and capability discussions. We focus mainly on the baseline mobility procedure.

Proposal 1: Study a 6GR unified handover (UHO) procedure unifying selected mobility procedures defined in 5G and 5G-Advanced. The study shall consider the following aspects:

- both UE and NW implementation aspects of the handover procedure.

- eMBB use-case shall be well addressed.

- the value added by each procedure to baseline UHO procedure.

### 2.1.2	Mobility features

In this section we briefly discuss several mobility features introduced in 5G and give Nokia’s views on these features. We firstly concentrate on interruption time reduction, and then we give our short view on other performance metrics (i.e. robustness and throughput).

#### 2.1.2.1	Handover interruption time reduction

One of the goals of mobility is to make the entire procedure as seamless as possible for the UE. This includes interruption time reduction. Dual Active Protocol Stack (DAPS) feature aimed to achieve this through the UE being connected to source and target cell at the same time. This approach was perceived as complex  and partly due to that 3GPP has studied another mechanism for reducing the interruption time, namely LTM. This feature aimed to reduce the interruption time through the introduction of the following key elements: decoupling of the configuration and execution command, early DL sync, early UL sync, early decoding and early processing.

##### Requirements

The initial aim of the DAPS was to enable ultra-reliable low latency communication (URLLC) use-case with 1 ms latency even during mobility procedures. LTM procedure aimed at a bit higher interruption time of around 20 ms to support XR, cloud gaming and better support to bi-directional traffic use-cases.

As part of the ongoing 6G requirements study in SA1, several emerging verticals are identified with stringent service continuity and mobility performance demands. Among these, Immersive Media Services for Advanced Air Mobility (AAM) [3] which has requirements of 10 ms of maximum interruption for a media service that supports 60 fps [5] and dynamic HO between TN and NTN networks [6], as well as Seamless Immersive Reality in Education [4] which requires 50 ms end-to-end latency [7,8], are two use cases that impose particularly tight constraints on handover procedures. These use cases exemplify the need for reduced interruption time and seamless mobility, which directly drive the need for a careful study on the handover mechanisms in 6G.

Handover delay typically consists of:

- processing of the HO message,

- processing of the target cell configuration

- DL sync acquisition

- Waiting for the first PRACH occasion

- UL grant to transmit the RRC Reconfiguration Complete message

##### RACH-less HO

RACH-less HO is a low complexity feature compared to DAPS, also aimed at reducing the handover interruption time. This is true, especially for RACH-less HO specified in 5G for two specific scenarios. A scenario is where the target cell timing advance (TA) is equal to 0 or is the same as source cell’s TA. Other scenario is where the target cell can have any TA. The RACH-less HO means that the UE can skip the RACH procedure and waiting for the RACH-occasions. Assuming a RACH periodicity of 20 ms and a fast success of RACH procedure, this means that interruption time can be reduced by around 30 ms.

Observation 2: RACH-less HO reduces handover interruption time via skipping the random-access procedure.

##### Early DL sync

Early downlink (DL) synchronization for LTM candidate cells was introduced as a way to shorten cell switch interruption in LTM. Early DL synchronization in LTM is realized through early candidate TCI state activation for one or more LTM candidate cells, where the TCI state(s) to be activated before cell switch are chosen from the candidate TCI states configured in the LTM candidate cell configuration. If the UE receives a cell switch command with an activated TCI state as the target TCI state, DL synchronization is not part of the cell switch delay. This, according to [2], reduces the cell switch interruption by the maximum of one SSB periodicity plus SSB processing delay of 2 ms.

Observation 3: Early DL synchronization through early candidate TCI state activation was introduced in Rel-18 LTM to reduce cell switch interruption duration.

##### Early decoding and processing

Early decoding feature is signaled using the capability of FastProcessingConfig in LTM procedure. It enables that before initiating the handover UE goes through the target cell configuration, does ASN.1 decoding and validity/compliance check for the target RRC configuration. This decreases the RRC processing delay down to 0 ms when the actual cell change occurs.

Early UE processing, called fast UE processing in LTM procedure, is a feature introduced to indicate that UE can do faster application of the candidate configuration for a FR1-FR1 handover decreasing the generic processing time from the typical value of 40 ms down to 10 ms.

Observation 4: Early UE processing and early configuration decoding can reduce the interruption time by tens of ms.

#### 2.1.2.2	Mobility robustness improvement

Mobility robustness is the metric describing how immune to failures the handover procedure is. This is typically reflected by the handover success rate and radio link failure RLF rate. In order to improve the robustness one feature considered since early days of 5G is to pre-configure the target cell’s configuration to the UE and allow UE to execute it when appropriate.

##### Conditional handover

In case the pre-configured target configuration is used based on a radio condition, this is called conditional handover (CHO). CHO is known to improve HO success rate and to decrease RLF. However, CHO has the non-negligible cost of resource reservation at the NW side. After providing the configuration to be used for handover procedure, in the target cell the reserved resources can’t be used for other UEs during the preparation time. Moreover, there can be more than one candidate cell for the conditional handover procedure. Thus, the resource reservation requirement is always higher compared to the baseline handover procedure.

Additionally, since CHO is prepared earlier, the configuration may need to be updated before they are used as the UE context may change between the time of HO request toward candidate target cells and the actual time when HO execution happens. In a nutshell, the mobility robustness gains are likely insufficient considering the complexity to maintain candidate configuration for a longer time [9].

Observation 5: Although CHO improves robustness, it requires allocated resources over a long time due to earlier preparation and potentially at more than one candidate target cell, leading to inefficiency in terms of NW resource usage.

##### Subsequent mobility

In case the pre-configured target configurations are maintained after executing handover, another feature using those configurations is called a subsequent mobility. The subsequent mobility feature provides a slight improvement to mobility robustness as the preparation time to provide the target cell configuration to the UE is avoided as UE keeps multiple target configuration after executing a handover. This is especially useful for ping pong scenario. However, subsequent mobility reverberates the resource reservation problem as target cell configuration is maintained for even longer time. But more importantly, subsequent mobility causes further issues related to handling of the handover configuration.

Observation 6: Subsequent mobility feature offers a small mobility robustness improvement [10] compared to handover configuration handling issue that it introduces.

#### 2.1.2.3	Cell edge throughput enhancement

A typical handover takes place at the cell edge and cell edge throughput should be an important performance indicator for 6G.

##### User plane treatment during mobility

Before diving into further features we have to clarify some aspects on PDCP and RLC treatment during mobility procedure. The basic user plane procedures during mobility such as PDCP and RLC re-establishment worked well in 5G. It would be ideal to strive to limit the PDCP re-establishment and RLC reset as the deployment would allow.

Observation 7: The basic 5G user plane handling procedures such as PDCP and RLC re-establishment did not show considerable issues with respect to mobility.

##### Early CSI acquisition

Early CSI acquisition for LTM was introduced to improve throughput for data transmission and reception immediately after a cell switch, by enabling appropriate link adaptation (e.g., modulation and coding scheme, number of layers, and precoding weights). In this feature, the UE is configured to acquire and report CSI parameters (CQI, PMI, RI, and/or LI) for the target cell, which can then be used for transmission and reception in the target cell after the switch.

In NR Rel-20 MIMO, this feature is also being extended to UEs moving from inactive/idle mode to connected mode, in order to achieve optimal link adaptation immediately after the UE transitions to connected mode. Furthermore, there are ongoing TEI19 discussions in RAN1 and RAN2 on adopting this feature for L3 handover in NR.

Observation 8: Early CSI acquisition was introduced in Rel-19 for LTM, and is also being extended to L3 handover, in order to enable high-throughput data transmission and reception immediately after a cell switch.

##### Mobility with multi-TRP and carrier aggregation

One way to further enhance throughput during mobility is to consider better integration of physical layer procedures to mobility such as multi-TRP and carrier aggregation. In Rel-18 and Rel-19, LTM considered aspects including: (i) cell switch with a single connection to a target cell configured with mTRP with two timing advances, and (ii) multi-TRP with two indicated beams in the serving cell, where the UE compares the quality of candidate beams with the serving beam. In Rel-20, the mobility enhancements work item has the objective of introducing LTM with carrier aggregation enhancements.

#### 2.1.2.4 Nokia’s view on mobility features discussed above

In the table below we summarize our view with respect to the 5G mobility features that were briefly described also above.

| Mobility features | KPI | Nokia view |
| --- | --- | --- |
| RACH-less HO | Interruption time reduction | Supportive |
| Early DL synchronization | Interruption time reduction | Supportive |
| Early decoding and processing | Interruption time reduction | Supportive |
| Conditional mobility | Mobility robustness improvement | We do not see sufficient gains versus complexity |
| Early CSI acquisition | Throughput increase | Supportive |
| Dual active protocol stack | Interruption time reduction | We do not see sufficient gains versus complexity |
| L1 measurement triggered mobility | Mobility robustness improvement for dynamic radio environment | Supportive for limited scenarios |
| Avoiding PDCP re-establishment | Throughput increase | Supportive for limited scenarios |
| Avoiding RLC reset | Throughput increase | Supportive for limited scenarios |
| MAC CE based cell change | Faster processing of cell change | We do not see sufficient gains in comparison to RRC-based mobility |
| Subsequent mobility | Mobility robustness improvement and signalling overhead decrease | We do not see sufficient gains versus complexity |

Proposal 2: Study procedures to enable interruption time reduction during mobility, ensure robustness and high cell edge throughput. In particular RAN2 considers the following:

- For interruption time reduction consider early DL synchronization, early UL synchronization and early decoding.

- For ensuring mobility robustness study the procedures that show clear gains versus complexity.

- For ensuring high cell edge throughput prioritize procedures that offers the highest gains.

## 2.2	Measurements

Radio measurements are inherently associated with mobility management. Our initial 6G focus should be on properly designing the intra-frequency measurements which seem to be the most important measurements for guaranteeing smooth and robust connectivity with network and facilitating robust mobility. Hence, measurements on the frequency layer of the cell(s) which is used for maintaining UE’s connection with the network, i.e., the serving cell, or is camped on, must have the highest priority.

Observation 9: The most important measurements are intra-frequency measurements performed on serving/primary cell carrier.

It shall of course be possible for the network to configure the UE to perform measurements on other carriers than serving carrier(s). However, such measurements should typically be performed less often than measurements on the serving/primary cell carrier and should be mostly targeted for specific purposes. The list of exemplary use cases is as follows:

•	Serving layer is not providing full and continuous coverage.

•	Load balancing between different frequency layers.

Observation 10: It shall be possible to configure the UE to perform inter-frequency measurements.

When discussing the measurement framework for 6G we need to consider necessity to provide scheduling gaps for conducting some of UE’s measurements and consider possible interruptions/disturbance caused by UE’s measurements without need for gaps.

Gap-supported inter-frequency measurements in our view are supposed to be the baseline for UEs in 6G. Gap design in previous mobile generations was working well, but some improvements are always possible and shall be considered. Gaps configurations should support use cases/scenarios such as HetNet- or SCell- detection, load balancing without excessive reconfiguration signalling and while gaps are likely still needed for some use cases, the use of gaps should be flexible and adjusted dynamically to instantaneous needs, such as balancing UE power savings and increasing network’s throughput. Lastly, it should be possible to easily distribute gaps in time and resources across UEs, to facilitate ease of scheduling at the network.

Observation 11: Measurement gaps in 6G shall enable efficient network operation.

6G must also account for the fact that 6G UEs might support non-gap assisted inter-frequency measurements – with or without interruptions.

Observation 12: 6G considers that UEs may support also non-gap assisted inter-frequency measurements with or without interruptions.

NR Rel-18 introduced assistance signalling related to UE performing non-gap assisted measurements with and without interruptions. However, no consensus has been reached as to the requirements of UEs not having interruptions outside defined areas. This forces non-trivial implementation of safeguards at the network.

Proposal 3: RAN2 to study how to reduce measurement overhead and scheduling limitations when compared to 5G. Gap-assisted measurements should consider the following aspects:

- non-uniform gap pattern to support efficient gap-based measurements of deactivated serving cells or for load balancing

- dynamic network-controlled gap use

- gap distribution in time and resource domain across UEs to enable efficient scheduling at the network.

In 5G the RRM measurements performed by a UE are configured for UE in RRC_CONNECTED. The UE in  RRC_IDLE and RRC_INACTIVE obtains the configuration from elsewhere (e.g. SIB or Release message). A UE transitioning from RRC_CONNECTED to non-connected states and in the opposite direction may discard any available measurements and start performing measurements based on the recently received configuration.

Observation 13: There are separate configurations for RRM measurements provided to the UE in different RRC states.

Measurement configuration includes information on what to measure, when to measure, which reference signals to use, etc. and may be at least partly the same across different RRC states as the measurement objects are corresponding to the frequency layers/cells in the vicinity of the UE in a given serving/camped cell.

Observation 14: There should be at least some measurement objects/frequencies that remain the same irrespective of UE’s RRC state.

A UE may make use of available measurements even after state transition, but in 5G 3GPP provided no means for reusing measurement results and/or common configurations. When discussing RRC states and mobility measurements for 6G, RAN2 needs to consider possible simplification and unification of the configurations as well as reuse of measurement results across RRC states.

Proposal 4: Study RRM measurements framework for different RRC states under single objective to simplify and unify measurement configurations. Consider reusing measurement results across RRC states and idle mode measurements for fast CA setup in 6G Day One.

## 2.3	Idle/Inactive mode mobility

The baseline cell reselection procedure appears to work well in 5G. Since 4G, the procedure remains roughly the same, with some relatively minor adaptations added for 5G. We also do not expect this will fundamentally change in 6G. However, there are some cases where 4G/5G-like cell reselection could be improved. For instance, cell reselection during RRC Resume/Setup​​. Another example is the delay in HO process during RRC Resume​​. There may be need for ‘immediate’ handover after the UE transitions from RRC_INACTIVE to RRC_CONNECTED at the cell edge. ​​If the serving cell radio conditions drop during RRC Resume, then the first possible time to trigger HO is after completion of RRC Resume and evaluation of measurements.​

Proposal 5: Study and identify the potential gaps of 5G cell reselection. Focus in particular on reselection and handover acceleration when UE moves from INACTIVE to CONNECTED (if INACTIVE-like state is supported in 6G).

## 2.4	Inter-RAT Mobility

Efficient inter-RAT mobility is always important for every new generation, as different generations tend to coexist in most deployments and the coverage of the different generations may differ, leading to the need for the UEs to move from one RAT to the other.

The system architecture also considers connectivity of both NR and 6GR nodes to single evolved core network that consists of components to handle both 5G and 6G core network functionality. Some core network components can be shared between the RAT depending on the required functionality and these aspects are part of study in SA working groups. This evolved single CN architecture for 6GR is similar to the standalone architecture of 5G.

For further analysis, we consider the following phases of the inter-RAT mobility procedures:

- Inter-RAT measurements

- Inter-system handover preparation

- Handover execution

- Inter-system context transfer after handover execution.

The handover preparation and inter-system context transfer aspects for inter-RAT mobility between 6GR and NR involves the 6GC and 5GC interactions. However, the procedures defined for NR to E-UTRA handover can be reused due to similarity in the architecture between both.

Observation 15: Inter-RAT handover preparation and inter-system context switching phases of 6GR to NR mobility procedure can reuse the principles of NR to E-UTRA handover procedure defined for standalone architecture in NR.

The Inter RAT measurements and handover execution steps of the mobility procedure require radio interface changes in 6GR and NR. The scenarios that trigger the inter-RAT mobility are an important part of study related to the radio interface part for the inter-RAT mobility. The network may trigger inter-RAT mobility based on other triggers such as load or energy efficiency optimization in the source cell.

Observation 16: Coverage-based mobility and load-based mobility are expected to be the main triggers for 6GR-NR mobility.

For the above mobility scenarios, the study needs to identify additional solutions for efficient IRAT mobility in terms of resource efficiency and better mobility performance. The solution components considered for unified mobility procedure (i.e, interruption time reduction, throughput performance during mobility) can be considered as reference for these additional solutions.

Proposal 6: Study and identify additional solutions needed for efficient 5G-6G inter-RAT mobility while considering NR-LTE IRAT mobility procedure as baseline.

## 2.5		Migration and Multi-RAT Spectrum Sharing

As indicated in the objectives of the study item [1], multi-RAT spectrum sharing (MRSS) is the primary spectrum migration option. Under MRSS, 6GR and NR share the spectrum (carrier) in a flexible and dynamic manner with suitable coordination across two RATs. As a part of 6G MRSS design guidelines, we observe the need for Day One operation and design simplicity, enabling faster and simpler MRSS migration for 6G. However, the way the two RATs share a carrier should be to a large extent transparent to the 6G UEs and needs to be totally transparent to the 5G UEs. Furthermore, the design should have very limited impact to RAN2 specifications. The exact need (if any) for RAN2 study for MRSS depend on how the 5G and 6G physical layers will co-exist, and there is not much that RAN2 needs to do until the RAN1 MRSS design has matured. In the MRSS setup the deployed 5G spectrum can be shared with new 6G carriers enabling soft migration from 5G to 6G and allowing the network to balance load between the two systems according to the traffic requirements. Furthermore, the 6G spectrum aggregation can be used to aggregate multiple 6G carriers e.g. to connect the UE to a 6G coverage carrier and capacity carrier at the same time. 6G CA should remain agnostic to whether some of the aggregated carriers are MRSS carriers or not.

As a reference to LTE-5G DSS, there were certain 5G control channel configuration options designed for DSS, some PDSCH DMRS locations were motivated by DSS, and the LTE CRS rate-matching configuration for the 5G UEs was the additional RRC functionality specifically needed for DSS. One should be able to expect that the RAN2 impacts due to MRSS may remain on a similar level and will be subject to RAN1 design decisions rather than requiring specific RAN2 studies.

Figure 1: FDD carrier as MRSS, new-band carrier as 6G-only

Proposal 7: RAN2 to assume 5G-6G MRSS as fully transparent to 5G UEs and fully or almost fully transparent to the 6G UEs in the 6G study.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: Study a 6GR unified handover (UHO) procedure unifying selected mobility procedures defined in 5G and 5G-Advanced. The study shall consider the following aspects:

- both UE and NW implementation aspects of the handover procedure.

- eMBB use-case shall be well addressed.

- the value added by each procedure to baseline UHO procedure.

Proposal 2: Study procedures to enable interruption time reduction during mobility, ensure robustness and high cell edge throughput. In particular RAN2 considers the following:

- For interruption time reduction consider early DL synchronization, early UL synchronization and early decoding.

- For ensuring mobility robustness study the procedures that show clear gains versus complexity.

- For ensuring high cell edge throughput prioritize procedures that offers the highest gains.

Proposal 3: RAN2 to study how to reduce measurement overhead and scheduling limitations when compared to 5G. Gap-assisted measurements should consider the following aspects:

- non-uniform gap pattern to support efficient gap-based measurements of deactivated serving cells or for load balancing

- dynamic network-controlled gap use

- gap distribution in time and resource domain across UEs to enable efficient scheduling at the network.

Proposal 4: Study RRM measurements framework for different RRC states under single objective to simplify and unify measurement configurations. Consider reusing measurement results across RRC states and idle mode measurements for fast CA setup in 6G Day One.

Proposal 5: Study and identify the potential gaps of 5G cell reselection. Focus in particular on reselection and handover acceleration when UE moves from INACTIVE to CONNECTED (if INACTIVE-like state is supported in 6G).

Proposal 6: Study and identify additional solutions needed for efficient 5G-6G inter-RAT mobility while considering NR-LTE IRAT mobility procedure as baseline.

Proposal 7: RAN2 to assume 5G-6G MRSS as fully transparent to 5G UEs and fully or almost fully transparent to the 6G UEs in the 6G study.

Observations are not reiterated here due to the space limitation (while these can be found directly in the document’s body).

# References

- RP-252912 Revised SID: Study on 6G Radio”, NTT Docomo, September 2025.

- 3GPP Technical Specification 38.133

- 3GPP Technical Report 22.870, Section 11.4

- 3GPP Technical Report 22.870, Section 9.4

- 3GPP Technical Report 22.870, Table 11.4.6-1

- 3GPP Technical Report 22.870, Section 11.4.3

- 3GPP Technical Report 22.870, Table 9.4.1-1

- 3GPP Technical Report 22.870, Table 9.4.6-1

- J. Stanczak, U. Karabulut and A. Awada, "Conditional Handover in 5G - Principles, Future Use Cases and FR2 Performance," 2022 International Wireless Communications and Mobile Computing (IWCMC), Dubrovnik, Croatia, 2022, pp. 660-665, doi: 10.1109/IWCMC55113.2022.9824571.

- S. Bin Iqbal, A. Awada, U. Karabulut, I. Viering, P. Schulz and G. P. Fettweis, "On the Modeling and Analysis of Fast Conditional Handover for 5G-Advanced," 2022 IEEE 33rd Annual International Symposium on Personal, Indoor and Mobile Radio Communications (PIMRC), Kyoto, Japan, 2022, pp. 595-601, doi: 10.1109/PIMRC54779.2022.9977719.