3GPP TSG RAN WG2 #131bis			    R2-2507392

Prague, Czech, Oct 13th – 17th, 2025

Agenda item:		10.3.2

Title:						Discussion on 6GR Control Plane

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:		Discussion

# Introduction

3GPP RAN has started the new “Study on 6G Radio” [1], which will lay the foundations of the upcoming 6G technology. We presented our high-level vision of what is most important to make 6G successful during RAN Meeting #109 [2] and first RAN considerations in [3]. In this contribution, we provide a technical vision for the control plane of 6G Radio (6GR) air interface. We discuss how the existing spectrum aggregation and basic control plane functionality could be streamlined and improved to match the most common deployment characteristics optimally while reducing UE and network energy consumption.

# Control Plane design for 6G

Long-established operators typically have a large number of fragmented spectrum assets and because of that the spectrum aggregation framework is essential to provide the best possible service to end users. However, the existing spectrum aggregation framework for 5G NR has grown to be overly complex and yet inflexible in a certain sense.

The basis of existing framework comes from a time where bands had similar bandwidths (10 – 20 MHz) and not so different propagation characteristics (e.g. 2.1 vs 2.6 GHz). Nowadays, 5G NR bandwidths differ 3 orders of magnitude (3 – 400 MHz) and propagation characteristics are completely different (e.g. FDD bands vs C-band vs FR2). This needs to be considered on a better aggregation framework for 6GR.

One issue with the existing carrier and carrier aggregation framework is that, in essence, every carrier is designed to be able to support a fully standalone system with that carrier only. Thus, carriers provide the whole support of functionalities and full overhead: SSBs, SIBs, paging, PRACH, RS, etc. Also, these overheads are multiplied by the number of analog beams used by the deployment (up to 8 in FR1, up to 64 in FR2). And yet in most cases carriers are used on a multi-carrier deployment. The sheer number of newly standardized features to turn off or remove functionality from carriers is a great indication that they are not always needed: SCell dormancy (Rel-16), Inter band SSB-less operation (Rel-18), SSB on-demand for SCells (Rel-19), SIB1 on-demand for multi-carrier scenarios (Rel-19). Always using fully-fledged carriers in all bands is unnecessary, reduces capacity and is detrimental to UE and network energy consumption.

Observation 1: Always using fully-fledged carriers in all bands is unnecessary, reduces capacity and is detrimental to UE and network energy consumption.

Another issue with the existing spectrum aggregation mechanism is that UEs tend to be anchored to a layer with more capacity (e.g. on C-band). Unfortunately, these are also typically layers with uplink coverage issues. The introduction of a supplementary uplink band is sought to help solve this issue, but it is not the most robust nor the simplest way to do it. In fact, as shown on the Figure 1 below, while 5G operates on the philosophy: “capacity first, all bands have duplexing, all bands have common signals”, 6GR could move to the philosophy: “coverage first, duplexing is flexible (multi-carrier), signals on-demand”. This would create a simpler, neater and more robust system.

Figure 1 – 6GR carrier aggregation can be made simpler, reduce overhead, increase capacity and reduce power consumption both at UE and network

While most mobile Internet traffic is downlink (92%),  FDD bands have equal share of downlink and uplink resources, while TDD bands in practice have an asymmetric but still static and unmatching share of resources: 76% DL / 24% UL . So far dynamic TDD did not become practical – full alignment is needed even among operators and DDDSU with S: 10D:2G:2U is most widely used. A wiser strategy to handle current traffic would be to have all (8%) uplink traffic on FDD bands – which anyway have more uplink resources (50%) and much better uplink coverage and use TDD bands for 100% downlink traffic. Given the asymmetry of traffic, the 24% UL time on TDD bands is not gained uplink capacity – it is lost downlink capacity.  Duplexing could be instead achieved with multi-carrier operation.

Observation 2: TDD deployment is practically restricted to use 24% UL time. If this time was used for DL it could increase 30% of peak data rate and capacity. Duplexing could be instead achieved with flexible multi-carrier operation.

Given all these considerations we propose that 6GR spectrum aggregation framework shall consider the following characteristics:

- 6GR is optimized to be deployed with control plane at low frequencies, especially FDD

- Duplexing in 6GR is designed more flexibly with flexible pairing a set of DL and UL bands

- 6GR adopts two different types of carriers: fully-fledged carriers (with SS, SIB, Paging, PRACH, mobility) and simplified carrier (capacity boosting only).

- 6GR Simplified carriers (capacity boosting) can operate in a single duplexing direction (downlink only or uplink only)

Proposal 1: 6GR is optimized to be deployed with control plane at low frequencies, especially FDD

Proposal 2: Duplexing in 6GR is designed more flexibly with flexible pairing of DL and UL bands

Proposal 3: 6GR adopts two different types of carriers: fully-fledged carriers (with SS, SIB, Paging, PRACH, mobility) and simplified carrier (capacity boosting only).

Proposal 4: 6GR Simplified carriers (capacity boosting) can operate in a single duplexing direction (downlink only or uplink only)

In 5G NR, the multi-carrier nature of the deployment is most pervasive in RRC_CONNECTED mode. In RRC_IDLE/RRC_INACTIVE the UE may perform cell re-selection among frequency layers but it does not really know whether a particular cell belongs to a set of cells of the same site/sector. This leads to suboptimal overhead where System Information and Paging have to be broadcast in all layers and PRACH resources have also to be configured in all layers. Such approach is suboptimal both in terms of overhead as well as Network Energy consumption.

Observation 3: 5G NR has suboptimal overhead on co-located multi-carrier deployments.

Instead, 6G can be designed such that high frequency layers avoid the typical broadcast common channels (system information, paging) and instead point to lower frequencies where such information can be obtained. On-demand SIB1 (Rel-19) was a first step in that direction, but with a clean slate design it can be done much more efficiently and have wide adoption.

Proposal 5: At very high frequencies (e.g. FR2) a carrier broadcasts reduced information on how to connect to a lower frequency (e.g. in FR1) instead of System Information

Another consideration is the need for paging in multiple carriers. In 5G NR a paging message needs to be sent in any frequency layer where the UE may be camping, even if just temporarily until cell re-selection is performed. This is a significant waste of resources as it is well known that coverage area on different carriers is widely different. Instead, it would be much more efficient to only page on the carriers which have a good coverage and good outdoor-to-indoor penetration properties.  In order to do that, 6G system information allows the UE to distinguish paging layers from non-paging layers.

Proposal 6: 6GR system information allows the UE to distinguish paging layers from non-paging layers

As possible paging and access design strategies in 5G one can favor coverage by configuring the highest priority to a low-frequency carrier (e.g. below 1 GHz) or favor capacity by configuring the highest priority to the carrier with largest bandwidth (e.g C-band). The UE will camp and access via a certain band. In general, however, paging is more efficient when done at frequencies with better coverage whereas the access itself needs to take into account the load on different carriers (PRACH load and traffic channel load)

Observation 4: In 5G NR one has to favor coverage or capacity when configuring layer priorities. However, paging is more efficient when done at frequencies with better coverage whereas the access itself needs to take into account the load on different carriers (PRACH load and traffic channel load)

This trade-off can be greatly improved by decoupling paging layers from access layers, i.e., a UE should be allowed to camp on a layer where paging is more efficient and be re-directed to perform random access on another layer as requested by the network.

Proposal 7: 6GR adopts decoupling paging layers from access layers, i.e., a UE should be allowed to camp on a layer where paging is more efficient and be re-directed to perform random access on another layer as requested by the network.

# Conclusions

In this contribution, we discussed an aspects of the 6G air interface control plane with the following observations and proposals:

Observation 1: Always using fully-fledged carriers in all bands is unnecessary, reduces capacity and is detrimental to UE and network energy consumption.

Observation 2: TDD deployment is practically restricted to use 24% UL time. If this time was used for DL it could increase 30% of peak data rate and capacity. Duplexing could be instead achieved with flexible multi-carrier operation.

Observation 3: 5G NR has suboptimal overhead on co-located multi-carrier deployments.

Observation 4: In 5G NR one has to favor coverage or capacity when configuring layer priorities. However, paging is more efficient when done at frequencies with better coverage whereas the access itself needs to take into account the load on different carriers (PRACH load and traffic channel load)

Proposal 1: 6GR is optimized to be deployed with control plane at low frequencies, especially FDD

Proposal 2: Duplexing in 6GR is designed more flexibly with flexible pairing of DL and UL bands

Proposal 3: 6GR adopts two different types of carriers: fully-fledged carriers (with SS, SIB, Paging, PRACH, mobility) and simplified carrier (capacity boosting only).

Proposal 4: 6GR Simplified carriers (capacity boosting) can operate in a single duplexing direction (downlink only or uplink only)

Proposal 5: At very high frequencies (e.g. FR2) a carrier broadcasts reduced information on how to connect to a lower frequency (e.g. in FR1) instead of System Information

Proposal 6: 6GR system information allows the UE to distinguish paging layers from non-paging layers

Proposal 7: 6GR adopts decoupling paging layers from access layers, i.e., a UE should be allowed to camp on a layer where paging is more efficient and be re-directed to perform random access on another layer as requested by the network.

# References

- RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO (Moderator), 3GPP TSG RAN Meeting #109 Beijing, China, September 15-18, 2025.

- RP-251515, “Views on New RAN WG SI on 6G”, Fraunhofer IIS, Fraunhofer HHI, 3GPP TSG RAN Meeting #108 Prague, Czech Republic, June 9-13, 2025.

- R1-2505813, “Overview of 6GR Air Interface”, Fraunhofer IIS, Fraunhofer HHI, 3GPP RAN1 Meeting #122 Bangaluru, India, August 25-29, 2025.