3GPP TSG-RAN WG2 Meeting #131bis	R2-2507409

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.10.2

Source:	Nokia

Title:	[RIL] N064, N065: RLF, SHR logging when CHO only HO is performed

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

During the Rel-19 CR review we have discovered a couple of problems with RLF, SHR logging when CHO only HO is performed ([RIL] N064, N065). This paper discusses these issues.

# 2	Discussion

Late in the release, a discussion was started about the coexistence of CHO with candidate SCGs (CwcS) and complementary CHO-only configurations for a given UE. At RAN2#131 the following agreements we achieved regarding this scenario:

UE reports the information of fulfilled condition(s), i.e., CHO, or CPAC, or neither, when CHO only configuration is received, in RLF report and/or SHR report.

UE reports following information in RLF report and SHR report:

－	Identifier of candidate PCell(s) which met the CHO execution conditions when CHO only configuration is received

－	Identifier of candidate PSCell(s) which met the CPAC execution conditions when CHO only configuration is received

This is an optional UE feature, TBD with or without signalling.

There are a couple of issues with the current agreements and their implementation in the 38.331 CR with respect to MRO related enhancements being considered in case of having a complementary CHO-only configuration in addition to the CHO with candidate SCG (CwcS) configuration. The agreements address RLF and legacy SHR cases for the scenario when UE operating in DC mode and being configured with CwcS will be configured with CHO-only later on in addition.

Issue 1: Restriction to scenario when CHO-only is configured after the CHO with candidate SCG

2>	if after receiving this CHO with candidate SCG configuration, the UE received a conditional handover configuration including condRRCReconfig for the same target candidate PCell as set in pCellId:

Our understanding is that the interesting case is when the CHO-only configuration is a complimentary configuration of CHO with candidate SCG configuration. In this case it is expected that they are configured together. When the CHO-only and the CHO with candidate SCG are configured independently, they are independent mechanisms that should be optimized independently based on already specified MRO mechanisms. It is also worth noting that optimizations of independent configurations together were rejected by RAN3.

Observation 1: Logging the new additional information (fulfilledConfigWhenChoOnly) is only relevant when the CHO-only and the CHO with candidate SCG were configured together.

Issue 2: Relevance of the new IE fulfilledConfigWhenChoOnly in case of RLF

3>	set fulfilledConfigWhenChoOnly to cho if condExecutionCond was fulfilled at the time of receiving the conditional handover configuration or cpc if condExecutionCondPSCell was fulfilled at the time of receiving the conditional handover configuration, otherwise set fulfilledConfigWhenChoOnly to neither;

In case of RLF, the new IE which provides information about the fulfilled conditions of an already existing CwcS configuration "at the time of receiving the conditional handover configuration" is not helpful, since the situation when RLF occurred is the relevant on and not the situation at the point of time when CHO-only configuration was received. The situation regarding the fulfilment of the CwcS condition could have changed a lot between configuration reception and RLF.

Observation 2: Since RLF may occur way later than the CHO-only configuration received, the information about the fulfilled conditions at the time when CHO-only configuration received may be irrelevant.

Moreover, the specification already specifies the capturing of all the relevant information in RLF reports:

2>	if all triggering events of both condExecutionCond and condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled:

3>	set firstFulfilledConfig to cho if condExecutionCond was fulfilled first or cpc if condExecutionCondPSCell was fulfilled first;

3>	set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions;

2>	else if all triggering events of only one of the condExecutionCond or condExecutionCondPSCell of the concerned entry of condReconfigList is fulfilled:

3>	set firstFulfilledConfig to cho or cpc, whichever was fulfilled;

3>	set timeBetweenLastFulfillmentAndEvent to the elapsed time between the point in time of fulfilling the last triggering event of the fulfilled execution condition and the RLF;

Which means that all relevant information is logged

Observation 3: Logging of all relevant information related to RLF related to the CHO with candidate SCG configuration has already been specified.

Issue 3: Relevance of new information for legacy SHR cases

5>	set fulfilledConfigWhenChoOnly to cho if condExecutionCond was fulfilled at the time of receiving the conditional handover configuration or cpc if condExecutionCondPSCell was fulfilled at the time of receiving the conditional handover configuration, otherwise set fulfilledConfigWhenChoOnly to neither;

Assuming that the near-failure for the SHR was caused by thresholdPercentageT310, which is detected before any PCell change is triggered and refers to a critical situation with the MCG link shortly before CHO has been triggered. We see the same issue as for the RLF case that the time when UE was configured in addition with CHO-only is different from the point in time when near failure occurred and, therefore, having this information about the fulfilled conditions of an already existing CwcS configuration when CHO-only configuration is received is irrelevant for the root case analysis as it is for the RLF case.

Irrespective of which sort of handover method will be carried out, the legacy SHR indicates a risky and almost too late PCell change, and the standard already guarantees that all relevant information is logged.

Observation 4: The new information is irrelevant for the root case analysis of the legacy SHR cases.

Furthermore, the new information is logged in SHR, but there is no information on which type of HO was actually executed (there is no lastHO -type IE in SHR). Therefore, the network will not know which parameters belong to which mobility mechanism (CHO with candidate SCGs or CHO-only) to fix.

Observation 5: The network does not know that the failure case reported in SHR belongs to a CHO with candidate SCG or a CHO-only handover.

Issue 4: Complimentary CHO-only is carried out even though CHO with candidate SCGs is configured, too

No SHR is generated when a complementary CHO-only is performed without fulfillment of any legacy SHR triggers, but there was a configuration for CHO with candidate SCGs available. Since CwcS is the preferred mobility option for UEs operating in DC mode (and even standard always guarantees the preference), the network should be informed about this case to be able to optimize the configuration.

Observation 6: Based on the current SHR triggers the network is not informed when a complimentary CHO-only is carried out while CHO with Candidate SCG is configured

Based on the observations listed above we have the following proposals:

Proposal 1: RAN2 confirms that the relevant case is when a complimentary CHO-only is executed when the CHO with candidate SCG is also configured.

Proposal 2: RAN2 agrees that the current specification does not fully address the case when a complimentary CHO-only is executed when the CHO with candidate SCG is also configured (e.g., nothing is logged when the two configurations are received together, and SHR is only created if one of the legacy near failure scenario happens).

Proposal 3: RAN2 agrees that based on current specification the UE logs information that is not relevant for the root cause analyses (e.g., logging happens about the condition fulfilment status at the time of configuration of the CHO-only is received).

Proposal 4: RAN2 agrees to address the case when a complimentary CHO-only is executed when the CHO with candidate SCG is also configured by logging the relevant information when this happens.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: Logging the new additional information (fulfilledConfigWhenChoOnly) is only relevant when the CHO-only and the CHO with candidate SCG were configured together.

Observation 2: Since RLF may occur way later than the CHO-only configuration received, the information about the fulfilled conditions at the time when CHO-only configuration received may be irrelevant.

Observation 3: Logging of all relevant information related to RLF related to the CHO with candidate SCG configuration has already been specified.

Observation 4: The new information is irrelevant for the root case analysis of the legacy SHR cases.

Observation 5: The network does not know that the failure case reported in SHR belongs to a CHO with candidate SCG or a CHO-only handover.

Observation 6: Based on the current SHR triggers the network is not informed when a complimentary CHO-only is carried out while CHO with Candidate SCG is configured

Proposal 1: RAN2 confirms that the relevant case is when a complimentary CHO-only is executed when the CHO with candidate SCG is also configured.

Proposal 2: RAN2 agrees that the current specification does not fully address the case when a complimentary CHO-only is executed when the CHO with candidate SCG is also configured (e.g., nothing is logged when the two configurations are received together, and SHR is only created if one of the legacy near failure scenario happens).

Proposal 3: RAN2 agrees that based on current specification the UE logs information that is not relevant for the root cause analyses (e.g., logging happens about the condition fulfilment status at the time of configuration of the CHO-only is received).

Proposal 4: RAN2 agrees to address the case when a complimentary CHO-only is executed when the CHO with candidate SCG is also configured by logging the relevant information when this happens.