3GPP TSG-RAN WG2 #131bis	R2-2507429

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	9.2.2

Source:	Ericsson

Title:	Aspects for Ambient IoT Topology 2

Document for:	Discussion/Decision

# 1	Introduction

The RAN2 objectives for the work item on Ambient IoT Rel-20 is defined as the following in [1].

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]:

- According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769.

  - No specification support for CW control

- According to D2T2-C for active Device(s).

  - Start after March 2026

- Traffic types DO-DTT and DT, and DO-A (only for active Device(s)).

  - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026

- Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command).

- FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum.

- Specify RRC-based architecture, in coordination with SA2.

- The following are not supported:

  - Coordination between UEs operating as UE readers

  - New Uu protocol layer(s)

  - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band

This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1.

The work is subject to the following constraints:

RAN2

- The objective includes specification of necessary Uu procedures to enable the A-IoT operations.

- RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios.

- A-IoT radio resource of the UE reader is only valid within the serving cell.

- Resource allocation to the UE Reader shall be via Uu RRC signaling.

We focus on aspects for Topology 2 including device addressing, handling out of connection and resource allocation in this paper.

# 2	Discussion

RAN2 has studied different aspects of Topology 2 during the Rel-19 study phase. The study outcome has been captured in the TR 38.369. As a general guideline, we think RAN2 shall focus on the study outcome captured in the TR. New options for the issues captured in the TR shall not be discussed unless it is justified.

In addition, this contribution focuses on single UE reader, single service scenario...

RAN2 sticks to the options captured in the TR 38.369. New options shall not be discussed unless it is fully justified.

## 2.1 Device AS ID handling

For Topology 1, it has been agreed device AS ID is applied at least for Inventory + command case and is assigned by reader. It is up to reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. This principle should be reused for Topology 2, i.e., device AS ID is assigned by the UE reader and it is up to the UE reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. If the device AS ID is assigned by the gNB, in CBRA case, the UE reader needs to inform the gNB that a random ID is received in Msg1, and gNB assigns a device AS ID and informs it to the UE reader. This requires extra signalling over Uu and unnecessarily prolongs the AIoT procedure. Some one may concern that the same AS ID may be assigned to devices across different UE readers, which may lead to confusion of AS ID among UE readers. However, we don’t think this is a valid concern since we can rely on the gNB implementation to distinguish the devices associated with the same AS ID but operated by different UE readers via for example, a combination of the device’s AS ID plus the UE reader’s Uu ID, e.g., C-RNTI.

- In Topology 1 device AS ID is assigned by reader and it is up to reader to decide whether to reuse the random ID.

- Allocating device AS ID by gNB in Topology 2 will lead to extra signalling overhead over Uu.

- we can rely on the gNB implementation to distinguish the devices associated with the same AS ID but operated by different UE readers via for example, a combination of the device’s AS ID plus the UE reader’s Uu ID, e.g., C-RNTI.

Reuse Rel-19 AS ID allocation mechanism for D2T2, i.e., device AS ID is assigned by the UE reader and it is up to the UE reader to decide whether to reuse the random ID as AS ID.

## 2.2 How to generate transaction ID

In Rel.19 it has been agreed that the transaction ID can be generated by reader based on CN corelation ID and how to generate transaction ID is left to reader implementation. In Topology 2, however, if the transaction ID is generated by UE reader, it could happen different UE readers map the same CN correlation ID to different transaction IDs and/or map different CN correlation IDs to the same transaction ID, unless there are coordination between UE readers which are not supported. For the former case the device will send duplicate responses to subsequent paging unnecessarily, for the latter case the device will regard the later received A-IoT paging message as subsequent paging message and ignore mistakenly if it has successfully responded earlier. To resolve this the gNB could generate the transaction ID and inform it to the UE reader (at least for CBRA) thus avoid the aforementioned problems. For CFA, RAN2 agreed that transaction ID is not included in paging message, therefore there is no need for the gNB to generate the transaction ID and inform it to the UE reader.

- In Topology 1 transaction ID is generated by reader. This leads to duplicate responses to subsequent paging and/or ignoring a paging message mistakenly.

In Topology 2 the transaction ID is generated by the gNB and informed to the UE reader at least for CBRA.

## 2.3 A-IoT operation when temporary out of connection

The WID states the RRC_CONNECTED reader including RLF and handover cases and radio resources for A-IoT are only valid in the serving cell.  Regardless of if the UE reader has occurred out of connection due to RLF or handover, it is beneficial for RAN2 to aim for designing a unified procedure to handle out of connection in terms of simplified UE reader behaviours and reduced design efforts for RAN2.

- it is beneficial for RAN2 to aim for designing a unified procedure to handle out of connection in terms of simplified UE reader behaviours and reduced design efforts for RAN2.

Aim for designing a unified way of handling out of connection in terms of A-IoT radio resource allocation for UE reader regardless of RLF or handover.

A-IoT device type 1 behaviour is transparent to whether UE reader is in RLF or handover.

For A-IoT operation when temporary out of connection, two options have been studied and captured in TR38.369:

-	Scenario 2: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases):

-	Option 1: The UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition.

-	Option 2: The UE reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations.

In our view option 1 is better because if the UE reader does not recover from out of connection it will not be able to deliver buffered D2R upper layer data to the NW nor command the A-IoT devices. Therefore, it may be a waste that the UE reader schedules A-IoT transmissions  upon temporary out of connection. In addition, it may be a risk to cause interference to other UE(s) by the UE reader if the UE reader continues using the radio resources allocated to the UE reader before out of connection occurs, since the UE reader in out of connection may already lose UL sync towards the gNB. We think the ongoing A-IoT session/procedure should be suspended. On the other hand, for D2R transmissions that are already triggered before the suspension, it could be up to the UE reader implementation whether to receive the D2R transmissions.

Adopt Option 1 in TR 38.169 for handing out of connection, i.e., the UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition.

The UE reader stops sending R2D message for ongoing A-IoT session/procedure during the temporary out of connection condition. FFS whether UE reader suspends or releases/terminates the session/procedure.

Besides, a suspended A-IoT session/procedure can be resumed after the connection is restored if the UE reader restores the connection in the same gNB, using the previous configured resources, or resources re-configured by the gNB. The suspended A-IoT session/procedure should be terminated in case the connection cannot be restored.

The UE reader resumes A-IoT session/procedure if UE reader is able to recover from the temporary out of connection condition.

The UE reader terminates A-IoT session/procedure if the UE reader cannot recover from the temporary out of connection condition.

If the UE reader can recover from the temporary out of connection condition, the UE reader may be provided with new resources if the UE reader restores its connection from a different cell. In this case, the UE reader can send a R2D signaling to reconfigure the resources to devices. it can be further discussed if the paging message can be applied for reconfiguring the resources to devices in the same paging round.

The UE reader may reconfigure radio resources to devices after its connection has been restored. FFS if the paging message can be applied.

## 2.4 Enhancements on Uu procedures to enable A-IoT operations

For Topology 2 only RRC-based architecture is considered. How to enhance Uu procedure to incorporate A-IoT operations should start from type of A-IoT information be delivered over Uu interface. We think there are two types of A-IoT information, A-IoT upper layer data and A-IoT control information.

- A-IoT upper layer data is information to be terminated at upper layer (A-IoT NAS), including, e.g.,:

  - Inventory request/response, command request/response,

- A-IoT control information is to be consumed by A-IoT AS sublayer, i.e., device and reader, including, e.g.,:

  - Assistance information (originated from CN) for a paging round sent from NG-RAN to UE reader e.g., approximate number of devices, etc

  - A-IoT resource management related information between NG-RAN and UE reader such as resource allocation signaling, resource consumption status and resource rejection from UE reader, resource reconfiguration, RLF and HO control signaling.

- A-IoT information to be exchanged over legacy Uu interface includes A-IoT upper layer data (inventory, command request and reply) and A-IoT control information (for assistance information and resource management).

One related question is how to transfer A-IoT information over Uu between UE reader and gNB. In our view, the A-IoT control information can be exchanged over existing RRC signaling, e.g., reconfiguration/SRB1. Whereas a new SRB can be defined to carry A-IoT upper layer data with new RRC messages to be defined

Define a new SRB for delivery of A-IoT upper layer data. FFS details.

Reusing existing SRB for delivery of A-IoT control information (e.g., A-IoT resource management). FFS details (including whether existing RRC messages are reused).

Figure 1: RRC based protocol stack

SA2 has discussed the RRC based protocol stack as shown in Figure 1. RAN2 can base on this protocol stack for further discussions.

RAN2 bases the RRC based protocol stack in Figure 1 to conduct discussions.

## 2.5 A-IoT resource allocation

In Topology 2 gNB is in charge of the A-IoT resource allocation and the resource shall be allocated to the UE reader via Uu RRC signaling. In TR38.369, two options have been captured for A-IoT resource allocation to UE reader in RRC_CONNECTED:

- Option 1:	UE reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly. This would be supported by the NR Uu RRC reconfiguration.

- Option 2: UE reader receives the resources configuration in NR Uu dedicated RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the resources remain valid for that time period, unless the resource configuration is explicitly released by the network by the NR Uu RRC reconfiguration.

In our view, it is enough to support option 2, which comprises option 1 by making the time period an optional parameter or setting the time period to a very large value (e.g., infinite).

A-IoT resource configured to UE reader is associated with an optional validity time period.

The resource is valid for that time period (if present) or until explicitly released by the network.

One related question is how (based on what) a serving gNB can allocate A-IoT resources.

For the radio resources allocation request, following alternatives are studied and captured in the TR38.369::

-	Alternative 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or

-	Alternative 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN.

NOTE 2:	The above two alternatives can co-exist. The configuration for radio resource allocation may also include the validity criteria (if applicable, see the time period in previous option 2 of those scenarios)

We think the above two alternative can co-exist. Alternative 2 shall be always applied by the gNB upon reception of a service request from CN.

However, Alternative 1 can be applied by the UE reader when the UE reader sees that the set A-IoT radio resources allocated by the gNB are not sufficient, e.g.,

- the UE reader may need to initiate subsequential paging towards devices, ,

- the UE reader may have fallen short of resources in current paging round, and thus, requires more resources to complete paging round (especially in case gNB has no good information of device population in the region)

which is beyond the resource validity period of the set A-IoT radio resources configured by the gNB.

Further, in order to help UE reader to execute A-IoT resource allocation to devices, especially in a CBRA manner, it would be beneficial if gNB provides assistance information it receives from CN over NGAP, e.g., approximate number of devices, D2R message sizes, etc. Such information can be part of RRC signalling

Adopt both alternatives for A-IoT radio resource allocation

        - Alternative 1: gNB configures/allocates a set A-IoT radio resource to the UE reader in response to a request from the UE reader via RRC resource allocation message.

        - Alternative 2: gNB configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN via RRC resource allocation message.

The RRC resource allocation message can include (filtered) assistance information based on A-IoT assistance information from CN to gNB over NGAP.

In order to apply Alternative 2, the UE reader may provide assistance information to the serving gNB for A-IoT resource allocation. The A-IoT resource allocation to the UE reader can fall short as discussed above but also can also be overbooked. Hence in either case, UE reader should be able provide assistance information to gNB regarding need for more resources or indicate unused resource in case of overbooking.

- The A-IoT resource refinement can be associated with

- Requesting additional resources if UE reader falls short of resources.

UE reader can provide assistance information to the serving gNB for A-IoT resource allocation refinement.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In Topology 1 device AS ID is assigned by reader and it is up to reader to decide whether to reuse the random ID.

Observation 2	Allocating device AS ID by gNB in Topology 2 will lead to extra signalling overhead over Uu.

Observation 3	we can rely on the gNB implementation to distinguish the devices associated with the same AS ID but operated by different UE readers via for example, a combination of the device’s AS ID plus the UE reader’s Uu ID, e.g., C-RNTI.

Observation 4	In Topology 1 transaction ID is generated by reader. This leads to duplicate responses to subsequent paging and/or ignoring a paging message mistakenly.

Observation 5	it is beneficial for RAN2 to aim for designing a unified procedure to handle out of connection in terms of simplified UE reader behaviours and reduced design efforts for RAN2.

Observation 6	A-IoT information to be exchanged over legacy Uu interface includes A-IoT upper layer data (inventory, command request and reply) and A-IoT control information (for assistance information and resource management).

Observation 7	The A-IoT resource refinement can be associated with

a.	Requesting additional resources if UE reader falls short of resources.

Proposal 1	RAN2 sticks to the options captured in the TR 38.369. New options shall not be discussed unless it is fully justified.

Proposal 2	Reuse Rel-19 AS ID allocation mechanism for D2T2, i.e., device AS ID is assigned by the UE reader and it is up to the UE reader to decide whether to reuse the random ID as AS ID.

Proposal 3	In Topology 2 the transaction ID is generated by the gNB and informed to the UE reader at least for CBRA.

Proposal 4	Aim for designing a unified way of handling out of connection in terms of A-IoT radio resource allocation for UE reader regardless of RLF or handover.

Proposal 5	A-IoT device type 1 behaviour is transparent to whether UE reader is in RLF or handover.

Proposal 6	Adopt Option 1 in TR 38.169 for handing out of connection, i.e., the UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition .

Proposal 7	The UE reader stops sending R2D message for ongoing A-IoT session/procedure during the temporary out of connection condition. FFS whether UE reader suspends or releases/terminates the session/procedure.

Proposal 8	The UE reader resumes A-IoT session/procedure if UE reader is able to recover from the temporary out of connection condition.

Proposal 9	The UE reader terminates A-IoT session/procedure if the UE reader cannot recover from the temporary out of connection condition.

Proposal 10	The UE reader may reconfigure radio resources to devices after its connection has been restored. FFS if the paging message can be applied.

Proposal 11	Define a new SRB for delivery of A-IoT upper layer data. FFS details.

Proposal 12	Reusing existing SRB for delivery of A-IoT control information (e.g., A-IoT resource management). FFS details (including whether existing RRC messages are reused).

Proposal 13	RAN2 bases the RRC based protocol stack in Figure 1 to conduct discussions.

Proposal 14	A-IoT resource configured to UE reader is associated with an optional validity time period.

Proposal 15	The resource is valid for that time period (if present) or until explicitly released by the network.

Proposal 16	Adopt both alternatives for A-IoT radio resource allocation

a.	Alternative 1: gNB configures/allocates a set A-IoT radio resource to the UE reader in response to a request from the UE reader via RRC resource allocation message.

b.	Alternative 2: gNB configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN via RRC resource allocation message.

Proposal 17	The RRC resource allocation message can include (filtered) assistance information based on A-IoT assistance information from CN to gNB over NGAP.

Proposal 18	UE reader can provide assistance information to the serving gNB for A-IoT resource allocation refinement.

# 4	References

RP-252894, “Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2”, Huawei, 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025.