3GPP RAN WG2 Meeting #131bis	R2-2507433

# Prague, Czechia,  October 13th – 17th, 2025

Agenda Item:	10.3.2

Source:	InterDigital, Inc.

Title:	Control Plane for 6GR.

Document for:	Discussion

# Introduction

The study on 6G contains the following CP related objectives.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

This contribution identifies control plane issues for 6G excluding mobility. The aim is to highlight where NR design created inefficiency or complexity, and where 6G can deliver simplification, scalability, and improved energy efficiency.

# Discussion

## 2.1	6G CP Design Considerations

Control plane design for 6G should be lean and scalable, avoiding the fragmentation and inefficiencies observed in NR. The following drawbacks from NR should be considered as lessons learned, and the following design targets should guide 6G.

The following drawbacks from NR should be considered:

- Always-on SSB/SIB1 created high network and UE energy consumption, especially in multi-carrier deployments.

- System Information (SI) is static, redundant, and unprotected, creating overhead and spoofing risks.

- Idle/Inactive states overlap, with Inactive adoption low due to limited benefit vs. Idle.

- RRC reconfiguration requires full messages with deep ASN.1 nesting, creating large overhead and latency.

- UE capability signalling has become excessively large, with poor support for dynamic changes.

- ASN.1 design in NR is complex and error-prone, making delta signalling cumbersome at times.

- Access control is coarse and not service- or device-aware, limiting vertical support.

6G offers an opportunity to streamline design for wider adoption and the following design targets should be considered to achieve that:

- Simplification: Reduce signalling overhead, remove redundancy, and streamline RRC state handling.

- Energy efficiency: Enable native support for network and UE energy saving from Day-1.

- Robustness: Ensure control plane procedures are resilient across diverse services and devices.

- Low latency: Provide fast transitions between states, lean reconfiguration, and minimized attach time.

- Modularity: Stored and reusable RRC profiles or configuration parts avoid full reconfiguration / full message refreshes.

- Service-awareness: Adapt control plane procedures to traffic/service type, device capability, and QoS.

- Scalability: Ensure a single framework supports IoT, XR, NTN, and eMBB devices efficiently.

Observation: NR control plane suffers from high overhead, overlapping states, and complex signalling. 6G should use single framework that minimizes complexity while being scalable, energy-efficient, and service-aware to deliver low-latency and robust operation.

## 2.2	Initial Access

In NR, SSB and SIB1 are transmitted periodically and always-on. This is a major source of UE and network power consumption, especially in multi-carrier deployments where each carrier duplicates overhead. In Rel-19 SIB1-less operation was introduced, however for backwards compatibility with legacy devices the applicability is somewhat limited.

Always-on design does not scale to diverse devices (IoT, XR, eMBB) or dense deployments. It also prevents efficient spectrum utilization. More importantly, it does not provide sufficient network energy saving (NES), where cells or beams may periodically switch off or operate with reduced signalling. On-demand transmissions cut always on transmission overhead allowing for network power saving. Multi-carrier-aware initial access enables flexible use of coverage and capacity layers. Access mechanisms should also take into account cases where SSB may be transmitted with different periodicity or not transmitted from some cells/carriers at a specific time.

Initial access mechanisms should therefore support:

- SSB transmission without mandatory SIB1 on every carrier, decoupling coverage and capacity layers.

- On-demand SIB1, reducing broadcast overhead and NW power consumption.

- Multi-carrier aware access, where coverage carriers provide full attach information while capacity carriers may provide lighter or conditional signalling.

- Cases where SSB is transmitted at longer periodicities or temporarily absent on some carriers to enable NES.

Proposal 1: Support on-demand System Information, including SSB transmission without SIB1 (SIB1-less), multi-carrier aware access, and cases where SSB may not always be present.

## 2.3	System Information

In NR, SIB1 is always broadcast (though Rel-19 introduces SIB-less operation), while other SIBs are provided on-demand. This reduces overhead compared to legacy LTE but still requires frequent broadcast of SIB1.

6G will support an even wider range of services (IoT, NTN, sensing), increasing the diversity of SI. A flexible framework is needed that goes beyond NR’s SIB1/on-demand split and provides scalable, secure SI delivery. The following aspects need to be considered:

- Service-based SI delivery: For example, device type specific SIBs ensures UEs only receive relevant information, lowering power consumption. Rather than every UE receiving the same set of SIBs, the network provides different SI content per service, slice, or device type.

- Fine-grained SI granularity: per PLMN/slice, service (or device type), enabling flexible sharing and tailored configurations.

Proposal 2: Support scalable SI delivery for 6G, with service-based tailoring and fine-grained granularity (per-PLMN/slice/service).

## 2.4	RRC States

NR defined three states: Idle, Inactive, and Connected. Inactive was introduced in Rel-15 to reduce latency compared to Idle by retaining UE context, but in practice its benefits over Idle were marginal. Uptake has been limited because Idle and Inactive both offer similar power saving, transition from Idle to Connected is still signalling-heavy, requiring CN involvement and full security/bearer setup, and inactive still requires RRC signalling to resume, which reduces its advantage.

In Rel-17/18, SDT improved the usefulness of Inactive, but the differentiation from Idle remained small. Rel-19 introduced SIB-less operation, but did not fundamentally change the state model due to maintaining backwards compatibility (legacy devices rely on the always-on signals such as SSB so idle mode is difficult to modify).

For 6G, RRC state handling must be simplified and enhanced:

- Idle should be reduced to a temporary “initial attach only” state, used at power-on or after long inactivity/exception, allowing it to be optimised for network power saving because fast cell detection for mobility purposes is not necessary.

- A semi-connected state (evolution of Inactive) should become the main dwell state, combining the energy savings of Idle with the context retention of Connected.

  - UE always has an RNTI or Area-RNTI, allowing direct addressing.

  - RAN, not CN, is responsible for location tracking (RNA scope).

  - Device-type-specific configurations (RedCap, IoT, XR, eMBB) can be tied to RNTI validity and reused across reselections.

- Connected remains the state for full throughput and lowest latency, but should be used less often than in NR – the network can use semi-connected/inactive more aggressively than NR, if state transition times are improved.

Figure: Proposed RRC State Model

Proposal 3: Define the 6G state model with:

- Idle: Initial attach/recovery only.

- Semi-connected/inactive: Default low-power state with stored UE context, RAN-based paging, UE controlled mobility.

- Connected: High-throughput state with full network-controlled mobility and bearer setup.

Many immersive and interactive services (e.g. XR, cloud gaming, sensing-assisted apps) require very low setup delay when switching from background to active sessions. A UE context (e.g. at least a cell specific or area specific RNTI, and associated configurations) stored and maintained in semi-connected state, can enable transition to connected with reduced signalling (e.g., reusing stored config, RACH-less resume, partial reconfiguration). This enables sub-10 ms resume latency for critical services, while maintaining power savings during semi-connected operation.

Proposal 4: Introduce fast transition between Semi-connected/Inactive and Connected states targeting sub-10 ms resume latency, using a stored UE context in the semi-connected state.

## 2.5	Reconfiguration Requirements (incl. MAC Control)

In NR, RRC reconfigurations are heavy, often requiring full configuration messages and large ASN.1 structures. MAC control is fragmented into many small, inflexible formats. As 6G introduces more features, this model would create even heavier signalling, increasing latency, UE complexity, and slowing feature adoption.

A modular reconfiguration framework addresses the size and flexibility issue by restructuring RRC into independent modules, or configuration parts (e.g. mobility, user-plane, security). Each module can be stored, reused, or updated separately, so only the relevant part of the configuration is refreshed when needed. This avoids large full reconfigurations and allows new features to be added without disturbing existing modules.

The LTM mechanism in NR was introduced to reduce latency by allowing mobility-related commands, in particular the cell switch command, to be signalled directly at MAC, instead using full RRC reconfiguration. This principle — moving control closer to the execution point — enables faster reaction to radio conditions and reduces handover latency.

A similar approach can be extended to RRC reconfiguration in general. Even with modular RRC, a small delta update must still traverse the full RRC → PDCP → RLC → MAC path, which creates latency for time-critical adjustments. To address this, extensible MAC Control Elements (MAC CEs) can be used to carry small configuration changes, or to trigger pre-configured (at RRC) reconfigurations, directly at the MAC layer.

Proposal 5: Support modular RRC signalling with reusable profiles, and MAC CE triggered reconfiguration.

## 2.6	Access Control

In NR, access control is relatively static and coarse, mainly distinguishing between broad categories such as MO-Data vs emergency. This approach lacks flexibility and does not adapt to service, slice, or device diversity.

With 6G expected to support IoT, XR, NTN, public safety, and vertical services, static AC is insufficient for several reasons:

- Lack of service differentiation: All MO-Data is treated the same, even though some traffic (e.g. XR or safety-critical) requires higher priority.

- Device diversity: IoT or RedCap devices should not compete with XR headsets or smartphones under identical barring conditions.

- Slice and PLMN awareness: Static AC cannot scope barring to a specific slice or PLMN, so barring one service may unintentionally affect others.

- Energy efficiency: Uniform AC leads to repeated retries and wasted signaling, whereas adaptive AC can defer or relax access attempts intelligently.

For 6G, access control should evolve towards an adaptive framework that is service-aware, device-aware, and slice-aware, while minimizing signalling updates through validity/scope mechanisms.

Proposal 6: Support adaptive access control with per-UE-type, per-service, per-slice granularity, and validity/scope mechanisms.

## 2.7	RRC Signaling Overhead / ASN.1 Restructuring

In NR, ASN.1 has deep nesting (e.g. ServingCell → BWP → CellGroup), which makes RRC messages large, complex, and difficult to maintain. Even with delta signalling, ambiguities and optional fields often lead to different vendor interpretations, creating interoperability risks. As new features and services are added, this structural complexity will only grow, making RRC signalling increasingly heavy and hard to extend.

For 6G, a modular RRC design is needed. Instead of extending one monolithic ASN.1 structure with ever more optional fields, RRC should be organized into independent message classes (“modules”). Each module (e.g. mobility, UP, measurement, security) can be defined and maintained separately, reducing nesting, avoiding unnecessary cross-dependencies, and making it easier to introduce new features. Optimized encoding for repetitive elements (e.g. band combinations) further reduces message size and complexity. Together, these measures simplify specification maintenance, improve interoperability, and ensure forward compatibility.

Proposal 7: ASN.1 should be designed toward modular RRC configuration parts and optimized encoding for repetitive elements (e.g. band combinations).

# Conclusion

This contribution outlines initial views on control plane in 6G, and makes the following observation and proposals:

Observation: NR control plane suffers from high overhead, overlapping states, and complex signalling. 6G should use single framework that minimizes complexity while being scalable, energy-efficient, and service-aware to deliver low-latency and robust operation.

Proposal 1: Support on-demand System Information, including SSB transmission without SIB1 (SIB1-less), multi-carrier aware access, and cases where SSB may not always be present.

Proposal 2: Support scalable SI delivery for 6G, with service-based tailoring and fine-grained granularity (per-PLMN/slice/service).

Proposal 3: Define the 6G state model with:

- Idle: Initial attach/recovery only.

- Semi-connected/inactive: Default low-power state with stored UE context, RAN-based paging, UE controlled mobility.

- Connected: High-throughput state with full network-controlled mobility and bearer setup.

Proposal 4: Introduce fast transition between Semi-connected/Inactive and Connected states targeting sub-10 ms resume latency, using a stored UE context in the semi-connected state.

Proposal 5: Support modular RRC signalling with reusable profiles, and MAC CE triggered reconfiguration.

Proposal 6: Support adaptive access control with per-UE-type, per-service, per-slice granularity, and validity/scope mechanisms.

Proposal 7: ASN.1 should be designed toward modular RRC configuration parts and optimized encoding for repetitive elements (e.g. band combinations).

# References

- RP-251881 “Study on 6G Radio”