3GPP TSG-RAN WG2 Meeting #131bis	R2-2507435

# Prague, Czech Republic, 13 – 17 October 2025

Agenda item:		8.6.3

Source:	Xiaomi

Title:	Discussion on mobility MAC open issues

Document for:		Discussion and Decision

# Introduction

After RAN2#131 meeting, email discussion “[POST131][116][MOB] (Vivo)” was held to further collect MAC open issues [1]. In this contribution, we discuss remaining mobility MAC issues.

# Discussion

## MAC open issues for event triggered L1 measurement

MAC-O02

The open issue is as follows:

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| OPPO | In 5.4.4, there are several note captured to clarify the SR/PUCCH resource handling for the cases that PUCCH resources are overlapped among the triggered SRs NOTE 1: Except for the cases specified in NOTE 3 below, the selection of which valid PUCCH resource for SR to signal SR on when the MAC entity has more than one overlapping valid PUCCH resource for the SR transmission occasion is left to UE implementation. NOTE 3: When the MAC entity has pending SR for SCell beam failure recovery and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery of a BFD-RS set) overlapping with PUCCH resource for SCell beam failure recovery for the SR transmission occasion, the MAC entity considers only the PUCCH resource for SCell beam failure recovery as valid. When the MAC entity has pending SR for beam failure recovery of a BFD-RS set of Serving Cell and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery) overlapping with PUCCH resource for beam failure recovery of that BFD-RS set for the SR transmission occasion, the MAC entity considers only the PUCCH resource for beam failure recovery of that BFD-RS set as valid. NOTE 6: When the MAC entity has PUCCH resource for pending SR for SCell beam failure recovery overlapping with PUCCH resource for pending SR for beam failure recovery of a BFD-RS set for the SR transmission occasion, it is up to UE implementation to select PUCCH resource for SCell beam failure recovery or PUCCH resource for beam failure recovery of a BFD-RS set. For pending SR for event triggered L1 MR MAC CE, similer text is needed for charlfication, especially for the case that PUCCH resource for pending SR for event triggered L1 MR MAC CE has overlapping with PUCCH resource for beam failure recovery. [Rapp]: As we have no conclusion on how to handle the case when pending SR for event triggered L1 MR MAC CE overlapping with other PUCCH resource, this issue is wroth to be discussed. | Issue Type: Not essential How to address it: Based on companies’ contribution. Issue Number: MAC-O02 |

This open issue is mainly related to the priority of SR for Event Triggered L1 Measurement Report MAC CE. In RAN#129 meeting, RAN2 agreed that “The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR.” In TS 38.321, the BFR SR has higher priority than the BSR SR, because the beam failure recovery is more urgent than the data transmission. From our understanding, the transmission of Event Triggered L1 MR MAC CE should also have higher priority, so that the LTM cell switch can be triggered in time. This means that the priority of the SR for Event Triggered L1 MR MAC CE should be higher than that of the SR for BSR. Given that LCH priorities of Event Triggered L1 MR MAC CE and BFR are the same, RAN2 can also conclude that priorities of SR for Event Triggered L1 MR MAC CE and BFR is the same.

Proposal 1: (MAC-O02) When handling the overlapping of PUCCH resource for pending SRs, the priority of Event Triggered L1 MR MAC CE is the same as that of BFR.

## MAC open issues for conditional intra-CU LTM

MAC open issues for CLTM TA

For the maintenance of CLTM TA, RAN2 has agreed the UE shall maintain the TA value and keep the CLTM TAT running for other candidate cells and target cell upon CLTM cell switch execution.

1. Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running. 2. C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

RAN2 has supported the coexistence of LTM and CLTM, as well as the coexistence of CLTM and L3 mobility (L3 HO and CHO). For these coexistence cases, before discussing the open issues, how to handle the CLTM TA of CLTM candidates during these mobility procedures (MAC CE triggered LTM cell switch, L3 HO and CHO execution) needs to be discussed.

Upon these mobility procedures, the CLTM configuration shall also be maintained by UE. Moreover, if the network re-configures CLTM execution conditions, the UE can still perform CLTM evaluation and trigger CLTM. Therefore, for subsequent CLTM, the CLTM TAT of all CLTM candidate cells including source cell, target cell (if the source cell/target cell is a CLTM candidate cell) shall be maintained, i.e., C-LTM TATs for all CLTM candidate cells are not stopped, which has been captured in the current Rel-19 specification, but there is no agreement on this specific scenario. We suggest RAN2 confirm this.

Proposal 2: RAN2 can confirm that the UE shall not stop the CLTM TA timers and shall maintain associated TA values for all CLTM candidate cells upon the execution of MAC CE triggered LTM cell switch, L3 HO, and CHO.

MAC-V04

The open issue is as follows:

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| vivo | How to handle the CLTM TAT of target cell when receiving an LTM Cell switch Command MAC CE with valid TA? Which is related the previous MAC-14 open issue. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: MAC-V04 |

For the coexistence of LTM and CLTM, RAN2 reached the following agreements:

RAN2#129bis: Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change). RAN2#131: If UE receives the LTM CS command triggered by NW, UE shall follow the R18 behaviour to initiate LTM CS in the target cell regardless of whether the UE has the available C-TA value.

Based on the above agreements, the LTM CSC can be used to trigger LTM Cell Switch to a CLTM candidate. Additionally, the CLTM TA stored by UE for the candidate cannot be used for MAC CE triggered LTM. For this case, MAC-V04 can be discussed, i.e., whether the TA value in LTM CSC can be used as the CLTM TA maintained by UE.

Upon CLTM execution, the UE shall not stop the CLTM TAT for target cell. If Proposal 2 is agreed, the UE also keep the CLTM TA of target cell upon MAC CE triggered LTM execution. The value of CLTM TA stored for the target cell need to match with the TA value indicated in LTM CSC. Otherwise, the UE may maintain two different TA values for the target cell (if two TAGs are not configured for the CLTM candidate cell), which may cause unnecessary confusion. In addition, even if the UE doesn’t store the CLTM TA for target before receiving the LTM CSC and there is no confusion, the valid TA in LTM CSC can also be used for subsequent CLTM.

Therefore, we think the UE shall store the valid TA in LTM CSC as CLTM TA of the target and start or re-start the associated CLTM TAT.

Proposal 3: (MAC-V04) When receiving an LTM Cell Switch Command MAC CE with valid TA, the UE shall store the TA value in the LTM CSC as the CLTM TA for the target cell, and shall start or restart the CLTM TAT associated with the target cell.

MAC-V05/MAC-K01/MAC-N01

The open issues are as follows:

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| vivo | When current serving cell is an LTM candidate cell, whether UE maintain and update its CLTM TAT based on legacy Timing Advance Command MAC CE? [Huawei] The UE could be allowed to use the valid TA value it has for the serving cell, this could be a note. [Rapp]: This has not been concluded. Let’s discuss it. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: MAC-V05 |
| ASUSTeK | If the source cell is an LTM candidate cell, upon Cell switch, does the UE store the valid TA and update its CLTM TAT of the source cell for possible RACH-less CLTM back to the source cell (e.g., if the CLTM TAT for the Source Cell is not running at the time upon Cell switch)? [Rapp]: This issue is same as above issue raised by vivo, i.e. MAC-V05. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: MAC-K01 |
| Nokia | Although the TAs of other candidates are maintained, the TA of the previous serving cell, which may also be a candidate, is discarded as it is maintained by the servingCell TAT and not the candidate TAT. For example, the corresponding candidate TAT may not have been started or it may have not been restarted upon reception of a serving cell TAC MAC CE. To address the issue: FFS whether to store the previous serving cell TA for the corresponding candidate and remaining TAT time in the CLTM TAT of the corresponding candidate upon cell switch or, as discussed above by vivo, maintain and update the cooresponding CLTM TAT based on reception of legacy TAC MAC CE for the serving cell. The former approach allows maintaining only one timer for the cell, the legacy TAT when it is serving and the candidate TAT when not. [Rapp]: This issue is same as above issue raised by vivo and ASUSTek, i.e. MAC-V05, and MAC-K01 | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: MAC-N01 |

After successfully completing LTM execution, the target cell becomes the current serving cell and the UE may maintain the CLTM TA for the current serving cell. Similar to the MAC open issue MAC-V04, the same reasoning applies. For subsequent CLTM and to avoid the mismatch between the CLTM TA and PTAG TA, the UE can maintain and update its CLTM TAT based on legacy Timing Advance Command MAC CE.

Proposal 4: (MAC-V05/MAC-K01/MAC-N01) When current serving cell is an LTM candidate cell, the UE can maintain and update its CLTM TAT based on legacy Timing Advance Command MAC CE.

MAC-O03

The open issue is as follows:

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| OPPO | How to handle CLTM TAT if the CLTM execution condition is released but candidate cofiugration is kept, i.e. NW reconfig CLTM to LTM candidate. [Rapp]: This issue is similar as above issue raised by vivo, i.e. MAC-V04. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: MAC-O03 |

In the current specification, the CLTM TA can only be used for the CLTM execution. If the CLTM execution condition is released but candidate configuration is kept, i.e., NW re-configure CLTM to LTM candidate, the CLTM TA of the candidate will become useless, unless the NW reconfigures the candidate as a CLTM candidate.

RAN2 has agreed that if the corresponding CLTM candidate configuration is released, the UE shall stop the running ltm-Candidate-TimeAlignmentTimer for the corresponding candidate cell and release the stored TA value for the candidate cell. Similarly, if the CLTM execution condition is released for a candidate, the UE shall stop the running CLTM TAT for the corresponding candidate cell and release the stored TA value for the candidate cell.

Proposal 5: (MAC-O03) If the CLTM execution condition is released but candidate configuration is kept (i.e., NW re-configure CLTM to LTM candidate), the UE shall stop the running CLTM TAT for the corresponding candidate cell and release the stored TA value for the candidate cell.

MAC-K02

The open issue is as follows:

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| ASUSTeK | In the last meeting, it was agreed that if the UE receives more TAs beyond its capability, it’s up to UE implementation to decide which one is released. Since the NW does not know which TA is released, whether the UE needs to report currently stored TAs to the NW to avoid unnecessary RA for early UL sync? [Huawei] The conclusion was to do nothing else, it is up to the network to avoid this. [Rapp]: I assume the conclusion didn’t imply it is up to the network to avoid it. While it is indeed a solution, and another solution is to inform network. Let’s discuss it. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: MAC-K02 |

RAN2 has agreed that if the UE receives more TAs beyond its capability, it’s up to UE implementation to decide which one is released. Therefore, the network may not know which TA is released by network.

Because whether and when to trigger the early UL sync is up to network implementation, the network can solve the issue by implementation to avoid unnecessary RA for early UL sync. For example, the network can always determine whether the CLTM TA is valid and whether to trigger early UL synchronization based on CLTM TAT, regardless of whether the UE has released the CLTM TA.

Proposal 6: (MAC-K02) When the UE receives more TAs beyond its capability, it is up to the network implementation to avoid unnecessary RA for early UL sync, and no specification impact is needed.

MAC-M01

The open issue is as follows:

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MediaTek | There is no clear guidance that UE will perform RACH-based LTM after CLTM recovery in stage-3 spec. In RRC spec, two different conditions (CLTM recovery, fulfilment of CLTM condition) have the same action: ##### 5.3.5.18.6 LTM cell switch execution …… 1> else (LTM cell switch triggered upon cell selection performed while timer T311 was running or upon the fulfilment of LTM cell switch execution conditions (as specified in clause 5.3.5.18.x): 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.5.18.x or 5.3.7.3) according to clause 5.3.5.3;. In MAC spec, either 5.18.35 (Enhanced) LTM Cell Switch Command or 5.y.3 Conditional LTM execution seems not reflect the action for CLTM recovery case. And it is not clear if CLTM fast recovery will go to 5.y.3 Conditional LTM execution, as the condition of CLTM fast recovery is not included ### 5.y.3 Conditional LTM execution The conditional LTM cell switch procedure is triggered when: - the MAC entity determines that the event for conditional LTM is satisfied based on L1 measurements as specified in clause 5.y.2; or - the event(s) for conditional LTM is satisfied based on L3 measurements indicated by upper layers.. For R18 LTM fast recovery UE does not have other stored TA value so it is natural to fallback to RACH-based LTM. While for R19 CLTM fast recovery, it is better to be specified somewhere to clarify UE’s behavior, as UE may have other stored TA and may be capable to perform RACH-less LTM. We can discuss whether to capture that “UE only could perform RACH-based LTM for CLTM recovery” in MAC spec, or keep it in stage-2 only. [Rapp]: It is true, i.e. we could discuss it. | Issue Type: Not essential not important How to address it: based on companies’ contribution. Issue Number: MAC-M01 |

The CLTM fast failure recovery reuses R18 LTM failure recovery mechanism (i.e. based on CBRA). However, in current RRC and MAC specifications, upon the LTM cell switch execution, the UE behaviour for CLTM based recovery and fulfilment of CLTM condition is the same. Specifically, RACH-less and CFRA are supported for CLTM based recovery based on the current specification.

Merely stating this at Stage 2 is not clear and may lead to inconsistent understandings between stage-2 and stage-3. Hence, we suggest to capture that “UE only could perform RACH-based LTM (CBRA) for CLTM recovery” in MAC spec.

Proposal 7: (MAC-M01) To capture that “UE only could perform RACH-based LTM (CBRA) for CLTM recovery” in the MAC specification.

## Other MAC open issues

MAC-Q05

The open issue is as follows:

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| Qualcomm | SP CSI-RS/CSI-IM Resource Set Act/deact for Candidate Cell MAC CE (1) In Clause 6.1.3.12a, the presence of CSI Resource Configuration ID2 is implicitly determined at UE based on the configuration of SP CSI-IM resource set for candidate cell(s) in RRC, but this implicit approach is not clear and would lead to unnecessary inclusion of CSI Resource Configuration ID2. - If CSI Resource Configuration ID1 corresponds to multiple NZP-CSI-RS resource IDs from different candidate cells, which candidate cell(s)’ CSI-IM resource configuration should be used to determine the presence of CSI Resource Configuration ID2? - CSI Resource Configuration ID2 is used only for CSI-IM for early CSI acquisition, not for L1-RSRP measurements. If this MAC CE is intended for SP CSI-RS for L1-RSRP measurements, should CSI Resource Configuration ID2 be still present just because CSI-IM resource is configured in RRC? To avoid any confusion, it would be better to use an explicit indicator such as “IM” in legacy SP CSI-RS/CSI-IM activation/deactivation MAC CE, like below, where CSI Resource Configuration ID2 is present when IM is set to 1, and absent otherwise. If IM is set to 0, TCI State ID0 will be present in Octet 2, next to IM. [Rapp]: This issue was indeed discussed in RAN2#130 meeting. Everyone agreed that UE could easily identify whether this CSI Resource Configuration ID is CSI Resource Configuration ID1 or CSI Resource Configuration ID2 based on RRC configuration due to the the following agreement： - M resource and NZP CSI resource cannot be configured for a CSI-RS resource configuration id. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Issue Number: MAC-Q05 |

The key question here is how UE can determine the presence of the field CSI Resource Configuration ID2. Related definition in TS 38.321 is as follows:

-	CSI Resource Configuration ID1: This field indicates the index of the LTM CSI resource configuration corresponding to LTM-CSI-ResourceConfigId as specified in TS 38.331 [5]. This LTM CSI resource configuration includes an SP CSI-RS resource set for the candidate cell(s) for measurement. The length of the field is 7 bits;

-	CSI Resource Configuration ID2: This field indicates the index of the LTM CSI resource configuration corresponding to LTM-CSI-ResourceConfigId as specified in TS 38.331 [5]. This LTM CSI resource configuration includes an SP CSI-IM resource set for the candidate cell(s). If the SP CSI-IM resource set for the candidate cell(s) is not configured in TS 38.331 [5], this field and the reserved bit in the same octet are absent. The length of the field is 7 bits;

Corresponding ASN.1 signalling in TS 38.331 is as follows:

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

[...Omitted...]

ltm-ResourceForInterferenceMeasurements-r19    LTM-CSI-ResourceConfigId-r18                                   OPTIONAL   -- Need R

]]

}

It is assumed that CSI Resource Configuration ID1 and CSI Resource Configuration ID1 refer to field ltm-ResourcesForChannelMeasurement and ltm-ResourceForInterferenceMeasurements, respectively. Corresponding RAN1 specification is in TS 38.214 clause 5.2.4a, as below:

A UE configured with LTM-Config can be provided configurations for CSI acquisition, by up to one Reporting Setting, ltm-CSI-ReportConfig, for a candidate cell. Each Reporting Setting ltm-CSI-ReportConfig is associated with either one or two Resource Settings

-	When one Resource Setting (given by higher layer parameter ltm-ResourcesForChannelMeasurement) is configured, it provides a list of NZP CSI-RS resources for both channel and interference measurements. The UE is not expected to be configured with more than 128 NZP CSI-RS ports in the CSI-RS resource set contained within the Resource Setting.

-	When two Resource Settings are configured, the first Resource Setting (given by higher layer parameter ltm-ResourcesForChannelMeasurement) provides a list of NZP CSI-RS resources for channel measurement, and the second Resource Setting (given by higher layer parameter ltm-ResourceForInterferenceMeasurements), provides a list of [CSI-IM resources] for interference measurement. The UE is not expected to be configured with more than 128 NZP CSI-RS ports in the CSI-RS resource set contained within the Resource Setting.

Although an LTM-CSI-ResourceConfigId cannot be used as both ltm-ResourcesForChannelMeasurement and ltm-ResourceForInterferenceMeasurements, it might be possible that one ltm-ResourceForInterferenceMeasurements can be associated with multiple ltm-ResourcesForChannelMeasurement. Then it is not clear how UE can determine the presence of the field CSI Resource Configuration ID2. To resolve the ambiguity, one way is that UE only considers ltm-ResourcesForChannelMeasurement and ltm-ResourceForInterferenceMeasurements in the same LTM-CSI-ReportConfig, i.e. if CSI Resource Configuration ID1 corresponds to ltm-ResourcesForChannelMeasurement in one LTM-CSI-ReportConfig-r18, then presence of field CSI Resource Configuration ID2 is determined by the presence of field ltm-ResourceForInterferenceMeasurements in the same LTM-CSI-ReportConfig.

Proposal 8: (MAC-Q05) For SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE, RAN2 to discuss how UE determines the presence of the field CSI Resource Configuration ID2.

# Conclusion

In this contribution, we discuss remaining mobility MAC issues, and proposal the following:

MAC open issues for event triggered L1 measurement

Proposal 1: (MAC-O02) When handling the overlapping of PUCCH resource for pending SRs, the priority of Event Triggered L1 MR MAC CE is the same as that of BFR.

MAC open issues for conditional intra-CU LTM

Proposal 2: RAN2 can confirm that the UE shall not stop the CLTM TA timers and shall maintain associated TA values for all CLTM candidate cells upon the execution of MAC CE triggered LTM cell switch, L3 HO, and CHO.

Proposal 3: (MAC-V04) When receiving an LTM Cell Switch Command MAC CE with valid TA, the UE shall store the TA value in the LTM CSC as the CLTM TA for the target cell, and shall start or restart the CLTM TAT associated with the target cell.

Proposal 4: (MAC-V05/MAC-K01/MAC-N01) When current serving cell is an LTM candidate cell, the UE can maintain and update its CLTM TAT based on legacy Timing Advance Command MAC CE.

Proposal 5: (MAC-O03) If the CLTM execution condition is released but candidate configuration is kept (i.e., NW re-configure CLTM to LTM candidate), the UE shall stop the running CLTM TAT for the corresponding candidate cell and release the stored TA value for the candidate cell.

Proposal 6: (MAC-K02) When the UE receives more TAs beyond its capability, it is up to the network implementation to avoid unnecessary RA for early UL sync, and no specification impact is needed.

Proposal 7: (MAC-M01) To capture that “UE only could perform RACH-based LTM (CBRA) for CLTM recovery” in the MAC specification.

Other MAC open issues

Proposal 8: (MAC-Q05) For SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE, RAN2 to discuss how UE determines the presence of the field CSI Resource Configuration ID2.

# References

[1] R2-2507013, vivo, "List of MAC open issues for R19 mobility"