3GPP TSG-RAN WG2 Meeting #131bis	R2-2507437

# Prague, Czech Republic, 13th Oct 2025 - 17th Oct 2025

Agenda item:	8.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining Open Issues related to SF Architecture aspects

WID/SID:	IoT_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

On the open issue related to the RAN2 impacts on different architecture options for SF mode operation, it was concluded in RAN2-131bis to discuss further in next meeting.

- Architecture option broadcasting Proposal 3: RAN2 to consider indicating S&F architecture option (Split MME or Full CN) in SIB. Proposal 4: UE does not initiate user plane establishment and user plane data transfer in the cell indicating split MME architecture. Can come back in the next meeting

In this discussion paper we discuss further on the impact of different architecture options on RAN2 aspects.

.

# 2	Discussion

For store and forward mode operation two architecture options are possible for the CN. These options are MME-Split option and Full EPC on board. In the first option only part of the MME functionalities presents in the satellite with the central MME functionality that authenticated and provide subscribed specific information is maintained at Ground node. In the other option, MME and SGW components are available on satellite, and it can act as complete PLMN for handling the NAS signalling and also store the user plane packets. In this option ground connectivity is needed to transfer the user plane packets between SGW and application server.

In typical deployments for given PLMN all the satellites will operate using any one of the architecture options. Any information if needed related to the chosen architecture option can be exchanged via NAS signalling or UE can deduce based on the NAS signalling interaction.  Hence indication of this parameter via system information is not needed for the UE to know the architecture option.

Observation 1: Architecture option is common across all satellites of given PLMN. UE can deduce the architecture option based on NAS signalling.

Proposal 1: S&F Architecture option need not be indicated via any RRC signalling including SIB indication.

However, the chosen architecture option determines the solution option for user data transmission for IoT. TS 23.401 reflects this dependency via following NOTE in SF operation, Attach and TAU procedure in SF mode. From the below NOTE and the details in the Annexure, it is clear that UP solution for user plane data transfer is not supported in split MME option.  In Full EPC option data transmission via control plane EPS optimization and user plane is possible.

NOTE 3: Some deployments cannot support all system features, for example user plane establishment and user plane data transfer cannot be supported, when a satellite is operating in S&F Mode with the split MME deployment described in Annex O.

For the Full EPC option how the AS security keys are transferred across satellites and possibility of RRC Resume across satellites in SF mode was not analysed in RAN2.  Earlier RAN2 concluded to wait for SA2 conclusion on the support for user plane solution. Now SA2 has completed the normative work and TS 23.401 does not provide clear normative procedures related to user plane data transfer and security context transfer details.  To support the user plane data transfer option along with RRC resume procedure across satellites further analysis needed along with close coordination with SA2. As the work item is already completed in RAN2, we will not have sufficient time to analyse and conclude on the RAN2 impacts. Hence, we propose that RAN2 confirm that user plane data transfer is not supported for SF mode operation in this release.

Proposal 2: RAN2 to confirm that user plane data transfer for SF mode operation is not supported in this release. Stage 2 specifications are updated to reflect these agreements. RAN2 to indicate the agreement to SA2 via LS.

# 3	Conclusion

Observation 1: Architecture option is common across all satellites of given PLMN. UE can deduce the architecture option based on NAS signaling.

Proposal 1: S&F Architecture option need not be indicated via any RRC signalling including SIB indication.

Proposal 2: RAN2 to confirm that user plane data transfer for SF mode operation is not supported in this release. Stage 2 specifications are updated to reflect these agreements. RAN2 to indicate the agreement to SA2 via LS.

# Reference

[1]	RP-240776 WID for IoT-NTN Enhancements