3GPP TSG-RAN WG2 Meeting #131bis                                    	R2-2507440

Prague, Czech Republic, 13th - 17th October 2025

Agenda item:	8.8.2

Source:	Qualcomm Incorporated

Title:	Remaining issues on beam hopping with multiple SMTC offsets [Q200]

Document for:	Discussion and Decision

# Introduction

In RAN2#131 meeting, following agreements were made.

1.    RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only.

2.	RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements

3	In connected mode the NW will only configure up to 4 STMC.

4.	UE Assistance Information message will include information to the NW to (re)configure the most relevant SMTCs for the area where the UE is located

5.	The procedure for UE assisted SMTC configuration in connected mode is follows:

-	For a UE indicating the relevant capability (FFS the name of the capability and it this is merged with some other capability) the network configures the UE via reconfiguration message with a list of reference locations and a parameter N, thereby configuring the UE to report the N closest reference locations (FFS whether also a distance threshold is indicated). If the list of reference locations is not provided, the list of reference locations provided in system information is reused.

-	The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations.

-	The UE can report the N closest reference locations via the RRCReconfigurationComplete message.

-	The network configures the relevant SMTC(s). The maximum number of configured SMTCs is according to legacy UE capability in terms of SMTC support (same as legacy).

-	The UE provides an update on the set of the N closest reference locations, if changed, via UE assistance information. Upon receiving an update, the network reconfigures the relevant SMTC(s), if changed.

In this document, we discuss remaining issues.

# Discussion

Multiple SMTCs

In the last meeting, RAN2 already finalized the solution for location-based SMTC enhancements. In IDLE mode, it is up to UE implementation how to select the SMTCs based on location information and its capability. In connected mode, UE provides the closest reference location and network can configure the correct SMTCs. In case UE moves or the SMTC needs to be updated, the UE can trigger the update of closest reference location information to the network.

Figure 1 An Example of beam hopping with extended multiple SMTC offsets (i.e., 6 offsets) and two periodicities

The location-based SMTC can also be applicable to moving cell scenario. In Rel-18, we introduced the concept of moving reference location which can be time variant, and UE can predict the future reference location. Since the reference location in this case is for one time use to perform down selection of the 4 SMTCs out of 6 SMTCs, the use of reference location at time t is sufficient and UE probably would not need to track the reference location. Therefore, network would be able to update the reference location when ephemeris and epoch time are updated without notifying UE.

Location-based SMTC enhancement is also supported in moving cell scenario. The existing movingReferenceLocation is reused for this purpose (i.e., network can update the movingReferenceLocation with epoch time without notifying UE).

Since RAN2 already adopted the location-based SMTC, we see no motivation to introduce another solution to solve the same problem in Rel-19. This solution works for both IDLE mode measurements (reference locations provided in SIB for each SMTC) and connected mode measurements (reference locations provided in MO for each SMTC). In future, for example, for the case of GNSS-less operation in Rel-20, it is worth exploring other solution.

Location-based SMTC is sufficient to solve issue of selecting four SMTCs both in connected mode and idle mode measurements and there is no need to introduce second solution to solve the same issue in Rel-19.

For inter-frequency measurements:

In our view, the discussion of beam hopping is limited to intra-frequency measurements scenarios, i.e., for energy saving, the satellite hops the beams but they all can operate in the same frequency. Multi-frequency beam hopping is an added complexity due to involvement of measurement gaps as no more than 2 concurrent MGs can be configured per UE given the UE supports parallelMeasurementGap-r17 and network may want efficient configuration choice.

With such consideration, RAN4 impact should also be investigated as in the LS [1], RAN2 only asked question for the intra-frequency neighbor cells.

Therefore, RAN2 first need to decide whether SMTC enhancement is applicable for inter-frequency measurements. If this is applicable for inter-frequency measurements, then for IDLE mode case, it should be left to UE how it selects 4 SMTCs and use 2 periodicities. However, for connected mode, there should be no change on the SMTC configuration as network still configures up to 4 SMTCs as in Rel-17 taking into account assistance information from UE. However, for the configuration of multiple periodicities for inter-frequency measurements, a new UE capability should be introduced due to involvement of measurement gap(s). Since configuration depends on UE’s assistance information, we think no further enhancement is needed for measurement gap itself.

RAN2 first needs to decide whether more than 4 SMTCs configuration is applicable for inter-frequency measurements.

If SMTC enhancement is applicable for inter-frequency measurements, it is up to UE how to select SMTC based on location and use multiple periodicities for IDLE mode inter-frequency measurements.

If SMTC enhancement is applicable for inter-frequency measurements, a new UE capability to support two periodicities for connected mode inter-frequency measurements is introduced and an LS is sent to RAN4 for any issue/concern.

In our view, the original intention of limiting to 4 SMTCs is to limit the number of satellites a UE can measure simultaneously. However, the SIB19 was extended without taking into account this. Now with the beam hopping enhancements, it is not clear whether 4 SMTCs is for the given single satellite beams or more than 4 satellites’ beams. We think it should be clarified that total number of satellites across intra and inter-frequency measurements at a given time instance does not exceed 4.

Total number of satellites across intra and inter-frequency measurements at a given time instance does not exceed 4.

# Conclusion

Following proposals are made.

Proposal 1	Location-based SMTC enhancement is also supported in moving cell scenario. The existing movingReferenceLocation is reused for this purpose (i.e., network can update the movingReferenceLocation with epoch time without notifying UE).

Proposal 2	Location-based SMTC is sufficient to solve issue of selecting four SMTCs both in connected mode and idle mode measurements and there is no need to introduce second solution to solve the same issue in Rel-19.

Proposal 3	RAN2 first needs to decide whether more than 4 SMTCs configuration is applicable for inter-frequency measurements.

Proposal 4	If SMTC enhancement is applicable for inter-frequency measurements, it is up to UE how to select SMTC based on location and use multiple periodicities for IDLE mode inter-frequency measurements.

Proposal 5	If SMTC enhancement is applicable for inter-frequency measurements, a new UE capability to support two periodicities for connected mode inter-frequency measurements is introduced and an LS is sent to RAN4 for any issue/concern.

Proposal 6	Total number of satellites across intra and inter-frequency measurements at a given time instance does not exceed 4.

# References

[1] R2-2503068, “LS on SMTC enhancements” RAN2#129bis, Source RAN2, To RAN4.