3GPP TSG RAN WG2 #131bis	R2-2507444

Prague, Czech Republic, October 13th-17th 2025

# Title:	[Draft] Reply LS on the RAN simulation assumptions for ULBC

# Response to:	R2-2505068/ S4-251584

# Release:	Release 20

# Work Item:	FS_ULBC

Source:	RAN WG2

To:	SA WG4

Cc:	RAN WG1, SA WG2, CT WG1

Contact Person:

Name:	Bharat Shrestha

E-mail Address:	bshresth AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

RAN2 would like to thank SA4 for the LS. RAN2 would also like to provide the following response.

Question 1: the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option.

RAN2 Answer: For NB-IoT UP solution, RAN2 expects overhead of 4 bytes for RoHC with UDP checksum enabled and UO-1 header type used, 1 byte for PDCP and 1 byte for RLC UM.

For NB-IoT CP solution, RAN2 expects overhead of 2 bytes for RRC, PDCP is bypassed and 2 bytes for RLC. However, for voice packet AM mode is not preferred and RLC UM needs to be defined for SRB carrying voice resulting in 1 byte overhead for RLC.

RAN2 discussed the down selection between CP and UP solutions for voice support over NB-IoT-NTN and reached the conclusion that both solutions can be specified. However, no consensus reached whether any AS layer optimization needs to be done. RAN2 will provide further updates in the upcoming meetings.

Question 2: specifically, whether a packet overhead of 1 byte of MAC header is realistic.

RAN2 answer: In TS 36.321 Figure 6.1.6-2, 1 byte R/R/E/LCID MAC subheader format is already supported given the MAC header is the only MAC subheader in the PDU, i.e., field E = 0. If only one MAC PDU fits in the TBS size then length field does not need to be included, otherwise 2 or more than 2 bytes of MAC header would be needed.

# 2. Actions:

# To SA4:

ACTION:

RAN2 kindly asks SA4 to take the above response into account.

# 3. Dates of Next RAN2 Meetings:

RAN2#132						17th to 21st November 2025		Dallas

RAN2#132						9th to 13th February 2026		Gothenburg