3GPP TSG-RAN WG2 Meeting #131bis                                    	R2-2507448

Prague, Czech Republic, October 13th-17th 2025

Agenda item:	9.7.2

Source:	Qualcomm Incorporated

Title:	Discussion on CP and UP solutions

Document for:	Discussion and Decision

- Introduction

In [1], following objectives are specified.

- Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2]

  - Coordination with SA is required

  - The objective can be updated with further details based on the outcome of the above down-selection

- Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1]

- Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2]

- Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2]

- Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4]

- Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements

- Note: Coordination with SA4 is expected

One major point is to down select UP or CP solutions. In this document, we provide importance of each solution and propose way forward.

- Discussion

## Voice over NB-IoT:

Currently NB-IoT UE mandatorily supports CP solution, however support of UP solution is optional. Due to lack of support, i.e., optional to support DRB and AS layer security, there are several features NB-IoT does not support such as coarse UE location report to RAN via AS layer and most importantly the handover.

NB-IoT UEs are very low complexity UEs, i.e., small bandwidth and low data rate, for not having complex features such has measurement gaps, handover and real time or time sensitive solutions such as voice. This is the reason why NB-IoT remains a popular choice today despite of new 5G NR and 5G redcap technologies.

When adding real time services, e.g., voice and emergency call, the simplicity of NB-IoT technologies should be maintained.

Table 1 TBS and PHY bitrate for 80ms bundling [2]

| TBS (bits) | 144 | 256 | 328 | 424 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.8 | 3.2 | 4.1 | 5.3 |
| Codec bitrate (kbps) | 1.1 | 2.5 | 3.4 | 4.6 |

To support voice over NB-IoT, SA4 has been working on defining new low codec rate as achievable PHY data rate over NB-IoT is very low (requires less than 3kb/s) [2]. As per discussion, the codec rate cannot be too low not to compromise the voice quality specially on GEO channel where uplink link budget is worse than in TN scenario. As shown in table 1, with 80ms bundling, to use the TBS size as small as 144 bits, the codec rate has to be reduced to as low as 1.1kb/s. This indicates that the total overhead cannot go more than 0.7 kb/s (i.e., 7 bytes) in a higher coverage level.

To achieve the total overhead of 7 bytes or less, both CP solution and UP solution can be explored. It is to note that no solution as supported by current specification will be able to achieve the 7 bytes or less overhead as shown in Table 2.

For UP solution, even though 3 bytes of ROHC overhead is considered, it is not the case most of the time due to talk spurts and synchronization issues between compressor and decompressor. We will further provide details in the next section.

Further optimization may need to be considered to keep the total overhead to be 7 bytes or less regardless of UP solution and CP solution.

Table 2 UP vs CP (using existing NIDD) solution headers considering no integrity protection.

In addition, SA4 has not included 4 bytes of overhead for integrity protection in their calculation [1]. In LTE, the DRB does not support integrity protection. This should be maintained for the support of voice over NB-IoT regardless of CP or UP solutions.

Similarly, it is not fair to compare RLC UM in UP solution vs RLC AS in CP solution. As retransmission at RLC layer is not expected for voice traffic over GEO, RLC UM is considered. Therefore, when a new SRB is defined for the CP solution, the RLC UM should be possible to configure. If segmentation is not expected, then RLC TM can also be considered.

RAN2 confirms integrity protection is not used for voice traffic regardless of UP solutions or CP solutions.

RAN2 confirms RLC UM is used for the voice traffic regardless of CP or UP solution. FFS if RLC TM can be considered.

## UP solution:

In case of UP solution, the DRB is setup and AS security is established. For RTP/UDP/IP traffic, the PDCP can be configured to enabled RoHC for header compression as shown in Figure 1.

Figure 1 RoHC compression at PDCP layer

As we can see more than 40 bytes of header from RTP/UDP/IP can be compressed to minimum of 3 bytes at RoHC SO state. However, as shown in Figure 2, the UE starts with IR state where full 40 bytes of header would need to be transmitted. For 80ms bundling with codec rate of 1.1kp/s, this requires TBS size of 440 bits and longer time to finish transmission and longer time to move to FO state. During this time, several voice packets may be discarded deteriorating user experience.

Due to dynamic change in ROHC context, i.e., talk to silent or silent to talk transmission and change in inter-arrival time of SIDs packet at codec, the RoHC state falls back to FO state from SO state and the RoHC header type UO-1 would have to be used. The header format UO-1 has 1 bit marker flag, marking the beginning of a talk spurt.

Figure 2 RoHC RFC 3095 O mode showing optimistic approach to move to SO state.

The commonly used RoHC mode is the Bidirectional Optimistic mode (O-mode) as it aims to maximize compression efficiency and sparse usage of the feedback channel as per RFC 3095.  It reduces the number of damaged headers delivered to the upper layers due to residual errors or context invalidation. As explained in RoHC protocol RFC 3095, in O mode, the compressor in FO state may use optimistic approach to send UOR-2 header to make sure synchronization between compressor and decompressor before moving to SO state as ACKs for UOR-2 packets may not be received.

In GEO channel with very long RTT greater than 500ms, sever factors contribute to loss of synchronization between compressor and decompressor, such as

- Jitter due to queueing delay in backhaul and Scheduling delay

- Poor link budget/Residual error

- Local clock error in compressor and decompressor (in case of time-based decompression)

- wraparound issue of compressed RTP SN and/or compressed RTP timestamp

In Bidirectional Optimistic mode (O mode), RoHC header type (i.e., UO-0 header type) can carry only 4 bits LSB of RTP SN and 3 bits CRC SN. When more than 16 consecutive packets are lost between compressor and decompressor, this may cause the LSBs of the RTP SN in compressed packets to be interpreted wrongly, because the decompressor has not moved the interpretation interval for lack of input (e.g., a kind of context damage). This can lead to CRC check failure (i.e., decompression failure). As per section 5.3.2.2.3 in RFC 3095, when the CRC check of a decompressed packet fails, context damage may be assumed and a NACK is sent.

Therefore, the RoHC needs to use the header format UOR-2 because of the following reasons.

- Set a new TS_STRIDE during transitions:

  - From voice to SID and vice-versa

  - Between different SID rates

- Re-sync time reference by providing raw (uncompressed/unscaled) time stamp:

  - Every time TS_STRIDE changes

  - When timing is out of sync (e.g., ROHC feedback is received)

- Establish confidence before moving to SO state

We also provide field log information obtained from the RoHC implementation at UE, and this shows confidence building is important part of ROHC implementation as UE may not know when it can receive the ACKs to move to SO state.

Figure 3 Field log of RoHC implementation

In our observation how RoHC operates is largely dependent on implementation and cannot be mandated by 3GPP specifications. For example, how decompressor at network side works to generate feedback in Mode O is not dictated by 3GPP specifications as below clarified by the RoHC protocol in RFC 3095.

3GPP specification cannot dictate whether a certain type of header is not allowed or change/modify the requirements specified by RFC 3095 protocol. Therefore, in our view, the dynamically changing RoHC header in UP IP solution cannot be handled without semi-persistence scheduling and loss of voice packets and hence degrading the user’s performance.

The dynamically changing RoHC header in UP IP solution cannot be handled without semi-persistence scheduling and voice packets discarding and hence degrading the user’s performance.

As shown in figure, when the UE’s voice packet size exceeds the TBS size allocated for voice traffic with negotiated codec rate and considering the 4 bytes of RoHC header, the RLC layer would need to segment the UL data.

RLC segmentation adds further complexity and larger overhead. Due to this, the segmented voice packet needs to be sent which will delay the transmission of the next arrived voice packet. It is to note that NB-IoT operates in half duplex mode and resources are already exhausted for allocating resources to UL voice traffic and DL voice traffic at the same time. For a TBS size of 144 bits, a minimum of 64 repetitions of NPUSCH transmission may be needed at worst coverage level. Network may not be able to allocate further UL resource to UE. This can further impact the network’s ability to create a feedback channel for RoHC and configure other resources such as PRACH, SR and BSR.

NB-IoT operates in half duplex mode and resources are already exhausted for allocating resources to periodic UL voice traffic and DL voice traffic at the same time. Network may not be able to allocate further UL resource to UE.

Figure 4 For HD NB-IoT, UL and DL voice traffic alone can overwhelm the resources making hard for ROHC feedback channel.

Eventually the older voice packets would need to be dropped to make space for the new fresh voice packets. Such voice packet drops will lead to bad user experience of voice call. In short, RoHC leads to dynamically changing packet size which can be very challenging to handle from resource scheduling point of view for NB-IoT UEs operating in HD mode.

RoHC leads to dynamically changing packet size which can be very hard to handle from resource scheduling point of view for NB-IoT UEs operating in HD mode.

In short, we observe several issues of using RoHC in UP IP solution:

- RoHC behavior is based on RFC 3095 protocol and is not mandated by 3GPP specifications.

- During initial call setup, ROHC starts with IR state and full header of more than 40bytes will need to be transmitted. The change of IR state to FO state can take significantly long due to lack of enough resource causing loss of several voice packets.

- In GEO channel with very long RTT greater than 500ms, sever factors contribute to loss of synchronization between compressor and decompressor, such as Jitter, Poor uplink link budget, Local clock error in compressor and decompressor (in case of time-based decompression), wraparound issue of compressed RTP SN and/or compressed RTP timestamp and Residual error.

- the RoHC needs to use the header format UOR-2 to resync and move to SO state. This leads to change in voice packet size and RLC segmentation which further leads to loss of voice packets due to lack of resource and scheduling delay.

- NB-IoT operates in half duplex mode and resources are already exhausted for allocating resources to periodic UL voice traffic and DL voice traffic. Network may not be able to allocate further UL resources to UE for transmitting RoHC feedback and other data such as SR, BSR, TA report etc.

## CP solution:

When UE is using CP solution, there is no AS security established. Currently SRB1bis is used to transport the NAS PDU and PDCP is bypassed. The NAS PDU may include any control signal or voice data.

Figure 5 CP non-IP solution

However, for CP solution, a new SRB would need to be introduced to carry IMS signaling and voice packets to differentiate it from other signaling data. This is fairly simple to do as shown below, i.e., define a default SRBx configuration with a logical channel ID. It can be specified to have same configuration as SRB1bis or clarified.

Table 3 TS 36.331 section 9.1.2.2a          SRBx Parameters

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| RLC configuration |  | PDCP is bypassed. RLC UM mode is used. |  |
| logicalChannelIdentity | 5 |  |  |

Second, we introduce the logical channel ID in the table below.

Table 4 TS 36.321 Table 6.2.1-2 Values of LCID for UL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 00000 | CCCH |
| 00001-01010 01001 | Identity of the logical channel |
| 01010 | Identity of the logical channel SRBx |
| 01011 | CCCH |
| 01100 | CCCH |
| 01101 | CCCH and Extended Power Headroom Report |

Since new SRBx is used, the AS protocol layer header can also be significantly reduced. For example, absence of RoHC header makes it more predicable packet size and hence the RRC header can be reduced to 1 byte from 2 byte. In this case, the NAS PDU can be carried by a RRC message having fixed size octet string.

DedicatedInfoNASFixedSize ::=       OCTET STRING (SIZE(FixedZize))

Unlike SRB1 and SRB1bis, the new SRB can be fairly easy to specify as RLC UM reducing the RLC header from 2 bytes to 1 byte. This altogether incurs total protocol header of 4 bytes considering 2 bytes of MAC header. As the packet size is predictable, the network can configure the SPS resource when the new SRBx is configured. Once the call set up is complete, the network can activate the SPS by MAC CE command. As shown in Table x, we observe that CP solution and UP solution result in same overhead at AS protocol layer to meet the desired codec rate of 1.2kb/s and TBS size of 144 bits. It is also possible to reduce the NAS layer header drastically.

Table 5 Header comparison between UP and CP solutions

How the NAS header compression is done for RTP can be left to SA2/CT1 decision. However, from RAN2 perspective, non-IP-based solution is simpler and more viable due to absence of RoHC. It is to note that non-IP solution will have similar performance over UP vs CP solution due to absence of RoHC.

Non-IP-based solution (i.e., CP non-IP or UP non-IP) is simpler and more viable from resource scheduling perspective due to absence of RoHC and can be considered as a solution for support of voice from RAN2 perspective.

- Conclusion

Following observations are made:

Observation 1.	When adding real time services, e.g., voice and emergency call, the simplicity of NB-IoT technologies should be maintained.

Observation 2.	Further optimization may need to be considered to keep the total overhead to be 7 bytes or less regardless of UP solution and CP solution.

Observation 3.	The dynamically changing RoHC header in UP IP solution cannot be handled without semi-persistence scheduling and voice packets discarding and hence degrading the user’s performance.

Observation 4.	NB-IoT operates in half duplex mode and resources are already exhausted for allocating resources to periodic UL voice traffic and DL voice traffic at the same time. Network may not be able to allocate further UL resource to UE.

Observation 5.	RoHC leads to dynamically changing packet size which can be very hard to handle from resource scheduling point of view for NB-IoT UEs operating in HD mode.

Following proposals are made:

Proposal 1	RAN2 confirms integrity protection is not used for voice traffic regardless of UP solutions or CP solutions.

Proposal 2	RAN2 confirms RLC UM is used for the voice traffic regardless of CP or UP solution. FFS if RLC TM can be considered.

Proposal 3	Non-IP-based solution (i.e., CP non-IP or UP non-IP) is simpler and more viable from resource scheduling perspective due to absence of RoHC and can be considered as a solution for support of voice from RAN2 perspective.

- References

[1] RP-251867, “New WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4”, Prague, Czech Republic, June 9-13, 2025.

[2] S4-251585, “Permanent Document FS_ULBC”, Version 0.2.0.