3GPP TSG-RAN WG2 #131bis	R2-2507460

# Prague, CZ, 13th – 17th October 2025

Agenda Item:	 8.1.2.3

Source:	Ericsson

Title:	Open Issues LCM for Positioning

Document for:	Discussion

# 1	Introduction

In this paper, we discuss and address the open issues for LCM for positioning which are based on SA2 LS and stage2 discussions.

# 2 	Discussion

## 2.1 Open Issue SA2 LS

SA2 has sent LS asking feedback for case3b

“SA2 kindly asks RAN2 to provide feedback on whether the ground truth data and its related data (e.g., quality indicator of ground truth data, time stamp of ground truth data) can be provided from UE to the LMF by reusing the LPP protocol.”

From RAN2 perspective, RAN2 has already made an agreement that.

RAN2#131 Agreement: Case 3a and Case 3b can be supported without new impact to LPP.

The current LPP already allows LMF to obtain UE location and the associated data such as quality of the obtained location (e.g.: timing quality, uncertainty and confidence). Besides the UE may also include the timestamp Or the LMF can deduce the timestamp itself based on when the UE provides the measurement. It is also possible for LMF to use scheduled location time T feature such that LMF can configure the UE to provide the time at certain time instance T. Hence, existing solution can be reused.

RAN2 sends LS to SA2 saying existing LPP procedure can be reused for ground truth collection.

Reply LS to SA2 says “existing LPP procedure can be reused for data collection with regards to ground truth data and its related data (e.g., quality indicator of ground truth data, time stamp of ground truth data)”.

## 2.3 	Stage 2

### 2.2.1 Multiple Positioning Method Configurations

R2-2503784 provided a correction with below changes. We believe this can be included as part of Rel-19 AI/ML since these aspects have been discussed as part of Rel-19 with switching/fallback cases.

5.4.1 User Equipment (UE) The UE may make measurements of downlink signals from NG-RAN, sidelink signals from other UEs, and other sources such as E-UTRAN, different GNSS and TBS systems, WLAN access points, Bluetooth beacons, UE barometric pressure and motion sensors. The measurements to be made will be determined by the chosen positioning method. The UE may also contain LCS applications, or access an LCS application either through communication with a network accessed by the UE or through another application residing in the UE. This LCS application may include the needed measurement and calculation functions to determine the UE's position with or without network assistance. This is outside of the scope of this specification. The UE may also, for example, contain an independent positioning function (e.g., GPS) and thus be able to report its position, independent of the NG-RAN transmissions. The UE with an independent positioning function may also make use of assistance information obtained from the network. When an LMF invokes multiple positioning methods for UE-based mode, the UE may combine the multiple measurements or location results to derive a single location estimate (hybrid positioning) or may select between the requested positioning methods to derive a single location estimate. NOTE: When multiple positioning methods are requested by an LMF for UE-based positioning, the derivation of a single location estimate by the UE is up to UE implementation.

This sort of explanation would be needed since it may also simplify some of the test cases that may be having to be written and conducted when AI/ML positioning method is tested along with other UE based positioning methods. We see some value to add such description to clarify that it is up to UE implementation how to derive or which positioning method to use.

Stage2 TP to include clarification on UE behaviour when multiple methods are configured is agreed.

### 2.2.2 On demand PRS

There is an ongoing discussion also to capture in stage2 TS that UE request specific TRPs for PRS transmission to ensure consistency between training and inference phase.

From our perspective, we do not need to detail all the stage3 fields in stage2 TS, else it would require lot of updates in stage2, i.e such granular details are not needed.

“UE requests for specific TRPs for PRS transmission with on-demand PRS configuration” is NOT added to stage2 description for on-demand procedure.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Reply LS to SA2 says “existing LPP procedure can be reused for data collection with regards to ground truth data and its related data (e.g., quality indicator of ground truth data, time stamp of ground truth data)”.

Proposal 2	Stage2 TP to include clarification on UE behaviour when multiple methods are configured is agreed.

Proposal 3	“UE requests for specific TRPs for PRS transmission with on-demand PRS configuration” is NOT added to stage2 description for on-demand procedure.

# References

TS 37.355, LPP