3GPP TSG-RAN WG2 Meeting #131bis	                                                              R2-2507461

Prague, Czech, 13rd – 17th Oct 2025

Agenda item:	10.3.1

Source:	CEWiT

Title:	Considerations for 6G User Plane Functions and Protocols

Document for:	Discussion and Decision

# Introduction

In RAN#109 meeting, revised SID on 6G Radio (6GR) was approved [1]. The relevant SID topic related to this paper is

- User Plane architecture and protocol design

In this contribution, we discuss the user plane protocol functionalities and considerations for 6G protocol stack design.

# Discussion

In 6G, the user plane protocol should be able to meet required performance of basic services like eMBB and new services like immersive communication and other latency critical applications. The 6G user plane protocol stack may be designed by incorporating 5G user plane functionalities as a baseline, but it should be allowed to evolve in later releases.

Proposal 1. Consider the following as essential user plane functionalities of 6G as similar to 5G NR:

- IP and Ethernet header compression

- Security: encryption and integrity protection of data

- Re-ordering for in-sequence delivery

- Retransmission

- Segmentation & re-assembly

- Multiplexing and demultiplexing of PDUs

5G NR approach of one size fit for all may not be an efficient way to deal with advanced 6GR services. Related observations and proposals are the following.

In 5G NR, RLC and PDCP contains separate sequence numbers for different purposes. It can be beneficial for a single sequence number-based protocol to reduce header reduction. In this regard, merging sequence number-based functionalities of RLC and PDCP into a single protocol stack layer may be considered in 6G to reduce protocol overhead and overall processing time.

RLC retransmissions are inefficient for latency sensitive applications. A more appropriate retransmission strategy would be to enhance the HARQ retransmission scheme by incorporating service awareness for latency sensitive applications.

In 5G NR, if a PDU or PDU set is not delivered in time, it can only be discarded at PDCP layer. It may be beneficial to enhance service awareness at lower layers to enable discarding at lower layer.

Proposal 2. Study new optimized set of protocol layer stack either as an additional option applicable to certain use cases (e.g. latency sensitive applications) or as a single protocol stack (which should enough to meet the requirements of all the use cases including control plane data traffic).

Proposal 3. Study for bringing enhanced service awareness at lower layers

Based on the above observations and proposals, we also need to rethink on the need of separate RLC protocol layer even for control plane data traffic. Explicit lower layer acknowledgement (e.g. RLC ACK) may not be needed for certain control plane data where implicit higher layer acknowledgement is present. For example, RRCReconfigurationComplete can be taken as an implicit ack for RRCReconfiguration at RRC layer. Therefore, it will be more appropriate for RRC or PDCP to determine whether to ensure guaranteed delivery is required or not. If retransmission is required to guarantee the delivery, a timer-based retransmission can be done at RRC layer and thereby avoiding RLC layer retransmissions.

Proposal 4: Study whether protocol stack should ensure guaranteed and in-sequence delivery for all control plane data traffic such as RRC or only for certain control plane data

# Conclusion

In this contribution, we discuss the user plane protocol of 6G. Following are the proposals:

Proposal 1. Consider the following as essential user plane functionalities of 6G as similar to 5G NR:

- IP and Ethernet header compression

- Security: encryption and integrity protection of data

- Re-ordering for in-sequence delivery

- Retransmission

- Segmentation & re-assembly

- Multiplexing and demultiplexing of PDUs

Proposal 2. Study new optimized set of protocol layer stack either as an additional option applicable to certain use cases (e.g. latency sensitive applications) or as a single protocol stack (which should enough to meet the requirements of all the use cases including control plane data traffic).

Proposal 3. Study for bringing enhanced service awareness at lower layers

Proposal 4: Study whether protocol stack should ensure guaranteed and in-sequence delivery for all control plane data traffic such as RRC or only for certain control plane data

# References

- RP-252912		Revised 	SID: Study on 6G Radio